3GPP TSG-RAN WG4 Meeting #88	R4-1809856

Gothenburg, SE, 20th – 24th Aug, 2018

Agenda Item:	7.13.1.4

Source: 	Intel Corporation

Title: 	NR UE PDCCH Demodulation requirements

Document for:	Discussion

# Introduction

In RAN4 AH1807 NR UE PDCCH demodulation requirements were discussed and way forward was agreed [1]. A set of test cases were agreed for initial simulation alignment. In this contribution we provide PDCCH simulation results for alignment purposes.

# Discussion

In [1] the set of test cases for simulation alignment were agreed and are captured in the table below:

Table 1: Simulation assumptions for initial alignment

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | Antenna configuration with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 3 | 15 | 1_1 | 24 | [51] | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x2 Low | 1x4 Low |
| 4 | 15 | 1_1 | 48 | [51] | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 5 | 15 | 1_1 | 48 | [51] | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 7 | 30 | 1_1 | 102 | [53] | 1 | 4 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 1x2 Low | 1x4 Low |
| 8 | 30 | 1_1 | 102 | [53] | 1 | 8 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 9 | 120 | 1_0 | [60] | 41 | 1 | 2 | Interleaved | 2 | TBD | 1x2 Low | NA |
| 10 | 120 | 1_1 | [60] | [53] | 1 | 4 | Interleaved | 2 | TBD | 1x2 Low | NA |
| 11 | 120 | 1_1 | [60] | [53] | 1 | 8 | Interleaved | 2 | TBD | 2x2 Low | NA |

Simulation results for alignment

Simulation results in FR1

For simulations with Format 1-1 the payload size used in Tests 3, 4, 5 was 50 and that for Tests 7, 8 was 52. Also, for 2Tx simulations the random precoder specified in [1] was used.

Table 2: Simulation results in FR1 with 2Rx

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | SNR @ 1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 6.3 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | -1 |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x2 Low | 5.6 |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 3.7 |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | -4.3 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x2 Low | 5.5 |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 1x2 Low | 1.5 |
| 8 | 30 | 1_1 | 102 | 52 | 1 | 8 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 2x2 Low | -2.5 |

Table 3: Simulation results in FR1 with 4Rx

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 4Rx | SNR @ 1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | 0.1 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x4 Low | -4.7 |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low | 0.5 |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | -2.2 |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x4 Low | -7.4 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x4 Low | 3.4 |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 1x4 Low | 0.0 |
| 8 | 30 | 1_1 | 102 | 52 | 1 | 8 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 2x4 Low | -4.2 |

Simulation results in FR2

For simulations with Format 1-1 the payload size used in Tests, 10, 11 was 52. The CORESET BW used for FR2 simulations was 60. The propagation channel for FR2 was TDL-A, 30ns, 10Hz. Also, for 2Tx simulations the random precoder specified in [1] was used.

Table 4: Simulation results in FR2

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | SNR @ 1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 120 | 1_0 | 60 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x2 Low | 4.3 |
| 10 | 120 | 1_1 | 60 | 52 | 1 | 4 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x2 Low | 1.4 |
| 11 | 120 | 1_1 | 60 | 52 | 1 | 8 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 2x2 Low | -2.5 |

Test case definitions

The testcases used for initial simulation alignment are a comprehensive set of test cases covering all the agreed parameter settings of Aggregation level, CORESET configurations, DCI formats. In addition we recommend adding a test case in FR1 with 15KHz SCS and 1 symbol CORESET.

Table 5: Proposed testcases for PDCCH Demodulation

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | Antenna configuration with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x2 Low | 1x4 Low |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 1x2 Low | 1x4 Low |
| 8 | 30 | 1_1 | 102 | 52 | 1 | 8 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 9 | 15 | 1_0 | 96 | 43 | 1 | 8 | Interleaved | 2 | TDL-B, 100ns, 100Hz | 2x2 Low | 2x4 Low |
| 10 | 120 | 1_0 | 60 | 41 | 1 | 2 | Interleaved | 2 | TBD | 1x2 Low | NA |
| 11 | 120 | 1_1 | 60 | 52 | 1 | 4 | Interleaved | 2 | TBD | 1x2 Low | NA |
| 12 | 120 | 1_1 | 60 | 52 | 1 | 8 | Interleaved | 2 | TBD | 2x2 Low | NA |

Proposal #1: Use the above listed testcases to define PDCCH demodulation requirements in NR

# Conclusion

In this paper we present the simulation results for initial alignment for PDCCH demodulation and recommend test case definitions. We propose the following test cases for PDCCH demodulation requirements:

Proposal #1: Use the listed testcases to define PDCCH demodulation requirements in NR

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | Antenna configuration with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x2 Low | 1x4 Low |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 1x2 Low | 1x4 Low |
| 8 | 30 | 1_1 | 102 | 52 | 1 | 8 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 9 | 15 | 1_0 | 96 | 43 | 1 | 8 | Interleaved | 2 | TDL-B, 100ns, 100Hz | 2x2 Low | 2x4 Low |
| 10 | 120 | 1_0 | 60 | 41 | 1 | 2 | Interleaved | 2 | TBD | 1x2 Low | NA |
| 11 | 120 | 1_1 | 60 | 52 | 1 | 4 | Interleaved | 2 | TBD | 1x2 Low | NA |
| 12 | 120 | 1_1 | 60 | 52 | 1 | 8 | Interleaved | 2 | TBD | 2x2 Low | NA |

# References

- R4-1809349, WF on simulation assumptions for NR UE PDCCH demodulation tests in Rel-15, Ericsson