3GPP TSG-RAN4 Meeting #88 	R4-1809867

# Gothenburg, Sweden, 20 – 24 Aug, 2018

Agenda item:	7.12.3.7

Source:	Intel Corporation

Title:	On test case design for TA accuracy

Document for:	Discussion

# Introduction

The phase 1 test case list was approved in last RAN4 meeting[1], and Intel will work on the TA related test cases as below,

| Test case number | Test purpose | Company for CR editor |
| --- | --- | --- |
| 1 | EN-DC/SA cell search and L1 meas period | Huawei |
| 3 | EN-DC Timing accuracy and adjustment | Qualcomm |
| 4 | SA Timing accuracy and adjustment (draft if possible for RAN4#88, otherwise RAN4#88bis) | Qualcomm |
| 5 | EN-DC TA accuracy | Intel |
| 6 | SA TA accuracy | Intel |
| 7 | EN-DC SSB RLM for PSCell IS and OOS | Mediatek |
| 9 | SA SSB RLM for PCell IS and OOS | Mediatek |
| 10 | Random access | Samsung |
| 11 | Intra-freq RSRP accuracy for FR1 and FR2 | Ericsson |
| 12 | EN-DC SCell activation/deact delay | Nokia |
| 13 | EN-DC/SA CSI RLM for PSCell or PCell | Nokia |
| 14 | EN-DC interruptions due to DRX transition or deactivated SCell operationsa | CATT |

In this contribution, we discuss the test case setup and testing requirement, the corresponding CRs are in [2][3].

# Discussion

## TA test case overview

In the current TS38.133, the TA requirements are specified as below,

## 7.3 Timing advance ### 7.3.1 Introduction The timing advance is initiated from gNB with MAC message that implies and adjustment of the timing advance, see TS 38.321 clause 5.2. ### 7.3.2 Requirements #### 7.3.2.1 Timing Advance adjustment delay UE shall adjust the timing of its uplink transmission timing at time slot n+[6] for a timing advance command received in time slot n. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure. #### 7.3.2.2 Timing Advance adjustment accuracy The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 7.3.2.2-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS38.213. Table 7.3.2.2-1: UE Timing Advance adjustment accuracy Sub Carrier Spacing, SCS kHz; 15; 30; 60; 120. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc.

The requirements are not differentiated between EN-DC and standalone, however, EN-DC may involve two TAGs and we need to verify the TA delay and accuracy on sTAG; so it make sense to have two sets of teat cases to cover NSA and SA. Secondly, in TA requirement in TS38.133, the TA adjustment accuracy are specified differently according to SCS size. We don’t think it is necessary to test all the SCS conditions in the test case design, and since we already approved the RMC configuration in WF[4], we can just pick out some of the configurations for testing. The agreement in [4] is:

- RMC configurations (SCS and Channel bandwidth): - SCS=15kHz with 10MHz for FDD and TDD (FR1) - SCS=30kHz with 40MHz for TDD (FR1) - SCS=120kHz with 100MHz for TDD (FR2)

So we propose to have test cases for TDD/FDD FR1 with SCS=15kHz and TDD FR2 with SCS=120kHz. In EN-DC, the duplex mode of LTE PCell will not have any impact on the PScell TA, so we choose LTE FDD for simplicity. To summarize the TA accuracy test cases coverage are as following,

| Operation mode | Test case | SCS |
| --- | --- | --- |
| EN-DC | - LTE FDD PCell + FR1 FDD PScell | 15kHz & 10MHz |
|  | - LTE FDD PCell + FR1 TDD PScell | 15kHz & 10MHz |
|  | - LTE FDD PCell + FR2 TDD PScell | 120kHz & 100MHz |
| SA | - FR1 FDD PCell | 15kHz & 10MHz |
|  | - FR1 TDD PCell | 15kHz & 10MHz |
|  | - FR2 TDD PCell | 120kHz & 100MHz |

Since TDD configurations are still under discussing in RAN4, we would like to focus on FR1 FDD case only in this meeting. TDD test cases shall be discussed after RAN4 has conclusion on TDD configurations.

Proposal 1: The TA accuracy test case coverage is as below. This meeting only focuses on test case #1 and #4, and the other test case shall be discussed once RAN4 has conclusions on TDD configurations.

| Operation mode | Test case | SCS & BW |
| --- | --- | --- |
| EN-DC | - LTE FDD PCell + FR1 FDD PScell | 15kHz & 10MHz |
|  | - LTE FDD PCell + FR1 TDD PScell | 15kHz & 10MHz |
|  | - LTE FDD PCell + FR2 TDD PScell | 120kHz & 100MHz |
| SA | - FR1 FDD PCell | 15kHz & 10MHz |
|  | - FR1 TDD PCell | 15kHz & 10MHz |
|  | - FR2 TDD PCell | 120kHz & 100MHz |

# Test case summary

Table 1. Test case summary

| - No | Feature/core requirements | Type of Test | No of tests | Basic test configuration | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and FDD PScell in FR1 | 1 | Number of cells = 2. Inter-Band EN-DC is configured. Cell1 = PCell (E-UTRAN FDD). Cell2 = PScell(NR FDD in FR1). Cell BW = 10+10 MHz Cell2 SCS=15kHz Non-DRX. Channel model: AWGN Two TAG configuration are configured, PCell in pTAG and PScell in sTAG. General Test Parameters for NR PScell in this test case is in Table 2. General Test Parameters for E-UTRAN PCell in this test case is in Table A.7.2.4.1-1 in TS36.133. Cell specific Test Parameters for NR PScell in this test case is in Table 3. Cell specific Test Parameters for E-UTRAN PCell in this test case is in Table A.7.2.4.1-2 in TS36.133. Sounding Reference Symbol Configuration for this test case is in table 4. | Two cells are used. The test is performed on PScell. The E-UTRAN PCell is needed to be able to configure the PScell. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 4, are sent from the UE and received by the test equipment. New proposed section: A.7.x.y UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and FDD PScell in FR1 |
| 2 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and TDD PScell in FR1 | 1 | TBD | TBD |
| 3 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and TDD PScell in FR2 | 1 | TBD | TBD |
| 4 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with FDD PCell in FR1 | 1 | Number of cells = 1. Cell1 = PCell (NR FDD in FR1). Cell BW = 10 MHz Cell1 SCS=15kHz Non-DRX. Channel model: AWGN General Test Parameters for this test case is in Table 2. Cell specific Test Parameters for NR PScell in this test case is in Table 3. Sounding Reference Symbol Configuration for this test case is in table 4. | One cell is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 4, are sent from the UE and received by the test equipment. New proposed section: A.7.x.y UE Timing Advance Adjustment Accuracy Test with FDD PCell in FR1 |
| 5 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with TDD PCell in FR1 | 1 | TBD | TBD |
| 6 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with TDD PCell in FR2 | 1 | TBD | TBD |

Table 2. General test parameters of NR PCell/PScell for TA accuracy tests

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | TBD | Up to RAN4 conclusion on RMC configuration |
| PDCCH parameters |  | TBD | Up to RAN4 conclusion on RMC configuration |
| SSB burst set periodicity | ms | 20 |  |
| Available SSB number in burst set |  | 1 |  |
| SSB index |  | 0 |  |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA_new = NTA_old + 8192*Tc (based on equation in TS38.213 section 4.2) |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table 3. Cell specific test parameters for NR PCell/PScell for TA accuracy tests

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number |  | “1” in SA test case and “2” in EN-DC test case |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TBD |  | TBD (up to RAN4 conclusion on OCNG configuration) |  |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| IoNote2 | dBm/9 MHz | -65.5 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that cell(s) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

Table 4. Sounding Reference Symbol Configuration for NR PCell/PScell for TA accuracy tests

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 12 | SRS BW= 48PRBs, and frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=0 | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | nonCodebook | Non-codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331. |  |  |

# Conclusion

In this contribution, we discuss the test case setup and testing requirement, the corresponding CRs are in [2][3].

Proposal 1: The TA accuracy test case coverage is as below. This meeting only focuses on test case #1 and #4, and the other test case shall be discussed once RAN4 has conclusions on TDD configurations.

| Operation mode | Test case | SCS & BW |
| --- | --- | --- |
| EN-DC | - LTE FDD PCell + FR1 FDD PScell | 15kHz & 10MHz |
|  | - LTE FDD PCell + FR1 TDD PScell | 15kHz & 10MHz |
|  | - LTE FDD PCell + FR2 TDD PScell | 120kHz & 100MHz |
| SA | - FR1 FDD PCell | 15kHz & 10MHz |
|  | - FR1 TDD PCell | 15kHz & 10MHz |
|  | - FR2 TDD PCell | 120kHz & 100MHz |

All the parameters are summarized in table 1, 2, 3 and 4.

# References

[1] R4-1809388, “Test case list for NR”, Ericsson, RAN4 #1807

[2] R4-1809868, “CR on UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and FDD PScell in FR1”, Intel, RAN4 #88

[3] R4-1809869, “CR on UE Timing Advance Adjustment Accuracy Test with FDD PCell in FR1”, Intel, RAN4 #88

[4] R4-1809400, “Way forward on RRM test configurations”, Ericsson, RAN4 #1807