3GPP TSG-RAN WG4 Meeting #88                                                                  R4-1809931

Gothenburg, Sweden, 20 – 24 August 2018

Source:	Samsung

Title:		Discussion on TR 38.826 Clause 6: Evaluation on the coverage
   related issue

Agenda Item:	   10.3

Document for:	Approval

- Intoriduction

This paper is to update TR 38.826 for the study on evaluation for 2 receiver exception in Rel-15 vehicle mounted User Equipment (UE) for NR from the draft TR v0.0.1 [1] based on Section 2.1 of [2]

- Reference

[1] 	R4-1810449, “TR38.826 Skeleton for Study on evaluation for 2Rx exception in rel-15 vehicle mounted UE for NR,” LG Electronics, RAN4#88, Aug 2018

[2] 	R4-1809930, “Coverage related issues for 2Rx RF requirements of vehicle mounted NR UE as exception case,” Samsung, LG Electronics, Volkswagen AG, Toyota InfoTechnology Center, RAN4#88, Aug 2018.

- Text proposal for TR 38.826

<Start of TP>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[6]	ECCREP228, Compatibility studies between Intelligent Transport Systems (ITS) in the band 5855-5925 MHz

# 6	Evaluation on the coverage related issue

The following study provided is comparing 4Rx link budget with 2Rx link budget. It has also been the basis for supporting 4Rx mandate based on higher maximum path loss (MPL).

Figure 6-1: MPL and sensitivity values with 4 RX NR vehicular UE

The following excerpt is also seen from the network industry:

-	It seems not fair to compare the coverage between the 2Rx vehicle device outside the vehicle (exterior 2Rx) and the 4Rx handheld UE inside the vehicle (interior 4Rx handheld UE)

The following is response to the study in Figure 6-1 which considers opinion of automotive industry:

-	While defining a technology, it is very essential to consider the realistic deployment conditions. It cannot be denied that the current discussion is related to vehicular UE.

-	In ECC report 228 [6] and in car manufacturer study, the antenna gain values are discussed for intelligent transportation systems. Figure 2 showcase gain realized in vehicular systems where the value is around 5 dBi and it is also recommended by “ITU”.

Figure 6-2: ITS OBU/TCU antenna gain value around 5 dBi [6]

Antenna system total gain = Antenna gain – cable loss

If cable loss for 2 m is considered, cable loss is equal to 2 dB and 3~5 dBi of antenna gain, then the antenna system total gain for vehicle UE is 1~3 dBi. The parameters depend on operating frequency of antenna.

The following findings in figure 6-3(a-c) from a commercial mobile phone provider (HH UE) showcase the average antenna gain values (passive) for Multi-band & Multi-mode supporting Antenna.

Figure 6-3(a): Antenna performance of commercial HH UE at low frequency

Figure 6-3(b): Antenna performance of commercial HH UE at mid frequency

Figure 6-3(c): Antenna performance of commercial HH UE at high frequency

It can be seen that the handheld UE antenna performance in figure 6-3 are worse compared to vehicular UE antenna performance in figure 6-2 by at least more than 3~4 dB, by considering vehicular antenna system total gain.

It should be noted that additional RF compensators shown in figure 6-4 might be employed in the vehicles to compensate for the RF losses of specific cables without introducing further gain in RF chain. This module can only be considered for car models with TCU and antenna systems where unavoidably high-loss RF chains exist. The utilization of such compensator would even further improve the RF system performance of vehicles.  However, RF compensators intensively increase the implementation complexity.

Figure 6-4: RF compensators used in cars

2 Rx SNR at receiver is ~ 3 dB worse compared to 4 RX receiver. Threrfore, SNR values perceived by a 4Rx handheld UE receiver and 2Rx vehicular UE is comparable, when considering Antenna system total gain (Antenna gain – cable loss). In regular radiation conditions, SNR of 2Rx vehicular UE receiver is better than 4Rx HH UE due to absence of hand held environments and body losses. It is recommended to update the following calculations in Figure 6-1 as follows:

Table 6-1: Updated SNR values for rel.15 V-UE

| SNR receiver values for 4Rx HH-UE (in dB) |  |  | SNR receiver values for 2Rx V-UE (in dB) |  |  |
| --- | --- | --- | --- | --- | --- |
| PBCH | PDCCH | PDSCH 10 Mbps | PBCH | PDCCH | PDSCH 10 Mbps |
| -12 | -8.2 | -7.5 | -8.95 – FFS if FFS = 3: = -11.95 | -4.86 – FFS if FFS = 3: = -7.86 | -4.6 – FFS if FFS = 3: =-7.6 |

Since Table 7.3-2 of [3] considers an improvement in sensitivity of 2.7dB for n7, n38, n41 and of 2.2dB for n77, n78, n79 when using 4Rx antennas, the reference sensitivity improvements for 4Rx can be granted by other means like higher antenna gains compared to handheld devices. Therefore, the sensitivity of 2Rx UE permanently mounted in a vehicle at system level can be considered as comparable to a 4Rx handheld UE at system level. When uplink is considered, vehicular UE shall enable higher coverage range compared to hand held UE.

<End of TP>