3GPP TSG-RAN WG4 Meeting #88	R4-1810192

Gothenburg, SE, 20th – 24th Aug, 2018

Agenda item:	7.13.1.3

Source: 		NTT DOCOMO, INC.

Title: 	Initial simulation results for SDR test

Document for:	Discussion

1. Introduction

This contribution presents our initial simulation results for SDR test. Our general views are shown in our companion contribution [1].

2. Discussion

SDR test should be specified in order to verify proper Layer 1/2 process for the maximum number of DL-SCH transport block bits supported for the UE. Link-level simulations are performed to discuss general test design principle. Major link-level simulation parameters are listed in Table A1. We evaluate throughput performance with the highest MCSs assuming AWGN channel to see peak throughput performance. Three different cases are simulated, which are modelled for LTE re-farming, sub-6 TDD and FR2 operations. Results are shown in Figures 1-3. Effect of the Tx EVM is taken into account in the figures.

(a) 64QAM                　　　　　　　　　      (b) 256QAM

Figure 1: Case 1 ({SCS, CBW}={15kHz, 20MHz}, 2Tx/2Rx)

(a) 64QAM                　　　　　　　　　      (b) 256QAM

Figure 2: Case 2 ({SCS, CBW}={30kHz, 100MHz}, 4Tx/4Rx)

Figure 3: Case 3 ({SCS, CBW}={120kHz, 200MHz}, 2Tx/2Rx, 64QAM)

Observation: Required SNR to achieve 85% throughput when the maximum number of HARQ transmission of 4 is as follows.

| SCS | CBW | Antenna | MCS (index) | Required SNR |
| --- | --- | --- | --- | --- |
| 15 kHz | 20 MHz | 2Tx2Rx | 64QAM (28) | 24.2 dB |
| 15 kHz | 20 MHz | 2Tx2Rx | 256QAM (27) | 29.6 dB |
| 30 kHz | 100 MHz | 4Tx4Rx | 64QAM (28) | 24.3 dB |
| 30 kHz | 100 MHz | 4Tx4Rx | 256QAM (27) | 29.9 dB |
| 120 kHz | 200 MHz | 2Tx2Rx | 64QAM (28) | 24.2 dB |

3. Conclusion

This contribution presents our initial simulation results for SDR test. Observation is derived as follows. Our general views are also shown in our companion contribution [1].

Observation: Required SNR to achieve 85% throughput when the maximum number of HARQ transmission of 4 is as follows.

| SCS | CBW | Antenna | MCS (index) | Required SNR |
| --- | --- | --- | --- | --- |
| 15 kHz | 20 MHz | 2Tx2Rx | 64QAM (28) | 24.2 dB |
| 15 kHz | 20 MHz | 2Tx2Rx | 256QAM (27) | 29.6 dB |
| 30 kHz | 100 MHz | 4Tx4Rx | 64QAM (28) | 24.3 dB |
| 30 kHz | 100 MHz | 4Tx4Rx | 256QAM (27) | 29.9 dB |
| 120 kHz | 200 MHz | 2Tx2Rx | 64QAM (28) | 24.2 dB |

References

[1] 3GPP, NTT DOCOMO, R4-1810471	High level views on SDR requirements, Aug. 2018.

Appendix

Table A1: Simulation Parameters

| Parameters | Case 1 | Case 2 |  | Case 3 |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz | 30 kHz |  | 120 kHz |
| CBW | 20 MHz | 100 MHz |  | 200 MHz |
| MIMO Config. | 2Tx + 2Rx | 4Tx + 4Rx |  | 2Tx + 2Rx |
| Transmission layer | 2 | 4 |  | 2 |
| Duplex mode | FDD | TDD |  | TDD |
| TDD configuration for TDD test cases | - | {7D1S2U} |  | {DDDSU} |
| Modulation (MCS index) | 64QAM (28), 256QAM (27) | 64QAM (28) , 256QAM (27) |  | 64QAM (28) |
| # of HARQ process | 8 | 16 |  | 16 |
| Max # of HARQ transmission | 1, 4 (RV = {0, 2, 3, 1}) |  |  |  |
| SSB transmission periodicity | 160 ms (FDM with PDSCH), 1 SSB only |  | 160 ms (No PDSCH allocation for a slot with SSB), 1 SSB only |  |
| PDSCH allocation | Full allocation |  |  |  |
| Slot length | 14 OFDM symbol |  |  |  |
| PDCCH | 1 OFDM symbol |  |  |  |
| DMRS configuration | Type 1, 1 symbol FL DMRS without AD DMRS |  |  |  |
| FDM of DMRS and PDSCH | Applied | N.A. |  | Applied |
| Phase noise model | - | - |  | No phase noise |
| PTRS configuration | - | - |  | Port 1, per 2PRB in frequency domain, per symbol in time domain (for overhead) |
| CSI-RS configuration | Same as Tx ports (5ms periodicity) |  |  |  |
| CSI type | Type I single panel |  |  |  |
| TRS configuration | N.A. |  |  |  |
| Phase/Time tracking | Ideal |  |  |  |
| Channel model | AWGN |  |  |  |
| Receiver | MMSE for all layers |  |  |  |
| Channel estimation | Realistic |  |  |  |
| Tx EVM | 64QAM=6%, 256QAM 3% |  |  |  |