3GPP TSG-RAN WG4 Meeting #88	R4-1810329

Gothenburg, Sweden, 20 - 24 Aug, 2018

Agenda item:	9.1.2

Source:	Dish Network

Title:	NR intra-band CA requirements <2700MHz

Document for:	Approval

# 1	Introduction

This contribution discusses generic RF requirements for NR intra-band CA <2700MHz. In NR release 15, there were no NR intra-band CA combinations <2700MHz, hence there are no respective generic RF requirements either.

In this meeting, we are aiming at a frame agreement on the principles of defining NR intra-band CA RX requirements for NR bands <2700MHz.

# 2	Discussion

Current NR CA basket #1 [1] includes two CA combinations <2700MHz, CA_n66(2A) and CA_n66B.

In NR release 15, all new NR bands were >2700MHz. The RX requirements for those bands were defined in a new manner, basically using wider blockers than in LTE. For LTE re-farming bands, RX requirements from LTE were re-used. The main reason behind re-using LTE requirements was maintaining the same minimum system performance when LTE band is re-farmed to NR. Another, more kind of practical reason was to copy something that is already defined, mostly due to time constraints during the release.

We propose re-using LTE intra-band CA requirements for intra-band NR CA<2700MHz as well. In most cases, NR intra-band CA <2700MHz is going to be at LTE re-farming bands, hence RAN4 should apply same requirements for those bands as for the legacy system.

Proposal: Generic RF RX requirements for NR intra-band CA (contiguous and non-contiguous) <2700MHz shall be defined re-using respective LTE intra-band CA requirements.

# 3	Conclusions

Principles how to define NR intra-band CA RX requirements <2700MHz were briefly discussed. The following proposal is made.

Proposal: Generic RF RX requirements for NR intra-band CA (contiguous and non-contiguous) <2700MHz shall be defined re-using respective LTE intra-band CA requirements.

# 4	References

[1] RP-181426 “New WID on NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum, (x>=y)”, Ericsson, Nokia, RAN#80