3GPP TSG-RAN WG4 Meeting #88						 R4-1810371

Gothenburg, SE, 20th – 24th Aug, 2018

Title: 	Discussion on BWP SCS restriction based on UE CC BW

Source: 	OPPO

Agenda item:	7.5.1 SCS restriction with BWP operation

Document for:	Discussion

Introduction

In this contribution, we discuss the SCS restrictions of BWP based on the configured UE CC BW. RAN4 should further discuss the remaining issues on SCS restrictions for different cases.

In last RAN4 #87 meeting, we discussed the SCS restriction on BWP operation based on the agreement from previous meetings [1]:

- On RF requirement, we agreed UE RF requirements of configured BWP are based on UE CC BW with no additional BWP related RF requirements. And both single BWP case and multiple BWPs case should be considered.

  - Single BWP case: single CC RF requirements apply with UE CC BW

  - Multiple BWPs case: intra-band CA RF requirements apply with UE CC BW

- On Baseband capability, we agreed how the UE configures it’s RF in order to operate the BWP and meet the CBW based RF requirements is implementation dependent.

And SCS restriction with BWP operation are discussed for both data and SSB. To be highlighted, SSB SCS restriction with BWP operation are remained to be further discussed in this RAN4 #88 meeting.

Discussion

In Rel-15, UE can configure up to 4 BWPs with only one BWP active at a time in one carrier for both UL and DL transmission. And RAN2 signaling is configuring UE with multiple CC BW’s per SCS, and BWP switch may imply switching from one CC BW to another which may cause change in RF requirements that UE has to satisfy.

Companies has pointed out that UE’s will have a baseband limitation in terms of FFT size and SCS of a BWP should be restricted so that UE operating in CC BW and BWP SCS is within its FFT size, especially for intra-band CA BWP switching. So it seems necessary for RAN4 to find out a way on how to make sure UE BWP operating within its baseband capability.

Although how UE configures it’s RF to operate BWP and meet the CBW based RF requirements is implementation dependent, RAN2 has clear definition as “SCS-SpecificCarrier” providing parameters determining the set of UE CC BW for each SCS.

-- ASN1START

-- TAG-SCS-SPECIFIC-CARRIER-START

SCS-SpecificCarrier ::=				SEQUENCE {

offsetToCarrier						INTEGER (0..2199),

subcarrierSpacing					SubcarrierSpacing,

carrierBandwidth					INTEGER (1..maxNrofPhysicalResourceBlocks),

...

}

-- TAG-SCS-SPECIFIC-CARRIER-STOP

-- ASN1STOP

Note: The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

In our view, SCS of a BWP basically should be restricted to ensure that RF requirements exist in TS corresponding BWP SCS and CC BW. For example, in Band X, UE can configure 15KHz SCS with 50MHz max UE CC BW, and 30KHz/60KHz SCS with 100MHz max UE CC BW.

Observation 1:  RAN4 can be open to define a UE capability for UE that will not need any SCS restriction.

Here, we would like to provide serval particular scenarios for further discussion:

- for single BWP:

  - if initial BWP is configured in UE CC BW with 100 MHz, then first active BWP of 40 MHz with 15KHz SCS is not possible with no RF requirement change.  This is because there are no RF requirements defined for 100 MHz BW, 15 KHz SCS.

- for multiple BWPs:

- if UE can support intra-band CA up to max UE BW 100MHz, then either BWP within a component CC of 40MHz with 15KHz is possible, but two DL or UL active BWPs for intra band CA with aggregated BW of 80MHz is not possible. There is a limitation BWP operation with no SCS restriction for intra-band CA.

In one hand, we wonder that this may limit network scheduling for BWP activation or BWP switching. In the other hand, multiple BWPs can only be supported by different carriers in Rel-15. How UE can meet RF requirement of aggregated CC BW and not exceed UE baseband capability like FFT size, still need to be further analyzed. Especially for intra-band CA, RAN4 haven’t considered the SCS limitation for BWP operation within intra-band CA in current spec, only with requirement of reporting CA bandwidth classes (“CA-BandwidthClassNR”). So we suggest RAN4 can clarify this case and decide whether some related requirements are needed or not.

Observation 2: RAN4 should clarify this case and decide whether some related requirements for intra-band CA are needed or not in this meeting.

In addition, RAN2 has an UE RF capability parameter “bwp-WithoutRestriction” which indicates support of BWP operation without bandwidth restriction.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::= SEQUENCE {

supportedBandListNR				SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList		BandCombinationList						OPTIONAL,

appliedFreqBandListFilter			FreqBandList							OPTIONAL

}

BandNR ::=	SEQUENCE {

bandNR							FreqBandIndicatorNR,

modifiedMPR-Behaviour			BIT STRING (SIZE (8))						OPTIONAL,

mimo-ParametersPerBand			MIMO-ParametersPerBand						OPTIONAL,

extendedCP						ENUMERATED {supported}						OPTIONAL,

multipleTCI						ENUMERATED {supported}						OPTIONAL,

bwp-WithoutRestriction			ENUMERATED {supported}						OPTIONAL,

bwp-SameNumerology					ENUMERATED {upto2, upto4}				OPTIONAL,

bwp-DiffNumerology					ENUMERATED {upto4}						OPTIONAL,

crossCarrierSchedulingDL-SameSCS		ENUMERATED {supported}						OPTIONAL,

crossCarrierSchedulingUL-SameSCS		ENUMERATED {supported}						OPTIONAL,

pdsch-256QAM-FR2				ENUMERATED {supported}						OPTIONAL,

pusch-256QAM					ENUMERATED {supported}						OPTIONAL,

ue-PowerClass					ENUMERATED {pc2, pc3}				OPTIONAL,

rateMatchingLTE-CRS				ENUMERATED {supported}						OPTIONAL,

...

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

The bandwidth restriction in terms of BWP for PCell and PSCell means that the bandwidth of a UE-specific RRC configured BWP may not include the bandwidth of initial DL BWP and SSB. For SCell, it means that the bandwidth of BWP may not include SSB.  Assuming the default DL BWP may become initial active DL BWP which can be configured with 15KHz SCS, it is not possible for UE to fall back to initial BWP with restriction if its CC BW is 100 MHz.

Somehow, network may be allowed to configure 15 kHz SSB even with the UE CC BW is larger than 50MHz. If this is the case, then SCS restriction of BWP operation including SSB is dependent from that for data transmission. Especially, if SSB is with 15 kHz SCS, an interruption may be needed for the measurement if CC BW is 100 MHz. Even if UE declares the capability to process SSB and data with different SCS, it is up to UE to ensure that it can meet that for all possible combinations of CC BW’s and SSB SCS.

Observation 3: UE should ensure that it can meet that for all possible combinations of CC BW’s and SSB SCS. And the principle for both SSB and data SCS restriction of BWP should be unified if a UE capability would be defined.

And RRM has discussed 4 scenarios for BWP switching, including

- Scenario 1: The reconfiguration involves changing the center frequency and baseband parameters of the BWP without changing its BW. The reconfiguration may or may not involve changing the SCS.

- Scenario 2: The reconfiguration involves changing the BW and baseband parameters of the BWP without changing its center frequency. The reconfiguration may or may not involve changing the SCS.

- Scenario 3: The reconfiguration involves changing the BW, the center frequency and baseband parameters of the BWP. The reconfiguration may or may not involve changing the SCS.

- Scenario 4: The reconfiguration involves changing only the SCS, where the center frequency, BW and baseband parameters of the BWP remain unchanged.

If a UE capability for SCS restriction is designed to make sure UE can always operate its RF in a same UE CC BW, it may make the baseband and RF adjustment easier, but reduce the flexibility of BWP configuration. In our view, we prefer to do such down selection for UE behavior, do not change the UE operating CC BW if the configured BWP is less than UE configured CC BW.

Observation 4: for UE behavior, do not change the UE operating CC BW if the configured BWP is less than UE configured CC BW.

Conclusions

In this paper, we have such observations on BWP SCS restriction based on UE CC BW:

Observation 1: RAN4 can be open to define a UE capability for UE that will not need any SCS restriction.

Observation 2: RAN4 should clarify this case and decide whether some related requirements for intra-band CA are needed or not in this meeting.

Observation 3: UE should ensure that it can meet that for all possible combinations of CC BW’s and SSB SCS. And the principle for both SSB and data SCS restriction of BWP should be unified if a UE capability would be defined.

Observation 4: for UE behavior, do not change the UE operating CC BW if the configured BWP is less than UE configured CC BW.

References

[1] R4-1801006, WF on wideband operation of NR UE, 3GPP TSG-RAN WG4 #AH-1801 Meeting, San Diego, California, USA, Jan. 22 – 26, 2018