3GPP TSG-RAN4 Meeting #88	R4- 1810509

Gothenburg, Sweden, 20th – 24th August 2018

Source:	Ericsson

Title:	MU framework and values for OTA Base station output power for CATR for FR2

Agenda item:	7.9.4.3.3.1

Document for:	Approval

# Introduction

This contribution presents our view on the MU budget for the OTA BS output power for FR2 for the CATR. We believe that similarly to FR1, the MU budget can be based on the EIRP accuracy budget as described below.

# Discussion

Based on OTA radiated transmit power, the framework on uncertainty budget has been reused.

Some issues remain open for the EIRP accuracy MU budget; one is the instrument uncertainty and the second is the need for an LNA.

We believe that the presented figures represent reasonable MU for instrumentation, but the discusson on the values should be taken separately.

Considering a BS output power TRP of around 30dBm and a pathloss (including gain in the measurement probe and cable losses) of e.g. 70-80dB in the far field, the measured TRP level will be around -40 to -50dBm. Of course, in regions where the power is concentrated there will be array gain and thus the measured power will be greater. This leaves room above a spectrum analyser noise floor where there is meaningful power. Thus, our preliminary view is that there is no need to take into account MU of an LNA for this requirement.

Compact antenna test range uncertainty contributions for OTA Base station output power measurement

| UID | Description of uncertainty contribution | Details in annex of [1] |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | B2-1 |
| 2 | RF power measurement equipment | E |
| 3 | Standing wave between DUT and test range antenna | B2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | B2-4 |
| 5 | QZ ripple DUT | B2-5 |
| 19 | Miscellaneous uncertainty | B2-14 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | E |
| 7 | Mismatch of receiver chain | B2-7 |
| 8 | Insertion loss variation of receiver chain | B2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | B2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | B2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | E |
| 12 | Misalignment positioning system | B2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | B2-1 |
| 14 | Rotary Joints | B2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | B2-3 |
| 16 | Quality of quiet zone | B2-5 |
| 20 | Switching uncertainty | B2-15 |

# Compact antenna test range uncertainty assessment for OTA Base station output power measurement

| UID | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 24.25<f <29.5GHz | Standard uncertainty ui [dB] 37<f <40GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0.2 | 0.2 | Exp. normal | 2 | 1 | 0.1 | 0.1 |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.5 | 0.7 | Gaussian | 1 | 1 | 0.5 | 0.7 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1 | 1 | 0.01 | 0.01 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.3 | 0.3 | Normal | 1 | 1 | 0.3 | 0.3 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.43 | 0.57 | U-shaped | √2 | 1 | 0.3 | 0.4 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.1 | 0.1 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.01 | 0.01 | Normal | 1 | 1 | 0.01 | 0.01 |
| 10 | Influence of the calibration antenna feed cable | 0.21 | 0.29 | U-shaped | √2 | 1 | 0.15 | 0.2 |
| 11 | SGH Calibration uncertainty | 0.52 | 0.52 | Rectangular | √3 | 1 | 0.3 | 0.3 |
| 12 | Misalignment positioning system | 0.2 | 0.2 | Exp. normal | 2 | 1 | 0.1 | 0.1 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |
| 14 | Rotary joints | 0 | 0 | U-shaped | √2 | 1 | 0 | 0 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.43 | 0.43 | Rectangular | √3 | 1 | 0.25 | 0.25 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | [0.86] | [1.03] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | [1.67] | [2.02] |

The basic budget considered here needs to be combined with any Systematic Error from the TRP process; the size of such a component is the subject of a separate discussion.

# Conclusion

This contribution presents a proposal of the measurement uncertainty framework and budget for OTA Base station output power measurements.

# References

TS 37.842, Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

Huawei, R4-1808500, “Draft CR for 37.843”, RAN4#87, May 2018