3GPP TSG-WG RAN4 Meeting #88	R4-1810803

# Gothenburg, Sweden, 20th – 24th August, 2018

Source: 	Nokia, Nokia Shanghai Bell, [T-Mobile USA]

Title: 					TP for TR 37.716-01-01 Introduction of DC_2A_n261A and DC_2A_n261(2A)

Agenda Item: 			9.3.3 DC_R16_1BLTE_1BNR_2DL2UL-Core

Document for:	Approval

# Introduction

This TP introduces two-band EN-DC configurations of DC_2A_n261A and DC_2A_n261(2A) to TR 37.716-01-01.

**************************** Start of changes ********************************************

## 6.2.X	DC_2_n261

### 6.2.X.1	Operating bands for DC

Table 6.2.X.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2_n261 | 2 | n261 | No |

### 6.2.X.2	Configuration for DC

Table 6.2.X.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A_n261A DC_2A_n261(2A) | DC_2A_n261A | 2 | n261A CA_n261(2A) |

Table 6.2.X.3-1: Supported bandwidths per DC band combination of 1 LTE band and 1 NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| DC_2A_n261A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 220 |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 420 |  |  |
| DC_2A_n261(2A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 420 |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 820 |  |  |

### 6.2.X.3	Spurious emission band UE co-existence for DC

Table 6.X.3-1 lists Band 2A +Band n261A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.2.X.3-1: Band 2 and Band n261 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 27500 | 28350 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 55000 | 56700 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 82500 | 85050 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25590 | 26500 | 29350 | 30260 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 24650 | 23680 | 53090 | 54850 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 31200 | 32170 | 56850 | 58610 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 22800 | 21770 | 80590 | 83200 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 33050 | 34080 | 84350 | 86960 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 53000 | 51180 | 58700 | 60520 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 111550 | 108090 | 19860 | 20950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 111850 | 115310 | 34900 | 35990 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 81350 | 78680 | 49270 | 51150 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 86200 | 88870 | 60550 | 62430 |

Based on Table 6.2.X.3-1

- 2nd order harmonics may fall into LTE frequencies of Band 43, n77, n78

- 3rd order harmonic may fall into LTE frequencies of Band 46

- 2nd order IMD products may fall into Rx frequencies of band n257 and n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.X.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.2.X.3-2: 2UL Band 2 +Band n261 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  |  |
| Galileo | 1559 | - | 1591 | YES |  |  |
| GLONASS | 1591 | - | 1610 | YES |  |  |
| GPS | 1563 | - | 1587 | YES |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.2.X.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.2.X.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A_n261A DC_2A_n261(2A) | Bands 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 48, 66, 70, 71, n260 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.2.X.5	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.2.X.6	∆TIB and ∆RIB values

For DC_2-n261, the TIB,c and RIB values are given in the tables below.

Table 6.2.X.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n261 | 2 | 0 |
|  | n261 | 0 |

Table 6.2.X.6-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n261 | 2 | 0 |
|  | n261 | 0 |

**************************** End of changes ********************************************