3GPP TSG-RAN WG4 Meeting #88	R4-1810847

## Gothenburg, Sweden, 20 - 24 August, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | DraftCR | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: OOBB test method MU background |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CATR test method for OOBB to be used for MU evaluation is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of CATR test method for OOBB to be used for MU evaluation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CATR test method for OOBB to be used for MU evaluation remains missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 10.8.3 	CATR

#### 10.8.3.1	General

This method measures the OTA out-of-band blocking in a compact antenna test range.

#### 10.8.3.2	Calibration

Figure 10.8.3.2-1: A typical CATR measurement setup for OOBB

- Path loss calibration for interfering signal, per polarization

- Option 1: Interferer antenna gain is known

- Measure interferer antenna reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓANT.

- Measure cable loss from point D to input of interferer antenna, call this L ANT↔D which is the equivalent of 20log|S21| from the use of a network analyser.

- Calculate the combined total path loss from D to interferer antenna using the following equating

Lcal = L ANT↔D + 10log(1 - |ΓANT|2) – Gia, where10log1-ΓSGH2 10log(1 - |ΓANT|2)  is the compensation for interferer antenna connector return loss,GSGH Gia GSGH is the known gain of the interferer antenna

- Calculate the free space path loss between DUT and interferer antenna based on the distance between them, call this as Lfspl.

- Calculate the test path loss by formula L = Lcal – Lfspl. This is the total path loss between D↔A.

- Option 2: Interferer antenna gain is not known

a)	Measure SGH (or other calibrated reference antenna) reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓSGH ΓSGH. .

b)	Measure cable loss from point C to input of SGH, call this LC↔SGH LC↔SGH  which is the equivalent of 20log|S21| from the use of a network analyser.

c)	Calculate the combined total path loss from C→A by using the following expression LSGHcal=Lc,SGH+10log1-ΓSGH2-GSGH LSGHcal =LC↔SGH LC↔SGH  + 10log(1 - |ΓSGH|2) - GSGHLSGHcal=Lc,SGH+10log1-ΓSGH2-GSGH, where10log1-ΓSGH2 10log(1 - |ΓSGH|2)  is the compensation for SGH connector return loss,GSGH GSGH GSGH is the known gain of the reference SGH.

d)	Connect SGH and C↔A cable.

e)	To remove polarization(s) mismatch between interferer antenna and SGH use positions to position the SGH in the beam peak direction of interferer antenna.

f)	Measure path loss D→C with network analyzer LD→C = 20log|S21|.

g)	Calculate the test path loss compensation factor. This is the total path loss between D↔A using the results from step c and f. L = LSGHcal LSGHcal  - LD→C.

- Path loss calibration for wanted signal

- According to section 10.3.2.1.2.2 of TR 37.842

#### 10.8.3.3	Procedure

- Place the AAS BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system

- Align the manufacturer declared coordinate system orientation of the AAS BS with the test system and set the AAS BS in the declared direction to be tested.

- Ensure the polarisation is accounted for such that all the power from the test antenna and interferer antenna are captured by th e AAS BS under test.

- Set the AAS BS and measurement equipment to transmit according to the appropriate test configuration.

- The AAS BS receives the wanted signal and the interferer signal for all supported polarizations (single or dual), in the reference direction from the test antenna(s).

In case interferer antenna needs to be changed during the test, the calibration procedure for interferer power level needs to be repeated.

#### 10.8.3.4	MU assessment

##### 10.8.3.4.1 	MU Budget

##### 10.8.3.4.2 	MU Value

### 10.8.4 	Shielded Anechoic Chamber

------------------------------ End of modified sections ------------------------------