3GPP TSG-RAN WG4 Meeting #88	R4-1810950

Gothenburg, Sweden, 20th- 24th August, 2018

Agenda item:		7.11.4.2.2

Source:	MediaTek Inc.

Title:	Discussion on inter-frequency requirements

Document for:		Discussion

# 1	Introduction

In last RAN4 #AH1807 meeting, the option 1 of gap sharing schemes had been agreed [1]:

- Applicability of measurement gap sharing factor: - Option 1: only gap occasions that both intra-frequency and inter-frequency/inter-RAT MOs exist

This agreement has impact on the design of inter-frequency measurement requirement. The modifications of Alt.1 and Alt.2 are needed. Beside, in our previous paper [2], we give some observations and evaluation results to address the issue of inter-frequency measurement requirement Alt. 3: UE can’t always obtain sufficient sample number in a measurement period. Considering the difficulty of listing all the fail cases in the spec, in this paper we propose the modified version of Alt. 3 to solve this issue. The idea is borrowed from Alt. 2 but still based on the assumption that gap occasion based round-robin inter-frequency measurement scheduling method [3] is used. Regarding the issue of 2DRX cycle configured by MN and SN, we suggest to define the minimum requirements to facilitate the UE implementation.

# 2	Inter-frequency Measurement Requirement

Based on the agreement in last meeting, gap sharing factors are only applied on gap occasions that both intra-frequency and inter-frequency/inter-RAT MOs exist. The inter-frequency measurement requirement needs to distinguish the gap occasions that need the gap sharing factors or not. The principle of Alt. 3 has addressed this issue in the formula as follows:

Tinter,#ialt3=M×Nscaling#i×max⁡(MGRP,TSMTC#i), i=1,2..Nfreq.

where Nscaling#i=Δij=0GMGRP-1εi,j

Detail definition of each parameter can be found in Appendix A. The scaling factor Nscaling#i considers the number θj and the type (intra-freq. or inter-freq.) of observed MOs in each gap occasion. It can be interpreted as the average measurement opportunity of target MO #i. However, as we explain in our previous paper [2], an average value means that UE does not always get an exact number in every measurement period. Sometimes more, but some time less.

To address this problem and also to provide a compromise solution between Alt.2 and Alt.3, we suggest to assume lowest measurement opportunity to calculate the scaling factor, i.e.,

- when it doesn’t need gap sharing in gap occasion #j
   εi,j=1θ, where θ=max0≤j'≤GMGRP-1 θj',   ,

- when it needs gap sharing in gap occasion #j

  - For an intra-frequency layer #i

εi,j=1Kintra×θintra, where   θintra =max 0≤j'≤GMGRP-1 θintra,j'

  - For an inter-frequency layer #i

εi,j=1Kinter×θinter, where   θinter =max0≤j'≤GMGRP-1 θinter,j'

This relaxation can provide UE flexibility on scheduling and there is no need to add ceiling function. Therefore, we propose

Proposal 1: The average measurement opportunity εi,j of inter-frequency measurement requirement Alt.3 is replaced with lowest measurement opportunity. When it doesn’t need gap sharing in gap occasion #j 
εi,j=1θ, where θ=max0≤j'≤GMGRP-1 θj'.
When it needs gap sharing in gap occasion #j, 
for an intra-frequency layer #i 
εi,j=1Kintra×θintra, where θintra =max 0≤j'≤GMGRP-1 θintra,j'
and for for an inter-frequency layer #i
εi,j=1Kinter×θinter, where θinter =max0≤j'≤GMGRP-1 θinter,j'.

# 3	Inter-frequency Measurement Requirement in DRX Mode

In DRX mode, it is agreed in last meeting that the power consumption issue for non-aligned SMTC periodicity and DRX cycle should be taken into account. The required sample number is multiplied with 1.5 when DRX cycle length ≤ 320ms and the overall formula becomes

Proposal 2: In DRX mode, the inter-frequency measurement requirement is  ceil1.5× 8×maxMGRP,TSMTC,TDRX×Δij=0GMGRP-1εi,j, when DRX cycle length is less and equal to 320ms.

In RA2#99 meeting, DRX configuration for EN-DC was discussed. In general, the principle of separate DRX configurations in LTE-DC was applied in EN-DC. When UE conducts the intra-frequency measurement, it is intuitive that the requirement follows the DRX configured to the cell groups that the serving cell is located. However, when UE conducts the inter-frequency measurement, which DRX configuration to be used for the measurement requirement become more complicated. For example:

- When both MCG and SCG exist, and MN configures a measurement object in FR2. UE need to use an RF chain in SCG to measure this frequency.

- When both MCG and SCG exist, and SN configures a measurement object in FR1. It is a UE implementation to use an RF chain in MCG or SCG to measure this frequency.

Above are two cases that one RF chain might need to meet the requirements based on 2 different DRX cycles. In [4], we have shown the evaluation on UE power consumption with 2 different DRX configuration assumptions and concluded that up to 13.5% additional UE power is consumed. Although separate DRX configurations are provided from MN and SN, the information can be exchanged between them. UE vendors understand the difficulty of DRX cycles alignment for Network. To accommodate the high power consumption issue, which might cause bad UE experience, we suggest RAN4 to define the minimum requirement and give UE flexibility to enter the power saving mode.

Proposal 3: RAN4 to define the minimum requirement in DRX mode and provide UE flexibility to enter the power saving mode.

RAN4 should clarify which DRX configuration will be used in the inter-frequency measurement requirement. Two principles are suggested:

- Requirements should not limit the UE implementation.

- Requirements should consider the UE power consumption.

Therefore, we list all the possible situations in Table 2.

Proposal 4: For inter-frequency measurement, the delay relay requirement follows the DRX table shown below

Table 1: Rule to select DRX cycle configured by MN or SN.

| DRX On/Off |  | MO configured by MN | MO Configured by SN |
| --- | --- | --- | --- |
| DRXMN | DRXSN |  |  |
| ON | OFF | DRXMN | DRXMN |
| OFF | ON | DRXSN | DRXSN |
| ON | ON | Max{ DRXMN, DRXSN } |  |

# 4	Summary

In this contribution, we propose

Proposal 1: The average measurement opportunity εi,j of inter-frequency measurement requirement Alt.3 is replaced with lowest measurement opportunity. When it doesn’t need gap sharing in gap occasion #j 
εi,j=1θ, where θ=max0≤j'≤GMGRP-1 θj'.
When it needs gap sharing in gap occasion #j, 
for an intra-frequency layer #i 
εi,j=1Kintra×θintra, where θintra =max 0≤j'≤GMGRP-1 θintra,j'
and for for an inter-frequency layer #i
εi,j=1Kinter×θinter, where θinter =max0≤j'≤GMGRP-1 θinter,j'.

Proposal 2: In DRX mode, the inter-frequency measurement requirement is  ceil1.5× 8×maxMGRP,TSMTC,TDRX×Δij=0GMGRP-1εi,j, when DRX cycle length is less and equal to 320ms.

Proposal 3: RAN4 to define the minimum requirement in DRX mode and provide UE flexibility to enter the power saving mode.

Proposal 4: For inter-frequency measurement, the delay relay requirement follows the DRX table shown below

Table 1: Rule to select DRX cycle configured by MN or SN.

| DRX On/Off |  | MO configured by MN | MO Configured by SN |
| --- | --- | --- | --- |
| DRXMN | DRXSN |  |  |
| ON | OFF | DRXMN | DRXMN |
| OFF | ON | DRXSN | DRXSN |
| ON | ON | Max{ DRXMN, DRXSN } |  |

# 5	Reference

[1] R4-1809353, Discussion summary for UE measurement gap AI 5.2.3, Intel, Jul. 2018.

[2] R4-1808926, Discussion on requirements of inter-frequency measurement, Mediatek, Jul. 2018.

[3] R4-1803787, Further aspects of measurement gap design for NR. Multiple layers, Ericsson, Apr. 2018.

[4] R2-1801530, “UE Power Consumption with DRX Coordination in EN-DC”, Mediatek, Jan. 2018.

# 6	Appendix A

For UE which support and are configured with per FR gaps, Nscaling,i is defined on a per frequency range basis, considering the gap pattern and carriers configured in the relevant frequency range. For UE which are configured with per UE gaps, Nscaling,i is defined considering the per UE gap pattern and all confifured carriers.

Considering that there are Nfreq,NSA carriers to be measured, denote each carrier with index i where i={0…Nfreq,NSA-1}

Determine the gap utilization reputation period G

G=max(MGRP, SMTC period0, … SMTC periodi , PRS period1,…))

Denote each gap in the period G with index j where j={0…(G/MGRP)-1}

For each carrier i, and each gap j, determine if the carrier i is a candidate to be measured in the measurement gap j, considering its SMTC configuration.

An NR carrier is a candidate to be measured if its SMTC occasion at least partially coincides in time with the considered measurement gap excluding RF switching time.

The result of this determination is

λi,j=1 if carrier i is a candidate to be measured in gap j

λi,j=0 otherwise

For each measurement gap, count the number of competing measurement objects according to

θj=i=0Nfreq,NSA-1λi,j

If gap sharing is configured with non-equal sharing, for each measurement gap, count the number of competing intra-frequency measurement objects according to

θintra,j=i=0Nfreq,NSA-1λi,jXi, where Xi=1,  if carrier i is intra-frequencyXi=0,  otherwise

θinter,j=i=0Nfreq,NSA-1λi,jYi ,where Yi=1,  if carrier i is inter-frequencyYi=0,  otherwise

For each measurement gap j and each carrier i, compute the assumed measurement scaling factor according to the following

- If λi,j=1 and carrier i involves a PRS measurement with periodicity larger than largest configured NR SMTC periodicity then

εi,j=1 and εk,j=0 for all k≠i, and

do not evaluate rules (2), (3), (4) and (5), for gap j.

- If λi,j=1, measGapSharingScheme is not “Equal splitting” and θintra,j>0 and θinter,j>0, and carrier i is an intra-frequency measurement object then  εi,j=K1Nintra,j

εi,j=1Kintra×θintra, where θintra=max 0≤j'≤GMGRP-1 θintra,j'.εi,j=1Kintra×Nintra,j

- If λi,j=1, measGapSharingScheme is not “Equal splitting” and θintra,j>0 and θinter,j>0, and carrier i is an inter-frequency/inter-RAT measurement object then

εi,j=1Kinter×θinter, where θinter=max0≤j'≤GMGRP-1 θinter,j'.εi,j=1Kintra×Nintra,j

- If λi,j=1 and measGapSharingScheme is “Equal splitting” then

εi,j=1θ, where  θ=max0≤j'≤GMGRP-1 θj'.

- If  λi,j=0 then

εi,j=0.

For each carrier i, compute the total number of measurement opportunities according to

Δi=j=0GMGRP-1λi,j

The scaling factor for carrier i is then determined according to

Nscaling,i=Δij=0GMGRP-1εi,jNscaling,i=Δij=0GMGRP-1εi,jNscaling,i=Δiceilj=0GMGRP-1εi,j