3GPP TSG-RAN WG4 Meeting #88	R4-1810959

# Gothenburg, Sweden, 20th – 24th, August 2018

Agenda item:	7.6.14.3

Source: 	Intel Corporation

Title: 	ACS and IBB for intra-band contiguous CA

Document for:	Approval

# Introduction

The definition of ACS and IBB for intra-band contiguous CA with two component carriers has been defined in [1]. The definition for intra-band contiguous CA for these requirement only consider the BW Class C, in which the aggregated channel BW starts from 110 MHz up to 200 MHz. In this contribution we provide our view for the requirements of ACS/IBB for intra-band contiguous CA for the BW F, in which the aggregated BW starts from 60 MHz to 100 MHz.

# Discussion

The definition for intra-band contiguous CA for ACS and IBB requirement only considers the BW Class C. This contribution provides a proposal for the requirements of ACS/IBB for intra-band contiguous CA for the BW F, in which the aggregated BW starts from 60 MHz to 100 MHz.

## Background

For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class with associated bandwidth combination sets. In this contribution we concentrate on intra-band contiguous CC for two components for Class F, as shown in the table below.

Table 1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel_CA ≤ BWChannel,max | 1 |  |
| B | 20 MHz ≤ CBW ≤ 100 MHz | 2 |  |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE: BWChannel, max is maximum channel bandwidth supported among all bands in a release |  |  |  |

In subsection 5.5A.1 Configuration for intra-band contiguous CA in [1], the following NR CA configuration combinations set were defined as in Table 2. The NR CA configuration consider NR bands such as n77, n78 and n79.

Table 2: CA bandwidth classes

| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  | aggregated bandwidth (MHz]) |
| --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for the other carrier (MHz |  |
| CA_n77F, CA_n78F, CA_n79F |  | 40 | 20 |  | 60 |
|  |  | 50 | 20 |  | 70 |
|  |  | 40 | 40 |  | 80 |
|  |  | 40 | 50 |  | 90 |
|  |  | 50 | 50 |  | 100 |

The UE shall fulfil the minimum requirement specified in the table below for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm. Table 3 considers only the ACS level for BW Class C from 110 MHz up to 200 MHz.

Table 3: ACS level for BW Class C

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz |
| ACS | dB | [33] | [33] | [33] | [33] | [33] |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 160 MHz | 180 MHz | 200 MHz |  |  |
| ACS | dB | [33] | [33] | [33] |  |  |

For IBB intra-band contiguous carrier aggregation with two component carriers and aggregated bandwidth BWChannel_CA, the UE shall fulfil the minimum requirement specified in Table 4. Similar to the ACS requirement, the definition to IBB intra-band contiguous considers only BW Class C.

Table 4: In-band Blocking for BW Class C

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | [-56] | [-44] |
| n77, n78, n79 | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and BW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3CBW to FDL_high + 3CBW |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

## ACS requirements for intra-band contiguous CA for BW Class F

Since the intra-band contiguous CA configuration considered in [1] is for NR bands n77, n78 and n79. We assume for the ACS level 33 dB for all channel bandwidths as previously defined for the NR bands and for the interferer BW we suggest to keep the agreement to have the interferer BW the same as the wanted signal BW.

Table 5: ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| ACS | dB | [33] | [33] | [33] | [33] | [33] |

Table 6: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + [45.5] dB |  |  |  |  |
| BWinterferer | MHz | 60 | 70 | 80 | 90 | 100 |
| Finterferer (offset) | MHz | 60 / -60 | 70 / -70 | 80 / -80 | 90 / -90 | 100 / -100 |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Table 7: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in transmission bandwidth configuration | dBm | [-56.5] |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 60 | 70 | 80 | 90 | 100 |
| Finterferer (offset) | MHz | 60 / -60 | 70 / -70 | 80 / -80 | 90 / -90 | 100 / -100 |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

## In-band Blocking requirement for intra-band contiguous CA for BW Class F

Since the intra-band contiguous CA configuration considered in [1] is for NR bands n77, n78 and n79. We are considering for the power in transmission BW configuration 6 dB for all channel bandwidths and for the interferer BW we suggest to keep the agreement to have the interferer BW the same as the wanted signal BW.

Table 8: In-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 60 | 70 | 80 | 90 | 100 |
| FIoffset, case 1 | MHz | 90 | 105 | 120 | 135 | 150 |
| FIoffset, case 2 | MHz | 150 | 175 | 200 | 225 | 250 |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Table 9: In-band blocking for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | [-56] | [-44] |
| n77, n78, n79 | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and BW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3CBW to FDL_high + 3CBW |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Proposal 1:	The definition of ACS and In-band Blocking intra-band contiguous CA for BW class F should be considered in TS 38.101-1

Proposal 2: The requirements for ACS and In-band Blocking for intra-band contiguous CA for BW class F should be defined as given in Tables 5, 6, 7, 8 and 9.

# Conclusion

In this contribution, we propose the definition of the ACS and In-Band Blocking requirement for intra-band contiguous CA for BW F.

Proposal 1:	The definition of the ACS and In-band Blocking intra-band contiguous CA for BW class F should be considered in TS 38.101-1

Proposal 2: The requirements for ACS and In-band Blocking for intra-band contiguous CA for BW class F should be defined as given in Tables 5, 6, 7, 8 and 9.

# References

- 3GPP TS 38.101-1 V15.1.0 (2018-06)