3GPP TSG-RAN WG4 Meeting #88	R4-1810960

# Gothenburg, Sweden, 20th – 24th, August 2018

Agenda item:	7.6.14.4

Source: 	Intel Corporation

Title: 	Out-of-band Blocking for intra-band contiguous CA

Document for:	Approval

# Introduction

The definition of Out-of-band blocking for intra-band contiguous CA with two component carriers has been defined in [1]. The definition for intra-band contiguous CA for these requirement only consider the BW Class C, in which the aggregated channel BW starts from 110 MHz up to 200 MHz. In this contribution we provide our view for the requirements of out-of-band blocking for intra-band contiguous CA for the BW F, in which the aggregated BW starts from 60 MHz to 100 MHz.

# Discussion

The definition for intra-band contiguous CA for out-of-band blocking requirement only considers the BW Class C. This contribution provides a proposal for the requirement of out-of-band blocking for intra-band contiguous CA for the BW F, in which the aggregated BW starts from 50 MHz to 100 MHz.

## Background

For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class with associated bandwidth combination sets as shown in the table below. In this contribution we concentrate on intra-band contiguous CC for two components (Class C and Class F).

Table 1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel_CA ≤ BWChannel,max | 1 |  |
| B | 20 MHz ≤ CBW ≤ 100 MHz | 2 |  |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE: BWChannel, max is maximum channel bandwidth supported among all bands in a release |  |  |  |

In subsection 5.5A.1 Configuration for intra-band contiguous CA in [1], the following NR CA configuration combinations set were defined as in the table below. The NR CA configuration are for NR bands n77, n78 and n79.

Table 2: CA bandwidth classes

| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  | aggregated bandwidth (MHz]) |
| --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for the other carrier (MHz |  |
| CA_n77F, CA_n78F, CA_n79F |  | 40 | 20 |  | 60 |
|  |  | 50 | 20 |  | 70 |
|  |  | 40 | 40 |  | 80 |
|  |  | 40 | 50 |  | 90 |
|  |  | 50 | 50 |  | 100 |

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6.X and Tables 7.6.Y in [1]. Table 3 provides only out-of-band blocking parameter for BW Class B.

Table 3: Out-of-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 160 MHz | 180 MHz | 200 MHz |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below | REFSENS + channel specific value below | REFSENS + channel specific value below |  |  |
|  | dB | 9 | 9 | 9 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below …. |  |  |  |  |  |  |

## Out-of-band blocking requirement for intra-band contiguous CA for BW Class F

Since the intra-band contiguous CA configuration considered in [1] is for NR bands n77, n78 and n79. We are considering for the power in transmission BW configuration 9 dB for all channel bandwidths as previously defined for the NR bands.

Table 4: Out-of-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below …. |  |  |  |  |  |  |

Table 5: Out of-band blocking for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -45 | -30 | -15 |
|  | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3CBW) or FDL_high + MAX(200,3CBW) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3CBW) or FDL_high + MAX(150,3CBW) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

Proposal 1:	The definition of the Out-of-band Blocking intra-band contiguous CA for BW class F should be considered in TS 38.101-1.

Proposal 2: The requirement for Out-of-band Blocking for intra-band contiguous CA for BW class F should be defined as given in Tables 4 and 5.

# Conclusion

In this contribution, we propose the definition of the Out-of-Band Blocking requirement for intra-band contiguous CA for BW F.

Proposal 1:	The definition of the Out-of-band Blocking intra-band contiguous CA for BW class F should be considered in TS 38.101-1.

Proposal 2: The requirement for Out-of-band Blocking for intra-band contiguous CA for BW class F should be defined as given in Tables 4 and 5.

# References

- 3GPP TS 38.101-1 V15.1.0 (2018-06)