3GPP TSG-RAN WG4 Meeting #AH88                	R4-1811033

Gothenburg, Sweden, Aug 20th – 24th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Measurements of intra-frequency and inter-frequency NR cells in NR idle mode

Agenda item:	7.11.5

Document for:	Discussion

Introduction

In the previous meetings, RAN4 has discussed the measurement requirements for intra-frequency and inter-frequency NR cells in NR idle mode. Latest updates were agreed in the Montreal meeting in [1]. However, the measurements in FR2 are still open. In this contribution we discuss the measurement requirements in FR2 in NR idle mode.

Discussion

In [1], RAN4 has specified the measurement requirements of intra-frequency and inter-frequency NR cells in NR idle mode as below:

Intra-frequency: Table 4.2.2.3-1 : Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_intra [s] (number of DRX cycles). 0.32; 11.52 x N1 [36 x N1]; 1.28 x N1 (4 x N1); 5.12 x N1 (16 x N1). 0.64; 17.92 x N1 [28 x N1]; 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 [25 x N1]; 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 [23 x N1]; 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note1: N1=[TBD] for frequency range FR2, and N1=1 for frequency range FR1.. Inter-frequency: Table 4.2.2.4-1 : Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter DRX cycle length [s]; Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_inter [s] (number of DRX cycles). 0.32; 11.52 x N1 [36 x N1]; 1.28 x N1 (4 x N1); 5.12 x N1 (16 x N1). 0.64; 17.92x N1 [28 x N1]; 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 [25 x N1]; 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 [23 x N1]; 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note1: N1=[TBD] for frequency range FR2, and N1=1 for frequency range FR1..

For FR2, since there is some additional consideration as UE Rx beamforming, the scaling factor for FR2 is TBD and left for further discussion in last meeting.

Since UE Rx beam forming is related to UE implementation, the scaling factor N for idle mode in FR2 should be discussed – whether or not it is necessary to account in idle mode.

In idle mode, the requirements are already very relaxed. To consider the balance between the power consumption and mobility performance, idle mode already supports very relaxed requirements for UE measurement requirements. Idle mode DRX can anyway be quite long and extending further by scaling due to UE Rx beam forming, will lead to challenges for mobility performance in FR2. Some proposals in Montreal meeting for the scaling factor for FR2 were raised as 2, 4, 8.  We listed the direct value for them in table 1, table 2, and table 3. From the tables, we could see that it is really too long. For example, in the condition that DRX cycle is 2.56s, currently the base requirements for detection is 58.88s, if the scaling factor is 8, then the detection time needs almost 8 minutes, it will be too long and not acceptable to detect a cell.

Table 1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra in FR2 with N1=2.

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 23.04 (72) | 2.56 (8) | 10.24 (32) |
| 0.64 | 35.84 (56) | 2.56 (4) | 10.24 (16) |
| 1.28 | 64 (50) | 2.56 (2) | 12.8 (10) |
| 2.56 | 117.76 (46) | 5.12 (2) | 15.36 (6) |

Table 2: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra in FR2 with N1=4.

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 46.08 (144) | 5.12 (16) | 20.48 (64) |
| 0.64 | 71.68 (112) | 5.12 (8) | 20.48 (32) |
| 1.28 | 128 (100) | 5.12 (4) | 25.6 (20) |
| 2.56 | 235.52 (92) | 10.24 (4) | 30.72 (12) |

Table 3: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra in FR2 with N1=8.

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 92.16 (288) | 10.24 (32) | 40.96 (128) |
| 0.64 | 143.36 (224) | 10.24 (16) | 40.96 (64) |
| 1.28 | 256 (200) | 10.24 (8) | 51.2 (40) |
| 2.56 | 471.04 (184) | 20.48 (8) | 61.44 (24) |

Considering these facts, we propose that the scaling factor is not applied in idle mode in FR2. If UE Rx beam forming scaling is used in FR2, then the base requirements need to be tightened to obtain feasible mobility performance in idle mode.

- Measurements requirement in NR intra-frequency cells in FR2 in NR idle mode could be:

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note1: UE measures each Rx beam at least once per DRX cycle. |  |  |  |

- Measurements requirement in NR inter-frequency cells in FR2 in NR idle mode could be:

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note1: UE measures each Rx beam at least once per DRX cycle. |  |  |  |

Additionally, it would need to be clarified that it is assumed that the measures all UE Rx beamforming once per idle mode DRX cycle. Proposals are included in our CR [2].

Conclusion

In this contribution we have discussed the measurement requirements of intra-frequency and inter-frequency NR cells in FR2 in NR idle mode. We have made the following proposals:

- Measurements requirement in NR intra-frequency cells in FR2 in NR idle mode could be:

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note1: UE measures each Rx beam at least once per DRX cycle. |  |  |  |

- Measurements requirement in NR inter-frequency cells in FR2 in NR idle mode could be:

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note1: UE measures each Rx beam at least once per DRX cycle. |  |  |  |

References

- 3GPP TS 38.133 v15.2.1

- R4-1811034, CR for 38.133 measurement in intra-F and inter-F in idle mode, Nokia, Nokia Shanghai Bell