3GPP TSG-RAN WG4 Meeting #88	R4-1811131

Gothenburg, Sweden, 20th – 24th August 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.141-2 – Overview of radiated Tx and Rx requirements (4.13)

Agenda item:	7.9.4.2.2

Document for:	Approval

# 1	Introduction

This document proposes to add a new subsection, entitled 4.13 Overview of radiated transmitter and receiver requirements to TS 38.141-2 [1].

Constructive comments are encouraged.

# 4	References

- After Montreal Version, Draft TS 38.141-2 (v0.3.0), Huawei

# 5	Text proposal

Draft TS 38.141-2 v0.3.0_afterMontreal

--------------Start of text proposal-------------

### 4.13	Overview of Tx and Rx requirements

Table 4.13-1: Overview of radiated Tx requirements

| Tx requirement |  | Classification | Notes |
| --- | --- | --- | --- |
| Radiated transmit power |  | Directional – OTA peak directions set | This requirement is based on Rel-13 AAS BS requirement for EIRP accuracy |
| OTA BS output power |  | TRP |  |
| OTA output power dynamics |  | Directional – OTA peak directions set | Conformance testing is carried in the reference direction |
| OTA transmit ON/OFF power |  | Co-location |  |
| OTA transient period |  | Co-location |  |
| OTA modulation quality |  | Directional – OTA coverage range | Conformance testing is carried in the reference direction and the maximum directions of the OTA coverage range on each axis |
| OTA frequency error |  | Directional – OTA coverage range | Conformance testing is carried out in the reference direction |
| OTA time alignment error |  | Directional – OTA coverage range | Conformance testing is carried out in the reference direction |
| OTA occupied bandwidth |  | Directional – OTA coverage range | Conformance testing is carried out in the reference direction |
| OTA ACLR |  | TRP |  |
| OTA operating band unwanted emission |  | TRP |  |
| OTA transmitter spurious emission | Mandatory | TRP | For FR1 and FR2 |
|  | Protection of the BS receiver of own or different BS | Co-location | For FR1 only |
|  | Additional spurious emissions | TRP | For FR1 only |
|  | Co-location with other base stations | Co-location | For FR1 only |
| OTA transmitter intermodulation |  | Co-location | For FR1 only |

Table 4.13-2: Overview of radiated Rx requirements

| Rx requirement |  | Classification | Applicability levels |  | Coverage range |  | Number of conformance directions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 | FR1 | FR2 |  |
| OTA sensitivity |  | Single direction | - | Not defined | Receiver target redirection range |  | 5 |
| OTA reference sensitivity |  | Single direction | OTA REFSENS | OTA REFSENS | OTA REFSENS RoAoA |  | 5 |
| OTA Dynamic range |  | Single direction | OTA REFSENS | Not defined | OTA REFSENS RoAoA |  | 1 |
| OTA adjacent channel selectivity |  | Single direction | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-band blocking |  | Single direction | OTA REFSENS and minSENS | OTA REFSENS | OTA REFSENS RoAoA and minSENS RoAoA | OTA REFSENS RoAoA | 5 |
| OTA out-of-band blocking | mandatory | Single direction | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
|  | Co-location with other base stations | Co-location | For FR1 only |  | N/A |  | N/A |
| OTA receiver spurious emissions |  | TRP | - |  | n/a |  | - |
| OTA receiver intermodulation |  | Single direction | OTA REFSENS and minSENS | OTA REFSENS | OTA REFSENS RoAoA and minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-channel selectivity |  | Single direction | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |

--------------end of text proposal-------------