3GPP TSG-RAN Working Group 4 Meeting #88	R4-1811133

Gothenburg, Sweden, 20th August – 24th August 2018

Source:	Nokia and Globalstar

Title:	TP to TR 36.791 (E-UTRA 2.4 GHz TDD Band for US): Frequency Band Arrangement and Regulatory Background

Document for:	Approval

Agenda Item:	          6.2.1

## 1	Introduction

The FCC has recently adopted rules for terrestrial use of the 2.4 GHz MSS band.  This band is authorized for TDD

operation and spans 11.5 MHz from 2483.5 MHz to 2495 MHz.  A new work item was created in RAN#79, to create

specifications for a corresponding new E-UTRA operating band [1].

The WI has a study phase, with the following core objectives:

- Define impact on 2.4 GHz Bluetooth operations

- Define impact on 2.4 GHz 802.11 operations

- Address potential Bluetooth and 802.11 coexistence issues

The present contribution provides a TP to TR 36.791 (E-UTRA 2.4 GHz TDD Band for US) on the frequency band arrangement and regulatory background of the 2.4 GHz Terrestrial Band.  This information is provided in support of the study phase of the approved work item (WI) for a new US 2.4 GHz TDD Band for LTE.  FCC regulations for the protection of both licensed services above 2495 MHz and unlicensed services below 2483.5 MHz are outlined.

## 2	References

[1]	  RP-180583, New WI Proposal for E-UTRA 2.4 GHz TDD Band, Globalstar, RAN#79

[2]	  3GPP TR 21.905 “Vocabulary for 3GPP Specifications.”

[3]	Terrestrial Use of the 2473[2483.5]-2495 MHz Band for Low-Power Mobile Broadband Networks; Amendments to Rules for the Ancillary Terrestrial Component of Mobile Satellite Service Systems, IB Docket No. 13-213, Report and Order, FCC 16-181

[4]	47 CFR Part 15.247.  “Operation within the bands 902-928 MHz, 2400-2483.5 MHz, and 5725-5850 MHz.”

[5]	Satellite Communications, Title 47, Part 25 of e-CFR, https://www.law.cornell.edu/cfr/text/47/part-25

## 3 	Definitions, Symbols and Abbreviations

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [2].

ATC	Ancillary Terrestrial Component

BAS	Broadcast Auxiliary Service

BRS	Broadband Radio Service

BS	Base Station

DL	Downlink

CFR		Code of Federal Regulation

EBS	Educational Broadband Service

EIRP	Effective Isotropic Radiated Power

E-UTRA	Evolved UTRA

FCC	Federal Communications Commission

FDD	Frequency Division Duplex

ISM	Industrial Scientific Medical

LTE	Long Term Evolution

MSS	Mobile Satellite Service

NOS	Network Operating System

RF	Radio Frequency

RX	Receiver

TDD	Time Division Duplex

TX	Transmitter

UE	User Equipment

US	United States

## 4	Background

A recent FCC Report and Order has created a band is authorized for TDD operation and spans 11.5 MHz from 2483.5 MHz to 2495 MHz [3].

- Figure 4-1:  US 2.4 GHz Terrestrial Band

As is customary, regulations contained within 47 CFR Part 25 for this allocation are designed to protect licensed services above 2495 MHz from harmful interference.  In regulating the 2.4 GHz Terrestrial Band, the FCC took additional and unusual steps to create special protections for Part 15 unlicensed services operating in spectrum below 2483.5 MHz.

This contribution presents an overview of FCC regulations governing the 2.4 GHz Terrestrial Band and their relationship to co-existence with adjacent band licensed and unlicensed services.  Further description of the technical basis for unlicensed band co-existence regulations may be found in a companion TP to TR 36.791 (R4-1811134)

<<< Start of TP>>>

5	Frequency Band Arrangement and Regulatory Background

## 5.1	Frequency band arrangement and channel bandwidths

## 5.1.1	Frequency band arrangement

The proposed band plan for the E-UTRA 2.4 GHz TDD Band for US (2.4 GHz Terrestrial Band) is presented below.  As shown in Table 5.1.1-1, the TDD configuration of the band places BS receive / UE transmit and BS transmit / UE receive on the same frequencies.  Channels may be placed from 2483.5 MHz to 2495 MHz.

### Table 5.1.1-1: Operating band arrangement

| Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| FUL_low | – FUL_high |  | FDL_low | – | FDL_high |  |
| 2483.5 MHz | – | 2495 MHz | 2483.5 MHz | – | 2495 MHz | TDD |

As shown in Figure 5.1.1-1, the US 2.4 GHz Terrestrial Band exists between the ISM band and BRS / EBS bands.  The ISM band terminates at 2483.5 MHz, with commonly used unlicensed services such as 802.11 Wi-Fi and Bluetooth terminating at 2473 MHz and 2480 MHz respectively.  BRS / EBS bands begin with the BRS-1 channel at 2496 MHz.  A 1 MHz guard band exists between the termination of the US 2.4 GHz Terrestrial Band at 2495 MHz and the beginning of the first BRS channel at 2496 MHz.

- Figure 5.1.1-1:  US 2.4 GHz Terrestrial Band and Adjacent Band Services

## 5.1.2	Channel bandwidths

The FCC authorized band spans 2483.5 MHz to 2495 MHz and the regulator has not divided the band into discrete channels or blocks.  As shown in Table 5.1.2-1, the 11.5 MHz allocation supports operating bandwidths up to 10 MHz.

### Table 5.1.2-1: Proposed operating bandwidths

| 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
| Yes | Yes | Yes | Yes | No | No |

## 5.2	Adjacent 3GPP Bands

## 5.2.1 		Introduction

Nearby 3GPP bands are summarized in Figure 5.2.1-1.  Band 7 and Band 38 are not used in the US regulatory domain, where the 2.4 GHz Terrestrial Band is currently authorized by the FCC.  However, Band 41 is commonly used in all areas of the US.

- Figure 5.2.1-1:  US 2.4 GHz Terrestrial Band and Adjacent 3GPP Bands

## 5.2.2 		Co-Existence Between 2.4 GHz and 3GPP Band 41

3GPP Band 41 is a TDD band, which spans 2496 MHz to 2690 MHz.  In the US, Band 41 operation is licensed through the Broadband Radio Service (BRS) and the Educational Broadband Service (EBS).  Figure 5.2.2-1 illustrates the 35 channel blocks that comprise the BRS / EBS services and their relationship with the 2.4 GHz Terrestrial Band.  The 5.5 MHz and 6 MHz channel blocks associated with BRS / EBS licenses are a vestige of educational television use of this spectrum.  While educational services still operate in the EBS channel blocks, Band 41 commercial LTE operations predominate across most of the US.  The 2.4 GHz Terrestrial Band spanning 2483.5 MHz to 2495 MHz is separated by a 1 MHz guard band from BRS-1, which is the lowest frequency channel block available for Band 41 operations.

Co-existence between the proposed 2.4 GHz Terrestrial Band and Band 41 was a central consideration during the FCC rule making process.  These co-existence requirements were addressed with power limits, emissions limits, and the mandated integration of a Network Operating System (NOS) to administer use of the 2.4 GHz Terrestrial Band (see Section 5.4).  While not required by the regulator, Band 41 and 2.4 GHz Terrestrial Band operators may also rely upon TDD synchronization to resolve interference issues that may be experienced as a result of co-located base stations.

## Figure 5.2.2-1:  US 2.4 GHz Terrestrial Band and Adjacent BRS / EBS Spectrum Comprising Band 41

## 5.2.3 		Co-Existence Between 2.4 GHz and 3GPP Bands 7 and 38

## The proposed 2.4 GHz TDD band spanning 2483.5 MHz to 2495 MHz is adjacent to 3GPP Bands 7 and 38.  3GPP Band 7 is an FDD band, with uplink spanning 2500 MHz to 2570 MHz and downlink spanning 2620 MHz to 2690 MHz.  3GPP Band 38 is a TDD band spanning 2570 MHz to 2620 MHz.

## Currently, the proposed 2.4 GHz TDD band has terrestrial license authority only in the US.  Due to the presence of Band 41, neither Band 7 nor Band 38 is utilized in the US regulatory domain.

## 5.3		Nearby non-3GPP Unlicensed Services

## 5.3.1 		Co-Existence Between 2.4 GHz and Part 15 Services

## Title 47 of the CFR, Part 15 permits unlicensed operations from 2400 MHz to 2483.5 MHz [4].  The 2.4 GHz unlicensed band (commonly known as the ISM band) is especially notable for the heavy presence of 802.11 Wi-Fi and Bluetooth operations.  In the US, 802.11 channel 11 is the highest channel effectively permitted by Part 15 rules, terminating at approximately 2473 MHz.  Across all regulatory domains, Bluetooth channel 39 is the highest, terminating at approximately 2480 MHz.  Figure 5.3.1-1 illustrates the relationship between the 2.4 GHz Terrestrial Band and adjacent non-3GPP unlicensed services in the ISM band.

- Figure 5.3.1-1:  US 2.4 GHz Terrestrial Band and Adjacent Unlicensed Band

## 5.4		FCC Regulatory Requirements

## 5.4.1 		Introduction

Licensed wireless communications services in the 2.4 GHz band are governed by Part 25 of Title 47 of the Code of Federal Regulations [5].

## 5.4.2		Frequency Limits

Frequency limits for the 2.4 GHz band are governed by Title 47 of the CFR, Part 25.149.  Terrestrial operation is limited to the licensed band from 2483.5 MHz to 2495 MHz.  Bonding or aggregation with unlicensed services below 2483.5 MHz is prohibited by Title 47 of the CFR, Parts 15.205, 15.209, 15.247 and 15.249.

## 5.4.3		Power Limits

Power limits for the 2.4 GHz band are governed by Title 47 of the CFR, Part 25.149(c)(4)(i)-(iv).  These power limits are identical to those found in Part 15 for unlicensed services authorized from 2400 MHz to 2483.5 MHz and they do not distinguish between base station and user equipment.  The relevant rules for operational equipment are listed below:

(4) Applications for equipment authorization of terrestrial low-power system equipment that will operate in the 2483.5-2495 MHz band shall demonstrate the following:

(i)	The transmitted signal is digitally modulated;

(ii)	The 6 dB bandwidth is at least 500 kHz;

(iii) 	The maximum transmit power is no more than 1 W with a peak EIRP of no more than 6 dBW;

(iv) 	The maximum power spectral density conducted to the antenna is not greater than 8 dBm in any 3 kHz band during any time interval of continuous transmission;

## 5.4.4		Emissions Limits

Emissions limits for the 2.4 GHz Terrestrial Band are governed by Title 47 of the CFR, Part 25.149(c)(4)(v)-(vii).  Figure 6.4.4-1 is a graphical summary of emissions limits for the 2.4 GHz Terrestrial Band, assuming a 10 MHz emission bandwidth.  The relevant rules for operational equipment are listed below:

(v) Emissions below 2483.5 MHz are attenuated below the transmitter power (P) measured in watts by a factor of at least 40 + 10 log (P) dB at the channel edge at 2483.5 MHz, 43 + 10 log (P) dB at 5 MHz from the channel edge, and 55 + 10 log (P) dB at X MHz from the channel edge where X is the greater of 6 MHz or the actual emission bandwidth.

(vi) Emissions above 2495 MHz are attenuated below the transmitter power (P) measured in watts by a factor of at least 43 + 10 log (P) dB on all frequencies between the channel edge at 2495 MHz and X MHz from this channel edge and 55 + 10 log (P) dB on all frequencies more than X MHz from this channel edge, where X is the greater of 6 MHz or the actual emission bandwidth;

(vii) Compliance with these rules is based on the use of measurement instrumentation employing a resolution bandwidth of 1 MHz or greater.  However, in the 1 MHz bands immediately above and adjacent to the 2495 MHz a resolution bandwidth of at least 1 percent of the emission bandwidth of the fundamental emission of the transmitter may be employed.  If 1 percent of the emission bandwidth of the fundamental emission is less than 1 MHz, the power measured must be integrated over the required measurement bandwidth of 1 MHz.  A resolution bandwidth narrower than 1 MHz is permitted to improve measurement accuracy, provided the measured power is integrated over the full required measurement bandwidth (i.e., 1 MHz).  The emission bandwidth of the fundamental emission of a transmitter is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.  When an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in this section;

## Figure 5.4.4-1:  US 2.4 GHz Terrestrial Band Emissions Limits (10 MHz Emission BW)

## 5.4.5 		Network Operating System

Special requirements for control of the 2.4 GHz band are governed by Title 47 of the CFR, Part 25.149(g)(2).  The relevant rules are listed below:

(2) An ATC licensee seeking to modify its license to add authority to operate a terrestrial low-power network shall certify in its modification application that its operations will utilize a Network Operating System (NOS), consisting of a network management system located at an operations center or centers.  The NOS shall have the technical capability to address and resolve interference issues related to the licensee’s network operations by (a) reducing operational power; (b) adjusting operational frequencies; (c) shutting off operations; or (d) any other appropriate means.  The NOS shall also have the ability to resolve interference from the terrestrial low-power network to the licensee’s MSS operations and to authorize access points to the network, which in turn may authorize access to the network by end-user devices.  The NOS operations center shall have a point of contact in the United States available 24 hours a day, seven days a week, with a phone number and address made publicly-available by the licensee.

<<< End of TP>>>