3GPP TSG-RAN WG4 Meeting #88 	R4-1811137

# Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.145-2: Adding spherical grids (Annex F) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Spherical grids for calculating TRP are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Spherical equal angle, equal area and Fibonacci grids are added to the conformance specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no means to test TRP-based requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

Annex F (normative):
TRP measurement grids

# F.1		General

# F.2	Spherical equal angle grid

TRPEstimate is defined as

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn

where N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The reference sampling intervals are specified in FFS.

# F.3	Spherical equal area grid

TRPEstimate is defined as

### TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified in FFS. Each (θn,ϕn) is a sampling point.

# F.4 	Spherical Fibonacci grid

TRPEstimate is defined as

TRPEstimate=1Ii=1IEIRP(θi,ϕi)

I is the total number of samples. Each (θi,ϕi) is a sampling point.

# F.5 Orthogonal 2 cut grid

TRPEstimate  shall be calculated as FFS

# F.6 Wave vector space grid

TRPEstimate  shall be calculated as FFS

<End of modified section>