3GPP TSG-RAN WG4 Meeting#88                	R4-1811300

Gothenburg, Sweden, August 20th – 24th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	UE SSB monitoring requirement for intra-band SCC

Agenda item:	9.11.2

Document for:	Discussion

# 1	Introduction

In the last number of RAN4 meetings, RAN4 has been discussing UE measurement capability related to number of cells and beams the UE shall be able to monitor – especially for FR2. Most numbers are agreed (although in []) while agreement is still open related to number of beams to measure on intra-band SCCs.

In this paper we provide our system level simulation results provided earlier to facilitate the discussion related to the number of SSB based beams to measure on additional intra-band SCCs.

2	Discussion

Latest agreements in RAN4 related to UE measurement capability ended with the following outcome at the Busan meeting in May (no further progress in Montreal AH):

- FR2 Main CC: This refers to either an intra-band PCell, intra-band PSCell or one intra-band SCC of no intra-band PCell or intra-band PSCell is configured in the band.

- Other CC than Main CC: This refers to any other intra-band CC than the Main CC in the band.

Based on the agreements made in Busan meeting the UE shall be able to monitor:

|  | FR1 |  | FR2 Main CC |  | FR2 other CCs than Main CC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cells | SSB Beams | Cells | SSB Beams | Cells | SSB Beams |
| Intra-frequency | 8 | [14] | [6] | [24] | 1 | [1 ~ 4] |
| Inter-frequency | 4 | [7] | 4 | [10] | N/A | N/A |

Additionally, following was agreed concerning cell detection, SSB detection and measurement periods for FR2 for no DRX:

- TPSS/SSS_sync:		max[ 600ms, ceil( [5] x Kp x KRLM) x N1 x SMTC period ]

- T SSB_measurement_period:	max[ 400ms, ceil( 5 x Kp x KRLM) x N3 x SMTC period ]

In earlier meetings we have provided SLS results targeted at helping the decision related to number of cells per carrier the UE would need to be able to monitor [1, 5 and 7549]. Based on these results we have earlier concluded in [2, 5] following for FR2:

Proposal 5: Monitoring of 2 beam/SSB per cell in FR2 is not sufficient.

Proposal 6: The number of beams the UE need to track per cell in FR2 is higher than 2.

Next, we discuss further on the number of SSB based beams the UE should monitor on other intra-band CCs than the main CC. We use earlier provided system level and in addition new system level and link level results.

## 2.1	Simulation setup

The simulator is a fully dynamic system simulator in which all the UEs are moving dynamically in a grid of cells with 30 km/h. There are 28 UEs active at a time with keep alive traffic (one UE per sector). SSB burst periodicities of 20ms or 40ms have been used which are also used as the UE sampling rate (L1 sampling interval). I.e. the UE measurement rate is 20ms or 40ms. This emulates SMTC periods of 20ms and 40ms without any measurement restrictions due to performing any other measurements. In the simulations we use an SSB transmission offset to reduce the interference seen by the UE when performing SSB based measurements. More details related to the simulation assumptions can be found in Appendix A.

In the simulations L1 measurement filter of 5 samples is used. For inter-cell mobility (between cells), the modeling of measurement report, handover command and handover complete transmissions are error-free and there is no radio link monitoring active. Measurement reporting to handover command delay (handover preparation delay) is included in the simulation and modeled as a static 50ms delay. Additionally, the handover command to handover complete delay is modeled using a latency of 20ms.

For intra-cell inter-beam management, i.e. beam change, the beam detection delay is 400ms and no Index reading latency is assumed. Cell change, or normal inter-cell mobility, is done using handover procedure, including a cell detection delay (600ms), measurement delay (400ms), measurement reporting to handover command delay (50ms), and handover command to handover complete delay (20ms).

In the simulations an intra-cell beam change is performed without delay (except from the beam measurement delay). An inter-cell beam change is done by handover procedure accounting all the handover delays described.

The simulations are based on the baseline simulation assumptions as agreed in [3] changing the necessary parameters to apply cell and SSB settings according to [4]. The network layout is illustrated in figure 1.

Figure 1 Illustration of the network layout used in the simulation.

We have used an Urban Macro layout with 7x4-sector sites, 28 sectors total and 200ms ISD. In the simulation setup we have used a realistic deployment setup accounting the necessary DL beam gain to ensure the necessary cell coverage. Additionally, the simulation setup is accounting the limitation that there can be no more than 64 SSB based beams per cell.

Accounting these aspects, the sector aperture in the simulations is 90 degrees. The results are based on using 48 SSBs per cell in 3 elevations. Note: the hexagonal grid is only for illustrative purposes and have no real meaning.

## 2.2	Simulation Results

In former meetings we have analyzed the impact of cell detection delay and measurement interval and how this affects the UEs ability to track the strongest SSB based beam. RAN4 has decided that the measurement period for FR2 is 400ms. Additionally, there is a need to apply certain relaxations to the UE measurement requirements in FR2 due to UE Rx beam forming.

2.2.1	Simulation Metric

To analyze the number of beams the UE should monitor per additional intra-band CC we have looked at the UE ability to track beams to ensure that UE is not camped on a beam which is much worse that another beam, we look at following metric:

- SS-RSRP differences between the serving SS beam and Nth best tracked beam.

2.2.2	SS-RSRP differences between the serving SS beam and Nth best tracked beam

We look at the SS-RSRP difference between the serving SS beam and the Nth best tracked beam in two scenarios:

- UE monitor 1 SSB in addition to serving, and

- UE monitor 4 SSBs in addition to serving

The results illustrated is the SS-RSRP(serving) – SS-RSRP(Nth beam). I.e. if the difference becomes negative it would mean that the Nth tracked beam is better than serving.

Next results illustrate the effect from tracking a different number of beams on a carrier. Of interest is to analyse if there are big differences in the SS-RSRP of the serving and Nth tracked beam as this should not happen. In these results we keep the measurement interval fixed at 40ms.

From the results, we see that when only one beam per cell is tracked, there is typically the largest difference between serving beam and other beams. This means that some good beams are not tracked and not included in the list of tracked beams.

In practise, when considering such deployment scenario as used in the simulations, there shouldn’t be a large difference between the serving beam and other beams. A large difference would mean that a non-serving beam is much better than the serving beam. If a large difference is observed it means that some beams which should have been tracked are not tracked due to UE tracking too few beams. I.e. UE is not able to feedback sufficient and timely beam measurements information to the network which restricts the network beam management.

When not monitoring sufficient SSBs, the latencies from potential beam measurements become visible. If the measurement latencies become too long this will impact the UE’s ability to follow/track the dynamics in field. I.e. the UE cannot provide measurement assistance information to network such that network can perform timely actions, which affects the beam management as the UE moves. This means that the UE mobility and BM is downgraded which could increase the risk of losing connection on the CC and reduce the overall UE and system TP.

Tracking too few beams impact the UE ability to follow the dynamics in beam changes when the UE moves.

Tracking a too low number of beams increases the risk of UE not being using the beam with best SS-RSRP.

- From the results we observe that 1 beam (per cell) is clearly not enough (as also observed in our earlier contributions). We also observe that when tracking what is equivalent to 4 beams per cell, the difference between the serving beam and the 3rd best beam is low. In general, we see that when tracking more beams, the difference between serving beam and best Nth beam diminishes which indicates that the UE is able to follow and track changes as it moves around.

In this scenario, tracking what is equal to 3 beams per cell is sufficient. However, this is only one scenario and without any margin. Based on this we think UE would need to more than what is equivalent to 3 beams per cell on a carrier.

UE would need to track more than what is equivalent to 3 beams per cell per carrier.

Based on this we propose:

In FR2 the UE shall be able to monitor at least 4 SSB based beams on other intra-band SCC than Main CC.

The main CC is here understood to be the PCC, PSCC or an SCC if the PCC or PSCC is not located in the same band as the SCC.

3	Conclusion

In this paper we provided our system level simulation results provided earlier to facilitate the discussion related to the number of SSB based beams to measure on additional intra-band SCCs. We have already earlier in our studies concluded following:

Proposal 5: Monitoring of 2 beam/SSB per cell in FR2 is not sufficient.

Proposal 6: The number of beams the UE need to track per cell in FR2 is higher than 2.

Based on the  the results in this paper we observe:

- Tracking too few beams impact the UE ability to follow the dynamics in beam changes when the UE moves.

Tracking a too low number of beams increases the risk of UE not being using the beam with best SS-RSRP.

UE would need to track more than what is equivalent to 3 beams per cell per carrier.

Based on which we propose:

- In FR2 the UE shall be able to monitor at least 4 SSB based beams on other intra-band SCC than Main CC.

References

- R4-1802798, Dynamic simulation results for NR Mobility in FR2, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1802799, Summary discussion on number of cells and beams in FR2, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1709903, System level simulation assumptions in NR RRM, Ericsson

- R4-1713151, Network Cell and Beam Dimensioning discussion, Nokia, Nokia Shanghai Bell

- R4-1805265, Additional SLS on UE measurement capability, Nokia, Nokia Shanghai Bell

# A	Simulation assumptions