3GPP TSG-RAN WG4 Meeting #88                                      R4-1811346

Gothenburg, Sweden, 20th – 24th August, 2018

Agenda item:			7.4.2

Title:	Way forward on NR-NR DC RRM

Source:			Samsung, Nokia, Nokia Shanghai Bell

Document for:		Approval

# 1	Background

In RAN#80, NR-NR DC is approved to be introduced in Rel-15 late drop, and the scope for NR-NR DC in exception sheet as below:

For SA (Option 2) only: · NR-NR Dual connectivity aspects ‐ Synchronous mode from physical layer aspects; ‐ Band combination(s) for FR1 + FR2; ‐ MCG fully in FR1 and SCG fully in FR2 ‐ Common radio protocols and network interfaces applicable to both synchronous and asynchronous mode of operations.

During RAN4 #88 RRM session, the followings proposals are agreed for NR-NR DC:

SFTD with NR PCell and NR PSCell: Proposal 4: Since only synchronous mode of NR-NR DC is introduced in Rel-15, no requirement on SFTD between NR PCell and NR PSCell is to be introduced in Rel-15 time frame. Kca scaling factor for NR-NR DC:  For the case with FR1-FR2 NR-NR DC case, the same relaxing factor requirement can be defined as FR1-FR2 CA case.

In the approved LS to RAN2 (R4-1811714) in RAN4 #88 RRM session, the following scenario for SFTD is agreed to be introduced, while the later one is related to NR-NR DC deployment:

RAN4 has concluded that Rel-15 requirements are needed for the following two use cases: - SFTD between NR PCell and E-UTRA PSCell - SFTD between NR PCell and NR neighbour cell, before NR PSCell is configured

# Way Forward

#### 2.1 Scope of Impacted RRM Requirement

Following Table-1 lists the expected impact on RRM requirement due to the introduced synchronous FR1-FR2 NR-NR DC in late drop:

Table-1. List of RRM requirement impacted by introducing sync. FR1-FR2 NR-NR DC

| Section/Type of Requirement | Impacted or Not? | Comment |
| --- | --- | --- |
| 3.1 Definition | Yes | Definition for “NR-NR DC” needs to be captured in Section 3.1. |
| 3.6 Applicability of requirements in this specification version | Yes | NR-NR dual connectivity is explained in Section 3.6. Number of serving carriers for NR-NR DC is captured in new subsection under Section 3.6 and the detailed number depends on RAN4 RF discussion for # of CCs in NR-NR DC requested. |
| 4 SA: RRC_IDLE state mobility | No | NR-NR DC is not applicable in RRC_IDLE state. |
| 5 SA: RRC_INACTIVE state mobility | No | NR-NR DC is not applicable in RRC_INACTIVE state. |
| 6.1 Handover | No | Handover requirement defined for NR SA can be reused for NR-NR DC, and no explicit revision is needed. |
| 6.2.1 SA: RRC Re-establishment | No | RRC re-establishment requirement defined for NR SA can be reused for PCell in NR-NR DC, and no explicit revision is needed |
| 6.2.2 Random access | No | Random access requirement can be reused for PCell/PSCell in NR-NR DC, and no explicit revision is needed |
| 6.2.3 SA: RRC Connection Release with Redirection | No | RRC connection release requirement defined for NR SA can be reused for PCell in NR-NR DC, and no explicit revision is needed |
| 7.1 UE transmit timing | Yes | Reference cell in case of NR-NR DC should be PCell, which is to be mentioned in Section 7.1. Existing requirement can be reused. |
| 7.2 UE timer accuracy | No | This is defined in generic way for any cell. |
| 7.3 Timing advance | No | Current requirement is defined in a generic way, and no explicit revision is needed. |
| 7.5 Maximum Transmission Timing Difference | Yes | add new subsection for NR-NR DC MRTD |
| 7.6 Maximum Receive Timing Difference | Yes | add new subsection for NR-NR DC MTTD |
| 8.1 Radio Link Monitoring | Yes | Current requirement is defined in a generic way for PCell or PSCell which can be reused. However, scheduling availability for NR-NR DC needs to be added (either revise 8.1.7.4 or add new subsection) |
| 8.2 Interruption | Yes | NR-NR DC interruption requirement needs to be added for the selected scenarios. |
| 8.3 SCell Activation and Deactivation Delay | Yes | The applicability description in TS38.133 Section 8.3.1 can be modified to take NR-NR DC into account, while the same requirement defined in Section 8.3.2 and 8.3.3 can be reused for NR-NR DC. |
| 8.4 UE UL carrier RRC reconfiguration Delay | No | No impact due to NR-NR DC. |
| 8.5 Link Recovery Procedures | Yes | Current requirement is defined in a generic way for PCell or PSCell which can be reused. However, scheduling availability for NR-NR DC needs to be added (either revise 8.5.7.4 or add new subsection) |
| 8.6 Active BWP switch delay | Yes | The applicability description for BWP switching delay in 8.6.1 needs to be extended for PSCell or any activated SCell in SCG for FR1-FR2 NR-NR DC. |
| 8.x NR PSCell addition and release delay | Yes (New section needed) | New section needed, while the content of requirement can be reused from TS36.133’s NR PSCell addition and release delay requirement. |
| 9.1 General measurement requirement | Yes | Measurement gap’s applicability rule for NR-NR DC is to be added in Section 9.1.2. UE measurement mode’s impact on UE behaviour and applicable performance requirement should be specified for NR-NR DC scenario for per-FR capable UE in Section 9.1.2. The scenario of NR-NR DC is expected to be added in the part for interrupted slot number (figure and table) in Section 9.1.2. Gap sharing optimized for NR-NR DC depends on corresponding gap sharing discussion. UE measurement capability in Section 9.1.3: Reuse SA requirement in 9.1.3.2a. |
| 9.2 NR intra-frequency measurements | Yes | In Section 9.2, scaling factor (similar to K_CA discussion) is needed to added for NR-NR DC case, which should be aligned with FR1-FR2 CA case. |
| 9.3 NR inter-frequency measurements | Yes | NR-NR DC scenario should be added in for 0.25ms switching time’s applicability, like EN-DC and SA. Other than this, current requirement is defined in a generic way, and no explicit revision is needed. |
| 9.4 Inter-RAT measurements | No | Current requirement is defined in a generic way, and no explicit revision is needed. |
| 9.5 CSI-RS based measurements | No | Current requirement is defined in a generic way, and no explicit revision is needed. |

#### 2.2 MTTD/MRTD

The following MRTD and MTTD values for FR1+FR2 NR-NR DC with synchronization operation are considered:

- Option 1:

Table-2. MRTD/MTTD for FR1+FR2 NR-NR DC with synchronization operation

|  | Value | Comment |
| --- | --- | --- |
| MRTD for FR1-FR2 NR-NR DC | 33 us | reuse MRTD from EN-DC |
| MTTD for FR1-FR2 NR-NR DC | 34.1us | including 30us for DL Propagation delay, 3us for Base station Timing Alignment Error, and 1.1us for total worst case uplink error between FR1 and FR2 carriers |

- Other options are not precluded.

#### 2.3 Scenarios for Interruption Requirement

The following scenarios are identified in which interruption requirement should be defined:

Table-3. Scenarios for NR-NR DC interruption requirement

| # | Scenario | Interruption Allowed and Victim Cell if Any |
| --- | --- | --- |
| Scn-1 | PSCell is added or released | Scenario Needed (Similar to TS36.133 7.32.2.1). Victim cell: PCell and activated SCell in MCG. |
| Scn-2 | PCell transitions between active and non-active during DRX | Scenario Needed (Similar to TS38.133 8.2.1.2.1). Victim cell: PSCell and activated SCell in MCG, and activated SCell in SCG. |
| Scn-2bis | PSCell transitions between active and non-active during DRX | Scenario Needed (Similar to TS38.133 8.2.1.2.1). Victim cell: PCell and activated SCell in MCG, and activated SCell in SCG. |
| Scn-3 | PCell transitions from non-DRX to DRX | Scenario Needed (Similar to TS38.133 8.2.1.2.2). Victim cell: PSCell and activated SCell in MCG, and activated SCell in SCG. |
| Scn-3bis | PSCell transitions from non-DRX to DRX | Scenario Needed (Similar to TS38.133 8.2.1.2.2). Victim cell: PCell and activated SCell in MCG, and activated SCell in SCG. |
| Scn-4 | SCell in either MCG or SCG is added or released | Scenario Needed (Similar to TS38.133 8.2.1.2.3). Victim cell: PCell/PSCell and activated SCell in MCG, and activated SCell in SCG. |
| Scn-5 | SCell in either MCG or SCG is activated or deactivated | Scenario Needed (Similar to TS38.133 8.2.1.2.4). Victim cell: PCell/PSCell and activated SCell in MCG, and activated SCell in SCG. |
| Scn-6 | measurements on SCC with deactivated SCell in either MCG or SCG | Scenario Needed (Similar to TS38.133 8.2.1.2.5). Victim cell: PCell/PSCell and activated SCell in MCG, and activated SCell in SCG. |

#### 2.4 Measurement related requirement

The following proposals list the expected impact for measurement related requirement for NR-NR DC:

Proposal-1: RAN4 adopt similar concepts of per-UE and per-NR measurement gap for NR-NR Dual Connectivity.

Proposal-2: RAN4 adopt the applicability rule for gap pattern configurations supported by the UE with NR-NR Dual Connectivity operation.

| Measurement gap pattern configuration | Serving cell | Measurement Purpose | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE measurement gap | FR1, or FR1 + FR2 | E-UTRA only Note 3 | 0,1,2,3 |
|  |  | FR1 and/or FR2 | 0-11 |
|  |  | E-UTRAN and FR1 and/or FR2 Note 3 | 0,1,2,3 |
| Per FR measurement gap | FR1 | E-UTRA only Note 3 | 0,1,2,3 |
|  | FR2 |  | No gap |
|  | FR1 | FR1 only | 0-11 |
|  | FR2 |  | No gap |
|  | FR1 | FR2 only | No gap |
|  | FR2 |  | 12-23 |
|  | FR1 | E-UTRA and FR1 Note 3 | 0,1,2,3 |
|  | FR2 |  | No gap |
|  | FR1 | FR1 and FR2 | 0-11 |
|  | FR2 |  | 12-23 |
|  | FR1 | E-UTRA and FR2 Note 3 | 0,1,2,3 |
|  | FR2 |  | 12-23 |
|  | FR1 | E-UTRA and FR1 and FR2 Note 3 | 0,1,2,3 |
|  | FR2 |  | 12-23 |
| NOTE 1: When E-UTRA inter-RAT RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. NOTE 2: Measurement purpose which includes E-UTRA measurements includes also inter-RAT E-UTRA RSRP and RSRQ measurements for E-CID NOTE 3: Editor’s note: a note to be added in Table 9.1.2-3 on that measurement gap patterns #2 and #3 are supported only by the UEs which have a corresponding capability of short measurement gap once RAN2 specifies the capability. |  |  |  |

Proposal 3: For UE with NR-NR DC configured, RAN4 adopt the same requirement for number of layers for multiple monitoring as NR Standalone mode, i.e.,

|  |  | NR inter-frequency carrier | E-UTRA TDD inter-RAT carrier | E-UTRA FDD inter-RAT carrier | Total effective carriers |
| --- | --- | --- | --- | --- | --- |
| # of effective carrier freq. | NR RRC_IDLE state | 7 | 7 | 7 | 14 (including one serving layer) |
|  | NR RRC_CONNECTED state | 7 | 7 | 7 | 13 |
|  | NR RRC_CONNECTED state with NR-NR DC configured | 7 | 7 | 7 | 13 |

For SFTD related requirement for NR-NR DC in Rel-15, RAN4 has concluded that Rel-15 requirements are needed for the following case:

- SFTD between NR PCell and NR neighbour cell, before NR PSCell is configured

# 3	Reference

| Tdoc No. | Title | Source | Type | For |
| --- | --- | --- | --- | --- |
| R4-1809943 | Overall Scope for NR-NR Dual Connectivity RRM Requirement | Samsung | discussion | Approval |
| R4-1809944 | Discussion on UE Measurement for NR-NR Dual Connectivity | Samsung | discussion | Discussion |
| R4-1809945 | Discussion on Timing for NR-NR Dual Connectivity | Samsung | discussion | Discussion |
| R4-1809946 | Discussion on Interruption and Delay for NR-NR Dual Connectivity | Samsung | discussion | Discussion |
| R4-1810603 | On SFTD for NR-NR DC | Ericsson | other | Discussion |
| R4-1810604 | DraftCR 38.133 Introduction of inter-frequency SFTD core requirements | Ericsson | draftCR | Endorsement |
| R4-1810784 | Overview of RRM requirements for NR-NR DC | Ericsson | other | Discussion |
| R4-1810785 | WF on RRM requirements for NR-NR DC | Ericsson | other | Approval |
| R4-1810924 | MRTD and MTTD requirements for synchronous NR-NR DC | Ericsson | other | Approval |
| R4-1810925 | Draft CR for TS 38.133: MRTD and MTTD for inter-band synchornous NR-NR DC | Ericsson | draftCR | Endorsement |
| R4-1811128 | NR-NR dual connectivity requirements | Nokia, Nokia Shanghai Bell | discussion | Discussion |