3GPP RAN WG4 Meeting #88		R4-1811521
Gothenburg, Sweden, August 20 – 24, 2018

Agenda item:	7.6.14.2.1

Source:	Apple Inc.

Title:	FR2 multi-band support adhoc minutes

Document for:	Approval

# Available data in RAN4

Based on the submitted data in [1] through [5] and also based on offline discussion and submission of new data in [6], Table 1 below collects the data set on peak and spherical coverage relaxations.

Table 1: Available multi-band relaxation data

| Case | Supported bands | Band | Peak relaxation |  |  |  |  |  | Spherical relaxation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [1] | [2] | [3] | [4] | [5] | [6] | [1] | [2] | [3] | [4] | [5] | [6] |
| 8 | n257, n258 | n257 |  | 1.0 | FFS |  | 2.0 | 1.5 |  | 0.0 | FFS |  | 2.0 | 1.5 |
|  | n257, n258 | n258 |  | 1.0 | FFS |  | 2.0 | 1.5 |  | 0.0 | FFS |  | 2.0 | 1.5 |
| 7 | n258, n260 | n258 |  | 1.0 | FFS |  |  |  |  | 0.0 | FFS |  |  |  |
|  | n258, n260 | n260 |  | 1.0 | FFS |  |  |  |  | 0.0 | FFS |  |  |  |
| 6 | n258, n261 | n258 |  | 1.0 | FFS |  | 2.0 |  |  | 0.0 | FFS |  | 2.0 |  |
|  | n258, n261 | n261 |  | 0.5 | FFS |  | 2.0 |  |  | 0.0 | FFS |  | 2.0 |  |
| 5 | n260, n261 | n260 | 0.0 | 0.0 | FFS |  |  |  | 1.6 | 0.0 | FFS |  |  |  |
|  | n260, n261 | n261 | 0.0 | 0.0 | FFS |  |  |  | 0.0 | 0.0 | FFS |  |  |  |
| 4 | n257, n258, n261 | n257 |  | 1.0 | FFS |  | 2.0 | 1.5 |  | 0.0 | FFS |  | 2.0 | 1.5 |
|  | n257, n258, n261 | n258 |  | 1.0 | FFS |  | 2.0 | 1.5 |  | 0.0 | FFS |  | 2.0 | 1.5 |
|  | n257, n258, n261 | n261 |  | 0.5 | FFS |  | 2.0 | 1.5 |  | 0.0 | FFS |  | 2.0 | 1.5 |
| 3 | n257, n260, n261 | n257 |  | 0.0 | FFS |  |  |  |  | 0.0 | FFS |  |  |  |
|  | n257, n260, n261 | n260 |  | 0.0 | FFS |  |  |  |  | 0.0 | FFS |  |  |  |
|  | n257, n260, n261 | n261 |  | 0 | FFS |  |  |  |  | 0.0 | FFS |  |  |  |
| 2 | n258, n260, n261 | n258 |  | 1.0 | FFS |  |  |  |  | 0.0 | FFS |  |  |  |
|  | n258, n260, n261 | n260 |  | 1.0 | FFS |  |  |  |  | 0.0 | FFS |  |  |  |
|  | n258, n260, n261 | n261 |  | 0.0 | FFS |  |  |  |  | 0.0 | FFS |  |  |  |
| 1 | n257, n258, n260, n261 | n257 | 2.1 |  | FFS | 1.8 |  | 2.0 | 1.9 |  | FFS | 2.4 |  | 2.0 |
|  | n257, n258, n260, n261 | n258 | 2.1 |  | FFS | 1.8 |  | 2.0 | 2.0 |  | FFS | 2.4 |  | 2.0 |
|  | n257, n258, n260, n261 | n260 | 1.5 |  | FFS | 0.0 |  | 2.0 | 1.8 |  | FFS | 1.3 |  | 2.0 |
|  | n257, n258, n260, n261 | n261 | 2.0 |  | FFS | 1.8 |  | 2.0 | 1.2 |  | FFS | 2.4 |  | 2.0 |

NOTE: it is needed to confirm the values from [2] for the highlighted cases

Discussion:

NTT DOCOMO: this is just data, not proposal?

Chair: yes

Samsung: our plan is to provide data for the next meeting

# Prioritization of cases

In an effort to focus the discussion, some prioritization of the cases can be useful.  The following is a suggested option to initiate the discussion.

Option 1: Case 1, Case 4, Case 5, Case 6, Case 8

Option 2: Case 5

Option 3: Case 5, Case 3

Option 4: Case 5, Case 1

Option 5: Case 5, Case 3, Case 1 (in priority order)

Option 6: Case 5, Case 4, Case 6, Case 8 (in priority order)

Discussion:

Verizon: for Cases 1 – 4, who is going to have these configurations?

Apple: could target different regions based on vendor’s choice

Sony: our data is the result of our analysis and is not our proposal; we do have some margin, and the relaxation can be different

Dish: would these be the cases for the Rel-15 specification; will other cases be excluded?

Verizon: what does Option 1 imply?

Apple: in our understanding, we target defining the MB relaxation values for these cases this meeting; these are the cases where we have at least two data points for analysis

Verizon: we propose Option 2

NTT DOCOMO: for release 15 we can consider Option 2

Samsung: we propose Option 3

Chair: we note that Case 3 (added in Option 3) only has 1 data point in the analysis

AT&T: we support Option 2

LGE: if we proceed with Option 2, then in the future release can we discuss MB relaxation for other combinations?

Apple: if we proceed with Option 2, why are we precluding other cases in Rel-15?

Huawei: if we proceed with Option 2, does it mean in Rel-15 the UE only has two choices: Option 2 or single band?

Verizon: the reason we choose Option 2 is because there is a lot of analysis here, but for Rel-15 we would like to make sure we complete this case due to timing

Chair: if we have an option with multiple cases and select Case 5 as the priority within the option, would this be acceptable to Verizon?

Verizon: our choice is based on market requirements

AT&T: we support Verizon; Case 5 is clearly the earliest rollout; to LGE: of course, future releases will look at other cases, as well as the possibility of adding new bands

Qualcomm: when new cases are added, do we have to wait for the next release, or can we add requirements in release-independent manner?

Agreement: power class requirements are release independent, and multi-band relaxations are part of power class requirements; therefore, they are release-independent

Apple: we would like to include Case 1 together with Case 5

Qualcomm: we should take Case 5, but we should not limit to just that one combination

Dish: we are OK with AT&T + Verizon proposal; Option 4 has merit; but if we take the analogy from LTE CA we started with easiest combo, and it was painful to work on the complex combo; it would be beneficial to take the difficult combo early

Intel: let’s vote

AT&T and Verizon: we would like to focus on the market need; Case 5 is clearly the priority; release independence allows us to have this discussion later when there is more market demand for other cases

Apple: in order to progress, can we agree to take Case 5 as the highest priority and then consider other cases?

Qualcomm: we agree with Apple

Agreement: proceed with Case 5 as the highest priority and then consider other cases

# Framework proposal

Based on the available data and the prioritization of cases, Table 2 below summarizes the data;  proposals on multi-band relaxation values for peak and spherical are left open to initiate the discussion.  Effort to develop the proposals is prioritized to target the selected cases from Step #2 above.  Columns to handle potential multiple alternate proposals are provided to help the discussion.

Table 2: Summary of multi-band relaxation data and proposals

| Case | Supported bands | Band | Peak Relaxation |  |  |  |  | Spherical relaxation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Alt.1 | Alt.2 | Alt.3 | Min | Max | Alt.1 | Alt.2 | Alt.3 |
| 8 | n257, n258 | n257 | 1.0 | 2.0 |  |  |  | 0.0 | 2.0 |  |  |  |
|  | n257, n258 | n258 | 1.0 | 2.0 |  |  |  | 0.0 | 2.0 |  |  |  |
| 7 | n258, n260 | n258 | 1.0 |  |  |  |  | 0.0 |  |  |  |  |
|  | n258, n260 | n260 | 1.0 |  |  |  |  | 0.0 |  |  |  |  |
| 6 | n258, n261 | n258 | 1.0 | 2.0 |  |  |  | 0.0 | 2.0 |  |  |  |
|  | n258, n261 | n261 | 0.5 | 2.0 |  |  |  | 0.0 | 2.0 |  |  |  |
| 5 | n260, n261 | n260 | 0.0 | 0.0 | 0.0 | 0.0 |  | 0.0 | 1.6 | 1.0 | 0.0 |  |
|  | n260, n261 | n261 | 0.0 | 0.0 | 0.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 |  |
| 4 | n257, n258, n261 | n257 | 1.0 | 2.0 |  |  |  | 0.0 | 2.0 |  |  |  |
|  | n257, n258, n261 | n258 | 1.0 | 2.0 |  |  |  | 0.0 | 2.0 |  |  |  |
|  | n257, n258, n261 | n261 | 0.5 | 2.0 |  |  |  | 0.0 | 2.0 |  |  |  |
| 3 | n257, n260, n261 | n257 | 0.0 |  |  |  |  | 0.0 |  |  |  |  |
|  | n257, n260, n261 | n260 | 0.0 |  |  |  |  | 0.0 |  |  |  |  |
|  | n257, n260, n261 | n261 | 0 |  |  |  |  | 0.0 |  |  |  |  |
| 2 | n258, n260, n261 | n258 | 1.0 |  |  |  |  | 0.0 |  |  |  |  |
|  | n258, n260, n261 | n260 | 1.0 |  |  |  |  | 0.0 |  |  |  |  |
|  | n258, n260, n261 | n261 | 0.0 |  |  |  |  | 0.0 |  |  |  |  |
| 1 | n257, n258, n260, n261 | n257 | 1.8 | 2.1 |  |  |  | 1.9 | 2.4 |  |  |  |
|  | n257, n258, n260, n261 | n258 | 1.8 | 2.1 |  |  |  | 2.0 | 2.4 |  |  |  |
|  | n257, n258, n260, n261 | n260 | 0.0 | 2.0 |  |  |  | 1.3 | 2.0 |  |  |  |
|  | n257, n258, n260, n261 | n261 | 1.8 | 2.0 |  |  |  | 1.2 | 2.4 |  |  |  |

Discussion:

Verizon: for n260 we prefer 0 dB relaxation

Apple: it would be good to consider analysis in our decision

Qualcomm: we would like to support the carriers

Apple: we can suggest 1 dB for n260 in Case 5; it would be incorrect to assert that there is no impact when analysis shows this

Chair: we will continue offline

LGE: if company provides data only for Case 5, and then in Rel-15 RAN4 will only allow this relaxation level in Rel-15; do we need further discussion in a release-independent manner?

Intel: we understood the agreement that any power class related parameter is release independent

Qualcomm: does release-independent mean that 3 months from if an operator wants a new band combination, it can be retroactively inserted into Rel-15?

Verizon: yes, based on existing RAN4 working procedures

Qualcomm: we agree

OPPO: regarding Qualcomm comment, does it mean that release-independence applies only 3 months from now?

Qualcomm: we meant at any future time

Verizon: this is also our understanding

# Applicability

The applicability of the multi-band relaxations should be defined.

## Power class applicability

Option 1: Applicable to all power classes

Option 2: Applicable to PC3 only, multi-band support for other power classes is FFS

Other options are not precluded

Option 3: Applicable to PC2, PC3, PC4, multi-band support for PC1 is FFS

Other options are not precluded

Discussion:

NXP: we prefer to separate fixed and mobile applications

LGE: based on market demand, we prefer to focus on PC3

Qualcomm: would this be release-independent? It would be logical to extend previous agreement

Ericsson: we don’t agree that MB relaxations are necessarily applicable to other power classes

Agreement: we have the ability, if shown that it is needed, to add the multi-band relaxation framework to power classes other than PC3 in a release-independent manner

## Requirement applicability

Option 1: Peak relaxations apply to peak EIRP & peak EIS; spherical relaxations apply to spherical coverage EIRP and spherical coverage EIS (if it is defined)

Option 2: Peak relaxations apply to peak EIRP; spherical relaxations apply to spherical coverage EIRP; multi-band support for EIS is FFS

Option 3: According to the shared pain principle, derive peak (MB_T) & spherical relaxation  (MB_S) according to analysis in Sections 2 & 3; then define Tx relaxation and Rx relaxation based on the total MB relaxation.

Option 4: Leave spherical coverage (EIRP and EIS) multi-band requirement unspecified

Discussion:

Option 1: Apple, Spreadtrum, LGE, Huawei, OPPO

Option 2: NTT DOCOMO, Verizon

Chair: more offline discussion is needed

LGE: this problem is related to antenna performance to support multi-band operation; why is the impact to EIRP only? We understand this impacts both peak and spherical coverage overall; spherical coverage requirement is quite tight; 1 panel case may not satisfy spherical EIS; we recommend applying the MB relaxations to both EIRP and EIS

Chair: can we try to agree on an option that completes the definition of the framework? Suggest Option 3

Verizon: we are not comfortable

Apple: FFS in Option 2 should be clarified; if we clarify, then we have clear option; how can we have relaxation on one direction and not in another? Is additional study required?

Oppo: if there is no relaxation for EIRP, then there is no relaxation for EIS; if there is relaxation for EIRP, does that mean we will have relaxation for EIS?

Intel: if these are the same antennas that are packaged together, then their performance has to be the same; how can they be different?

Qualcomm: we would like to discuss this topic

Agreement: if there is a multi-band relaxation for EIRP (peak or spherical) that is zero, then it is zero for multi-band relaxation for EIS (peak or spherical)

Proposal: if there is a multi-band relaxation for EIRP (peak or spherical) that is non-zero, then multi-band relaxation for EIS (peak or spherical) is non-zero.

NTT DOCOMO: we have a concern; we would like to study further

Sony: we had a similar situation with LTE TRP/TRS which did not conclude; we propose Option 4

OPPO: even though we have the single-band requirement in the specification, we have to design to multi-band scenario; regarding the LTE case, it did not start in the beginning with Rel-8; some companies did continue the work; this should be the last thing we consider; maybe in the beginning we don’t need to consider such a scenario

Proposal: if there is no agreement on any multi-band relaxations (peak or spherical) until November meeting, then we agree to Option 4

Intel: does this mean multi-band relaxations for both peak & spherical or just spherical?

OPPO: we just agreed that this definition would be release-independent; how can we leave this requirement undefined in the specification?

OPPO: until November is too tough; since we agreed release-independence, we should not have a hard deadline

Qualcomm: what is the implication of Option 4? Does it mean you support multi-band but have no requirement?

AT&T: we agree with the original two proposals; we need to agree to something this meeting

MediaTek: we agree with OPPO; we don’t need to close the door to discuss

OPPO: we suggest not to discuss deadlines in the main session; this may suggest incorrect information to other standardization bodies, and this can have negative on RAN4 as a whole

# Remaining issues

Discussion:

# References

- R4-1809786, “Extending FR2 power class requirements to multi-band UEs,” Apple Inc.

- R4-1811021, “FR2 Multi-band Framework for PC3 UEs,” Qualcomm

- R4-1810038, “FR2 MOP Multiband Relaxation Discussion,” MediaTek

- R4-1810611, “UE Spherical EIRP for the 28GHz and 39GHz bands,” Sony

- R4-1811205, “CR for 38.101-2: UE maximum output power for power class 3,” Huawei, HiSilicon

- R4-18xxxxx, OPPO

- R4-1809787, “Draft CR to TS 38.101-2: Introducing multi-band applicability for PC3,” Apple Inc.

- R4-1811022, “Draft CR to 38.101-2: FR2 EIRP Multiband Framework,” Qualcomm