3GPP TSG-RAN WG4 Meeting #88bis	R4-1812053

Chengdu, CN, 8th – 12th Oct 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					3CC UL configurations in REL-15 EN-DC specification

Agenda Item: 			7.6.2

Document for:	Approval

# 1	Introduction

In this contribution we discuss 3CC UL configurations in REL-15 EN-DC specification and if those are allowed or not.

# 2	Discussion

It is our understanding that all Rel-15 EN-DC combinations are type LTE xDL/1UL + 1 NR Band. It is also our understanding that in Rel-15 all baskets were split by number of carriers not by number of bands like in Rel-16.

Now this means that in REL-15 38.101-3 specification there may be up to 2 UL CC’s for any EN-DC configuration, however if we take a look current 38.101-3 we find some EN-DC configuration that have 3 uplink CCs, these are listed in table 1.  Note that one configuration has even non-contiguous intraband uplink CA for band 7 and that has not been standardized for even for LTE not to talk about EN-DC.

Table 1: 3CC UL configurations in REL-15 EN-DC specification

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_41C_n77A | DC_41C_n77A | CA_41C | n77A |
| DC_41C_n78A | DC_41C_n78A | CA_41C | n78A |
| DC_41C_n79A | DC_41C_n79A | CA_41C | n79A |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A DC_41C_n77A | CA_1A-41A CA_1A-41C | n77 |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A | CA_1A-41A CA_1A-41C | n78 |
| DC_1A-41C_n79A | DC_1A_n79A DC_41C_n79A | CA_1A-41C | n79 |
| DC_3A-7C_n78A | DC_3A_n78A DC_7C_n78A | CA_3A-7C | n78A |
| DC_3C-7C_n78A | DC_3A_n78A DC_7C_n78A | CA_3C-7C | n78A |
| DC_7C-28A_n78A | DC_7C_n78A DC_28A_n78A | CA_7C-28A | n78A |
| DC_7A-46C_n78A | DC_7A_n78A DC_46C_n78A | CA_7A-46C | n78A |
| DC_41C-42A_n77A | DC_41C_n77A | CA_41C-42A | n77A |
| DC_41C-42A_n78A | DC_41C_n78A | CA_41C-42A | n78A |
| DC_41C-42A_n79A | DC_41C_n79A | CA_41C-42A | n79A |
| DC_2C_n257A | DC_2C_n257A | CA_2C | n257A |
| DC_2C_n260A | DC_2C_n260A | CA_2C | n260A |
| DC_5B_n257A | DC_5B_n257A | CA_5B | n257A |
| DC_5B_n260A | DC_5B_n260A | CA_5B | n260A |
| DC_41C_n257A | DC_41C_n257A | CA_41C | n257A |
| DC_42D_n257A | DC_42C_n257A | CA_42C | n257A |
| DC_48C_n257A | DC_48C_n257A | CA_48C | n257A |
| DC_48C_n260A | DC_48C_n260A | CA_48C | n260A |
| DC_66C_n257A | DC_66C_n257A | CA_66C | n257A |
| DC_1A-7A-7A_n257A | DC_1A_n257A DC_7A-7A_n257A | CA_1A-7A-7A | n257A |
| DC_1A-41C_n257A | DC_1A_n257A DC_41C_n257A | CA_1A-41C | n257A |
| DC_41A-42C_n257A | DC_41A_n257A DC_42C_n257A | CA_41A-42C | n257A |
| DC_41C-42A_n257A | DC_41C_n257A DC_42A_n257A | CA_41C-42A | n257A |
| DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F | DC_42C_n257A | CA_42C | n257A CA_n257D CA_n257E CA_n257F |

Proposal 1: All 3CC UL configurations in REL-15 EN-DC specification are changed to 2CC UL configuration (1 LTE + 1 NR).

Above proposal means in practise that B-->A, C --> A or A-A --> A.

Unfortunately, same error is captured into release independence specification which states 2 LTE UL CCs. Reason for this is that [1] which introduced the error into 38.307 was made by looking what is in 38.101-3 and then creating the release independence spec. Now as the 38.101-3 may have illegal configuration these were also captured into 38.307.

Proposal 2: Maximum number of E-UTRA uplink carriers in EN-DC mode is change from 2 to 1 in release independence specification. This means Table 8.1.2.1-1 and Table 8.1.2.2-1 of REL-15 in TS 38.307.

# 3	Conclusion

In this contribution we have made following proposal

Proposal 1: All 3CC UL configurations in REL-15 EN-DC specification are changed to 2CC UL configuration (1 LTE + 1 NR).

Proposal 2: Maximum number of E-UTRA uplink carriers in EN-DC mode is change from 2 to 1 in release independence specification. This means Table 8.1.2.1-1 and Table 8.1.2.2-1 of REL-15 in TS 38.307.

If these proposals are acceptable we can provide CR’s to next meeting which implement the proposals.

# 4	References

[1] R4-1806983, TP to TS 38.307: Addition of new Rel-15 features, Nokia, Nokia Shanghai Bell, RAN4#87