3GPP TSG-RAN WG4 Meeting #88	R4-1812132

# Gothenburg, Sweden, 20th – 24th, August 2018

Agenda item:	7.6.12

Source: 	Intel Corporation

Title: 	ACS/IBB/OBB requirements for BW B and BW F

Document for:	Approval

# Introduction

In RAN4#88 meeting it was discussed that for two contiguous CC only CA BW Class C has been considered for the requirements of ACS, IBB and OBB in Rel-15. However, these requirements have not been defined for neither CA BW Class F nor CA BW Class B. In this contribution we provide two possible options for the definition of these Rx requirements for BW Class F and BW Class B

# Discussion

Table 1 shows that for two contiguous CC the NR CA bandwidth classes B, C and F can be taken into account. However, in 38.101-1 the requirements ACS, IBB and OBB defined for two contiguous CC is only CA BW Class C.

Table 1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel_CA ≤ BWChannel,max | 1 |  |
| B | 20 MHz ≤ CBW ≤ 50 MHz | 2 |  |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE: BWChannel, max is maximum channel bandwidth supported among all bands in a release |  |  |  |

It is worth mentioning that currently in the spec TS 38.101-1 for the intra-band contiguous CA only the BW F is defined for n77, n78 and n78 as shown in Table 2.

Table 2: Configurations for intra-band contiguous CA in TS 38.101-1

| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  | aggregated bandwidth (MHz]) |
| --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for the other carrier (MHz |  |
| CA_n77F, CA_n78F, CA_n79F |  | 40 | 20 |  | 60 |
|  |  | 50 | 20 |  | 70 |
|  |  | 40 | 40 |  | 80 |
|  |  | 40 | 50 |  | 90 |
|  |  | 50 | 50 |  | 100 |

In the WF for definition of ACS, IBB and OBB requirements for intra-band contiguous CA [1] two options were provided:

- Option 1:

  - Remove intra-band contiguous CA configuration combinations set for BW Class B and BW F in Rel-15 and define in Rel-16

  - Remove ACS, IBB and OBB requirements for intra-band contiguous CA for BW Class B and F in Rel-15 and define in Rel-16

- Option 2:

  - Mantain intra-band contiguous CA configuration combinations set for BW Class B and BW F in Rel-15

  - Define ACS, IBB and OBB requirements for intra-band contiguous CA for BW Class B and F in Rel-15

  - Unless agreement is not reached on ACS, IBB and OBB requirements remove BW Class B and F at the end of RAN4#89

In order to keep consistency in specification between the defined CA combination and the Rx requirement in the TS 38.101-1, we propose to decide for one of the options explained above, either:

Proposal 1:	Agree on Option 1, remove intra-band contiguous CA configuration combinations set for BW Class B and BW F in Rel-15 and define BW Class B and BW F with ACS, IBB, OBB requirement in Rel-16

OR

Proposal 2: Agree on Option 2, maintain intra-band contiguous CA configuration ser for BW Class B and BW F in Rel-15 and define ACS, IBB and OBB requirements for BW Class F and BW Class B in Rel-15

# Conclusion

In this contribution we provide two options for the definition of the ACS, In-band Blocking and Out-band Blocking for the BW Class F and BW Class B, which are the following:

Proposal 1:	Agree on Option 1, remove intra-band contiguous CA configuration combinations set for BW Class B and BW F in Rel-15 and define BW Class B and BW F with ACS, IBB, OBB requirement in Rel-16

OR

Proposal 2: Agree on Option 2, maintain intra-band contiguous CA configuration set for BW Class B and BW F in Rel-15 and define ACS, IBB and OBB requirements for BW Class F and BW Class B in Rel-15

# References

- R4-1811806, WF for definition of ACS, IBB and OBB requirements for intra-band contiguous CA, Intel

- R4-1810959,  ACS and IBB for intra-band contiguous CA , Intel

- R4-1810960, Out-of-band Blocking for intra-band contiguous CA, Intel

- 3GPP TS 38.101-1 V15.3.0 (2018-09)