3GPP TSG-RAN4 Meeting #88bis 	R4-1812157

# Chengdu, China, 8 – 12 Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NR HO test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-9-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No tests for NR HO defined in TS38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add NR HO Tests for the cases below: - intra-frequency, NR FR1-FR1 HO , TDD , known target cell - inter-frequency, NR FR1-FR1 HO , FDD , unknown target cell - inter-frequency NR FR1-FR2 HO ,TDD ,known target cell - intra-frequency NR FR2-FR2 HO ,TDDl ,unknown target cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test for NR HO is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

### A.6.1.x	NR FR1-FR1 Intra frequency handover

#### A.6.1.x.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements specified in clause 6.1.1.2[TS38.133 v15.2.1].

The test scenario comprises of 1 NR FR1 TDD carrier and 2 cells as given in tables A.6.1.x.1-1 and A.6.1.x.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.6.1.x.1-1: General test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel SR.2.1 TDD | As specified in clause A.3.1.1.2 |
| RMSI CORESET |  |  | DL Reference Measurement Channel CR.2.1 TDD | As specified in clause A.3.1.2.2 |
| SMTC configuration |  |  | SMTC.2 FR1 | As specified in clause A.3.2.1.1.1 |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| NR RF Channel Number |  |  | 1 | Only one TDD carrier frequency is used. |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | 10 | As specified in table Table 6.3.3.2-3 in TS 38.211 |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.1.x.1-2: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T3 | T1 |  | T2 |  |  |  | T3 |  |  |
| NR RF Channel Number |  | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 40 |  |  |  |  | 40 |  |  |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 |  | OP.1 | OP.1 |  | OP.1 |  | OP.1 | OP.1 |  |  |  | OP.1 |  |  |  |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 |  | -3.3 |  | -Infinity |  |  |  | 2.36 |  |  | 2.36 |  |
| Note 2 | dBm/30 KHz | -98 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | 8 |  | 8 |  | -Infinity |  |  | 11 |  |  |  |  | 11 |
| RSRP Note 3 | dBm/30 KHz | -90 | -90 |  | -90 |  | - Infinity |  |  |  | -87 |  |  | -87 |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.6.1.x.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [RRCProcess + 90ms] ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = [TBD] ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt =60 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2 [TS38.133].

This gives a total of [RRCProcess + 60] ms.

### A.6.1.xx 	NR FR1-FR1 Inter frequency handover; unknown Target Cell

#### A.6.1.xx.1	Test Purpose and Environment

This test is to verify the NR FR1-FR1 inter-frequency handover requirements for the case when the target cell is unknown as specified in clause 6.1.1.2.1.

The test scenario comprises of two NR FR1 FDD carriers and one cell on each carrier as given in tables A.6.1.xx.1-1 and A.6.1.xx.1-2. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.6.1.xx.1-1: General test parameters for the NR FR1-FR1 Inter-Frequency handover test case when the target cell is unknown

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel SR.1.1 FDD | As specified in clause A.3.1.1.2 |
| RMSI CORESET |  |  | DL Reference Measurement Channel CR.1.1 FDD | As specified in clause A.3.1.2.2 |
| SMTC configuration |  |  | SMTC.1 FR1 | As specified in clause A.3.2.1.1.1 |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| NR RF channel number |  |  | 1, 2 | Two FDD carriers |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| PRACH configuration |  |  | 10 | As specified in table Table 6.3.3.2-2 in TS 38.211 |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| Gap pattern configuration |  |  | 0 | As specified in TS 38.133 clause 8.1.2.1. |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.6.1.xx.1-2: Cell specific test parameters for the NR FR1-FR1 Inter frequency handover test case when the target cell is unknown

| Parameter | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |  |
| NR RF Channel Number |  | 1 |  | 2 |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.1 |  | OP.1 | OP.1 | OP.1 |  | OP.1 |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |  |  |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -93 |  |
|  | dB | 4 | 4 | -Infinity | 5 |  |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -93 |  |
|  | dB | 4 | 4 | -Infinity | 5 |  |
| Propagation Condition |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### A.6.1.xx.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [RRCProcess + 152ms] from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay	= [TBD]ms, which is specified in clause 12 in TS 38.331 [2].

Tinterrupt	= 152 ms in the test. See clause 6.1.1.2.2.

This gives a total of [TBD+152] ms.

### A.6.1.y	NR FR1-FR2 Inter frequency handover

#### A.6.1.y.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-FR2 inter frequency handover requirements specified in clause 6.1.1.5.

The test scenario comprises of two NR TDD carriers and one cell on each carrier as given in tables Table A.6.1.y.1-1 and Table A.6.1.y.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would not enter the DRX state. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3.

Table A.6.1.y.1-1: General test parameters for NR FR1-FR2 Inter frequency handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Gap Pattern Id |  |  | 0 | As specified in TS 38.133 clause 8.1.2.1. |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbour cell |  | Cell 2 |  |
| Final conditions | Active cell |  | Cell 2 |  |
| NR RF channel number |  |  | 1, 2 | Two TDD carriers are used(one is FR1, the other FR2) |
| A3-Offset |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| Time to Trigger |  | ms | 0 |  |
| Filter coefficient |  |  | 0 |  |
| DRX |  |  | OFF |  |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | 10 | As specified in Table 6.3.3.2-4 in TS 38.211 |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.1.y.1-2: Cell specific test parameters for NR FR1-FR2 Inter frequency handover test case

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  |  |  | T2 |  |  | T3 | T1 |  |  | T2 |  |  | T3 |  |  |
| RF Channel number |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 40 |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel SR.2.1 TDD |  |  |  |  |  |  |  | DL Reference Measurement Channel SR.3.1 TDD |  |  |  |  |  |  |  |  |
| PDCCH parameters |  | DL Reference Measurement Channel CR.2.1 TDD |  |  |  |  |  |  |  | DL Reference Measurement Channel CR.3.1 TDD |  |  |  |  |  |  |  |  |
| SMTC configuration |  | SMTC.2 FR1 |  |  |  |  |  |  |  | SMTC.1 FR2 |  |  |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OCNG pattern 1) |  | OP.1 | OP.1 |  |  |  | OP.1 |  |  | OP.1 | OP.1 |  |  |  | OP.1 |  |  |  |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 4 |  |  |  | 4 |  |  | 4 | -Infinity |  |  |  | 7 |  |  |  | 7 |
| Note 2 | dBm/SCS | -98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 4 |  | 4 |  |  |  |  | 4 | -Infinity |  | 7 |  |  |  |  | 7 |  |
| RSRP Note 3 | dBm/SCS | -94 |  |  | -94 |  |  | -94 |  | -infinity |  |  |  | -91 |  |  |  | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.6.1.y.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [RRCProcess + 90 ms] from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = [TBD] ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 90 ms in the test;  Tinterrupt is defined in clause 6.1.1.5.2[TS38.133].

This gives a total of [TBD+90] ms.

.

### A.6.1.yy 	NR FR2-FR2 Intra frequency handover; unknown Target Cell

#### A.6.1.yy.1	Test Purpose and Environment

This test is to verify the NR FR2-FR2 intra-frequency handover requirements for the case when the target cell is unknown as specified in clause 6.1.1.4.1.

The test scenario comprises of one NR FR2 TDD carriers and 2 cells  as given in tables A.6.1.yy.1-1 and A.6.1.yy.1-2. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.6.1.yy.1-1: General test parameters for the NR FR2-FR2 intra-Frequency handover test case when the target cell is unknown

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel SR.3.1 TDD | As specified in clause A.3.1.1.2 |
| PDCCH parameters |  |  | DL Reference Measurement Channel CR.3.1 TDD | As specified in clause A.3.1.2.2 |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| NR RF channel number |  |  | 1 | Only oneTDD carrier |
| DRX |  |  | OFF | Non-DRX test |
| PRACH configuration |  |  | 11 | As specified in table Table 6.3.3.2-2 in TS 38.211 |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| Gap pattern configuration |  |  | - | No gap pattern configured |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.6.1.xx.1-2: Cell specific test parameters for the NR FR2-FR2 Intra frequency handover test case when the target cell is unknown

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |  |
| RF Channel Number |  | 1 |  | 1 |  |  |
| BWchannel | MHz | 100 |  | 100 |  |  |
| OCNG Patterns defined in A.3.2.1.1 |  | OP.1 | OP.1 | OP.1 |  | OP.1 |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |  |  |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |  |  |  |
| Note 3 | dBm/120kHz | -98 |  |  |  |  |
| RSRP Note 4 | dBm/120kHz | -94 | -94 | -Infinity | -93 |  |
|  | dB | 4 | 4 | -Infinity | 5 |  |
| SCH_RP Note 4 | dBm/120kHz | -94 | -94 | -Infinity | -93 |  |
|  | dB | 4 | 4 | -Infinity | 5 |  |
| Propagation Condition |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### A.6.1.xx.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [RRCProcess + 572 ] from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay	= [TBD]ms, which is specified in clause 12 in TS 38.331 [2].

Tinterrupt	= 572 ms in the test. See clause 6.1.1.4.2.

This gives a total of [RRCProcess + 572] ms.

End of Change 1