3GPP TSG-RAN WG4#88Bis Meeting	R4-1812179

Chengdu, China, 8 – 12 October 2018

Agenda item:	7.13.1.4

Title: 	Simulation results and discussion on NR CQI reporting under AWGN conditions

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests. In RAN4#88 Meeting (August 2018), in [5], we provided our initial results for FR1 FDD and FR2 TDD CQI reporting tests. Also, in RAN4#88 Meeting, a WF [6] was agreed and companies were encouraged to bring static CQI simulation results with median CQI +/-1 and their BLER within [0 ~ 20]dB with 2dB step size. In this contribution, we present our static CQI results for alignment purpose.

# Initial CQI Reporting Results and Discussion

Table 1 lists the common parameters for the CQI reporting test cases specified in this contribution.

Table 1 Test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| Bandwidth | MHz | 10 | 40 | 100 |
| SCS | kHz | 15 | 30 | 120 |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| TDD DL/UL pattern |  | N/A | 7D1S2U | DDSU |
| Propagation channel |  | AWGN | AWGN | AWGN |
| Correlation and antenna configuration |  | 2×2 / 2×4 with static channel specified in [36.101] | 2×2 / 2×4 with static channel specified in [36.101] | 2×2 with static channel specified in [36.101] |
| CSI-RS resourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS periodicity/slot offset |  | 5/1 | 5/1 | 5/1 |
| CSI-RS number of ports (nrofPorts) |  | 2 ports {3000,3001} | 2 ports {3000,3001} | 2 ports {3000,3001} |
| CSI-RS cdm-Type |  | fd-CDM2 | fd-CDM2 | fd-CDM2 |
| CSI-RS firstOFDMSymbolInTimeDomain (CSI-RS-ResourceMapping, l0) |  | [5] | [5] | [5] |
| CSI-RS frequencyDomainAllocation (CSI-RS-ResourceMapping, k0) |  | [000001] | [000001] | [000001] |
| CSI-RS density |  | 1 | 1 | 1 |
| CSI-IM periodicity/slot offset |  | 5/1 | 5/1 | 5/1 |
| CSI-IM-RE-Pattern |  | Pattern0 | Pattern0 | Pattern0 |
| ZPCSI-RS number of ports (nrofPorts) |  | 4 ports {3000, …, 3003} | 4 ports {3000, …, 3003} | 4 ports {3000, …, 3003} |
| ZPCSI-RS firstOFDMSymbolInTimeDomain (CSI-RS-ResourceMapping, l0) |  | [5] | [5] | [5] |
| ZPCSI-RS frequencyDomainAllocation (CSI-RS-ResourceMapping, k0) |  | [000100] | [000100] | [000100] |
| ZPCSI-RS density |  | 1 | 1 | 1 |
| ZP CSI-RS reportConfigType |  | Periodic | Periodic | Periodic |
| Reporting periodicity | ms | 5 | 5 | 5 |
| Reporting offset | ms | 4 | 4 | 4 |
| CQI delay | ms | 8 | 8 | 8 |
| Reporting granularity |  | Wideband | Wideband | Wideband |
| Physical channel for reporting |  | [PUCCH] | [PUCCH] | [PUCCH] |
| reportQuantity |  | [cri-RI-CQI] | [cri-RI-CQI] | [cri-RI-CQI] |
| Codebook type |  | Type I single-panel | Type I single-panel | Type I single-panel |
| Codebook subset restriction |  | 010000 | 010000 | 010000 |
| CQI table |  | Table 5.2.2.1-3 [38.214] | Table 5.2.2.1-3 [38.214] | Table 5.2.2.1-2 [38.214] |

Table 2 and 6 below list our results of Median CQI reporting vs. SNR for FR1 (FDD & TDD, 2x2 & 2x4) with 256QAM CQI table and FR2 (TDD, 2x2) with 64QAM CQI table. It should be noted that the figures listed in Table 2 and 6 were obtained with rather preliminary simulation assumptions and for information only, and modifications to these tables could be needed based on further discussion on NR UE CSI feedback performance requirements.

Table 2. Median CQI vs. SNR (256QAM CQI table for FR1 FDD 2x2)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 1 |  | 0.015 | 0.995 |
| 2 | 3 | 1 | 0 | 0.495 |  |
| 4 | 3 | 1 |  | 0.005 | 0.985 |
| 6 | 4 | 1 |  | 0.085 | 0.98 |
| 8 | 5 | 1 |  | 0.045 | 0.995 |
| 10 | 6 | 1 |  | 0.055 | 0.98 |
| 12 | 7 | 1 |  | 0.045 | 0.965 |
| 14 | 8 | 1 |  | 0.02 | 0.805 |
| 16 | 9 | 1 |  | 0.02 | 0.885 |
| 18 | 10 | 1 |  | 0.01 | 0.83 |
| 20 | 11 | 1 |  | 0.01 | 0.78 |

Table 3. Median CQI vs. SNR (256QAM CQI table for FR1 FDD 2x4)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1 |  | 0 | 1 |
| 2 | 3 | 1 |  | 0 | 0.7674 |
| 4 | 4 | 1 |  | 0 | 0.2512 |
| 6 | 5 | 1 |  | 0 | 0.2 |
| 8 | 7 | 1 |  | 0.0186 | 1 |
| 10 | 8 | 1 |  | 0.0977 | 1 |
| 12 | 9 | 1 |  | 0.014 | 1 |
| 14 | 10 | 1 |  | 0.0837 | 1 |
| 16 | 11 | 1 | 0 | 0.2 |  |
| 18 | 12 | 1 |  | 0.014 | 1 |
| 20 | 13 | 1 |  | 0.0326 | 1 |

Table 4. Median CQI vs. SNR (256QAM CQI table for FR1 TDD 2x2)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 1 |  | 0 | 1 |
| 2 | 3 | 1 |  | 0 | 1 |
| 4 | 3 | 1 |  | 0 | 1 |
| 6 | 4 | 1 |  | 0 | 1 |
| 8 | 5 | 1 |  | 0.045 | 0.995 |
| 10 | 6 | 1 |  | 0 | 0.4145 |
| 12 | 7 | 1 |  | 0 | 0.3224 |
| 14 | 8 | 1 |  | 0 | 0.2895 |
| 16 | 9 | 1 |  | 0 | 0.3750 |
| 18 | 10 | 1 |  | 0 | 0.3750 |
| 20 | 11 | 1 |  | 0 | 0.2829 |

Table 5. Median CQI vs. SNR (256QAM CQI table for FR1 TDD 2x4)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1 |  | 0 | 1 |
| 2 | 3 | 1 |  | 0 | 0.7632 |
| 4 | 4 | 1 |  | 0 | 0.4079 |
| 6 | 6 | 1 | 0 | 0.3487 |  |
| 8 | 7 | 1 |  | 0.0329 | 1 |
| 10 | 8 | 1 |  | 0.0724 | 1 |
| 12 | 9 | 1 |  | 0.0263 | 1 |
| 14 | 10 | 1 | 0 | 0.2829 |  |
| 16 | 11 | 1 | 0 | 0.2829 |  |
| 18 | 12 | 1 |  | 0.0066 | 1 |
| 20 | 13 | 1 |  | 0.1974 | 1 |

Table 6. Median CQI vs. SNR (64QAM CQI table for FR2 TDD 2x2)

| SNR | Median CQI | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1 |  | 0.0583 | 0.6 |
| 2 | 4 | 1 |  | 0.0333 | 0.7583 |
| 4 | 5 | 1 |  | 0.0417 | 0.4083 |
| 6 | 6 | 1 |  | 0.0083 | 0.5083 |
| 8 | 7 | 1 |  | 0.0083 | 0.425 |
| 10 | 8 | 1 |  | 0 | 0.525 |
| 12 | 9 | 1 |  | 0 | 0.5167 |
| 14 | 10 | 1 |  | 0 | 0.6583 |
| 16 | 11 | 1 |  | 0.0083 | 0.5083 |
| 18 | 12 | 1 |  | 0.025 | 0.8917 |
| 20 | 13 | 1 |  | 0.05 | 1 |

Moreover, in order to further align the simulation results, we make the following proposal on NR CQI reporting performance tests.

Proposal 1: Apply simulation assumptions in Table 1 for NR static CQI tests.

# Conclusion

In this contribution, we present our static CQI results for alignment purpose. We make the following proposal.

Proposal 1: Apply simulation assumptions in Table 1 for NR static CQI tests.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1809798, “Initial simulation results and discussion on NR CQI tests”, Intel Corporation, Aug. 2018

[6] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018