3GPP TSG-RAN WG4 Meeting #88bis	R4-1812196

# Chengdu, China, 8th – 12th, October 2018

Agenda item:	7.6.8

Source: 	Intel Corporation

Title: 	CA in FR2

Document for:	Approval

# Introduction

We present our view on applicable CA in FR2 for Rel-15.

# Discussion

Agreement:

The scocpe is Intra band contiguous 800MHz ULCA for FR2 is in Rel15.

- Operators are requested to provide their configurations in the next meetings

- Up to how many CCs and Max agreaged bandwith is feasible is FFS.

In RAN4#88 meeting, we reached the following agreement about UL CA [1].

It was agreed that in Rel-15 only Intra-band contiguous UL CA with up to 800MHz aggregated BW will be defined. But FFS for number of CCs and aggregated BW.

The following table is our proposed CA capability to be supported in Rel-15

Table 1 Proposed CA capability in Rel-15

| Parameters\ Link direction | DL | UL |
| --- | --- | --- |
| Maximum CA bandwidth (MHz) | 1400 | 800 |
| Maximum number of CCs | 8 | 8 |
| Contiguousness | Non-contiguous CCs, Contiguous CCs | Contiguous CCs only |

# Conclusion

In this contribution, we provide our proposed CA capability for Rel-15.

# References

- RAN4#88 Chairman note