3GPP TSG-RAN WG4 # 88 bis Meeting	             R4-1812315

Chengdu, China, 8 – 12 Oct, 2018

Agenda item:	10.3.2

Source: 		Samsung

Title: 	TP on link budget evaluation of 2 RX exception in Rel-15 vehicle mounted UE for NR

Document for:	Approval

# Introduction

According to SI in RAN #81 [1], evaluation to allow exception on 2 RX vehicle mounted NR UE is currently being studied in RAN WG 4. This contribution would aim at focusing on aspects of link budget if 4 RX is deployed in vehicles integrated with 5G NR TCUs. This contribution takes into account the guidance provided by RAN #81 [2], where it was agreed to study coverage in terms of cell edge performance according to European Commission’s Broadband Europe vision of 30 Mbps downlink throughput.

# 5G NR vehicular UE

## Link Budget for antenna systems with 4Rx equipped TCU

As seen in [3], link budgets have been discussed to compare performance of 2 RX vehicle mounted UE and 4 RX HH UE (outdoor, passenger holding and dashboard). In this paper, a specific highlight on link budget extension for 4 RX vehicular UE based on consolidated view of all automotive OEMS in 5GAA [4] is discussed.

Link Budget Analysis for n7/n41 (20MHz CBW) [3]

|  |  | 4Rx V-UE | 2Rx V-UE |
| --- | --- | --- | --- |
| UE Antenna Configuration |  | 4Rx/2Tx | 2Rx/2Tx |
| BS Antenna Gain1 [dB] |  | 13 | 13 |
| UE Antenna Gain | Antenna Gain2,6 [dB] | -12 | -3 |
|  | Body Loss [dB] | 0 | 0 |
|  | Penetration Loss3 [dB] | 9 | 0 |
|  | Total [dB] | -21 | -3 |
| Fading Margin [dB] |  | 9 | 9 |
| Interference Margin [dB] |  | 3 | 3 |
| DL | BS Tx Power [dBm] | 46 | 46 |
|  | UE REFSENS4 [dBm] | -87.4 | -84.7 |
|  | MCLDL [dB] | 113.4 | 128.7 |
| UL | UE Tx Power [dBm] | 26 | 26 |
|  | BS REFSENS5 [dBm] | -95.3 | -95.3 |
|  | MCLUL [dB] | 101.3 | 119.3 |
| Note 6: Based on 5GAA LS [4] for 4 RX V-UE antenna systems in majority of automotive |  |  |  |

# Conclusion

Proposal 1: 5G NR vehicular UE with 4 RX antenna systems cannot perform better than 2 RX vehicular UE due to automotive implementation difficulties. It is therefore essential to consider 2 RX as exception for 5G NR vehicular UEs in Rel-15

****************** Start of the TP in Sub-clause 6 of TR38.826 ************************

# 6. Evaluation on the coverage related issue

As seen in [3], link budgets have been discussed to compare performance of 2 RX vehicle mounted UE and 4 RX HH UE (outdoor, passenger holding and dashboard). In this paper, a specific highlight on link budget extension for 4 RX vehicular UE based on consolidated view of all automotive OEMS in 5GAA [4] is discussed.

Link Budget Analysis for n7/n41 (20MHz CBW) [3]

|  |  | 4Rx V-UE | 2Rx V-UE |
| --- | --- | --- | --- |
| UE Antenna Configuration |  | 4Rx/2Tx | 2Rx/2Tx |
| BS Antenna Gain1 [dB] |  | 13 | 13 |
| UE Antenna Gain | Antenna Gain2,6 [dB] | -12 | -3 |
|  | Body Loss [dB] | 0 | 0 |
|  | Penetration Loss3 [dB] | 9 | 0 |
|  | Total [dB] | -21 | -3 |
| Fading Margin [dB] |  | 9 | 9 |
| Interference Margin [dB] |  | 3 | 3 |
| DL | BS Tx Power [dBm] | 46 | 46 |
|  | UE REFSENS4 [dBm] | -87.4 | -84.7 |
|  | MCLDL [dB] | 113.4 | 128.7 |
| UL | UE Tx Power [dBm] | 26 | 26 |
|  | BS REFSENS5 [dBm] | -95.3 | -95.3 |
|  | MCLUL [dB] | 101.3 | 119.3 |
| Note 6: Based on 5GAA LS [4] for 4 RX V-UE antenna systems in majority of automotive |  |  |  |

****************** End of the TP in Sub-clause 6  of TR38.826 ************************

# References

- RP-181990, Revised SID: Study on evaluation for 2 RX exception in Rel-15 vehicle mounted UE for NR

- RP-182169, LS on impact of 2Rx vehicle mounted UEs on coverage

- R4-1812492, Coverage analysis based on 2RX vehicle NR UE link budget evaluation

- R4-1811528, LS from 5GAA for rel-15 2 RX vehicle mounted UE, 5GAA WG2