3GPP TSG-RAN4 Meeting #88bis	R4-1812528

# Chengdu, China, 8th Oct – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5951 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS36.133 for EN-DC BWP switching interruption on E-UTRA test case(section A7.x) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-9-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no test cases for BWP switching interruptions on E-UTRA serving cells in EN-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test cases: - E-UTRAN FDD – NR FR1 FDD DL active BWP switching interruptions of PCell with non-DRX in asynchronous EN-DC - E-UTRAN TDD – NR FR1 TDD DL active BWP switching interruptions of PCell with non-DRX in synchronous EN-DC - E-UTRAN TDD – NR FR2 TDD DL active BWP switching interruptions of PCell with non-DRX in synchronous EN-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for BWP switching interruptions on E-UTRA serving cells in EN-DC are missed in TS36.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.x.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

A.7.x.x	Active BWP switch interruptions

A.7.x.x.1	E-UTRAN FDD – NR FR1 FDD DL active BWP switch interruptions of PCell with non-DRX in asynchronous EN-DC

A.7.x.x.1.1	Test Purpose and Environment

The purpose of this test is to verify that the DL BWP switch interruption requirements stated in section 7.32.x.

The test scenario comprises of one E-UTRA FDD PCell (Cell 1), one NR FDD PSCell (Cell 2) as given in Tables A.7.x.x.1.1-1. Cell-specific parameters of E-UTRA PCell are specified in Table A.7.x.x.1.1-2 and cell-specific parameters of NR PSCell are specified in Table A.7.x.x.1.1-3 below, each with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 1 interruption.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

All cells have constant signal levels throughout the test.

The test consists of 2 successive time periods, with durations of T1, and T2, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted i. The UE should switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be continuously scheduled on PSCell’s BWP-2.

PCell(Cell 1) interruption due to BWP switch of PSCell shall occur in the slot i to (i + TBD).

During T2,

Time period T2 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 back to BWP-1.

The UE shall be continuously scheduled on PSCell’s BWP-1.

PCell(Cell 1) interruption due to BWP switch of PSCell shall occur in the slot j to (j + TBD).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of PSCell, respectively.

Table 7.x.x.1.1-1: General test parameters for FDD DL BWP switch in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 500 | Asynchronous EN-DC |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 38.104 | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |

Table 7.x.x.1.1-2: E-UTRAN Cell specific test parameters for FDD DL BWP switch in asynchronous EN-DC

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| E-UTRA RF Channel Number |  | 1 |
| PDSCH parametersNote1 |  | R.3 FDD |
| PCFICH/PDCCH/PHICH Note1 |  | R.6 FDD |
| BWchannel | MHz | 10 |
| OCNG PatternsNote1 |  | OP.10 FDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote 2 | dB |  |
| OCNG_RBNote 2 | dB |  |
| NocNote 3 | dBm/15 kHz | -104 |
| RSRP Note 4 | dBm/15 kHz | -87 |
| Ês/Iot | dB | 17 |
| SCH_RP Note 4 | dBm/15 kHz | -87 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table 7.x.x.1.1-3: NR Cell specific test parameters for FDD DL BWP switch in asynchronous EN-DC

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| Frequency Range |  | FR1 |
| BWchannel | MHz | 20 |
| Subcarrier Spacing | KHz | 15 |
| PDSCH parameters |  | SR.1.1 FDD |
| CORESET parameters |  | CR.1.1 FDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| BWP ID |  | 1, 2 |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR1 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.7.x.x.1.2	Test Requirements

During T1 interruption of PCell during PSCell active BWP switch shall not happen outside the slots i to (i+TBD).

During T2 interruption of PCell during PSCell active BWP switch shall not happen outside the slots j to (j+TBD).

The interruption of PCell shall not be more than the values specified for active BWP switch in Section 7.32.x.x.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2 if there are no uplink resources for reporting the ACK/NACK in a slot (i+TBD), (j+TBD) then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

A.7.x.x.2	E-UTRAN TDD – NR FR1 TDD DL active BWP switch interruptions of PCell with non-DRX in synchronous EN-DC

A.7.x.x.2.1	Test Purpose and Environment

The purpose of this test is to verify that the DL BWP switch interruption requirements stated in section 7.32.x.

The test scenario comprises of one E-UTRA TDD PCell (Cell 1), one NR TDD PSCell (Cell 2) as given in Tables A.7.x.x.2.1-1. Cell-specific parameters of E-UTRA PCell are specified in Table A.7.x.x.2.1-2 and cell-specific parameters of NR PSCell are specified in Table A.7.x.x.2.1-3 below, each with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 1 interruption.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

All cells have constant signal levels throughout the test.

The test consists of 2 successive time periods, with durations of T1, and T2, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted i. The UE should switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be continuously scheduled on PSCell’s BWP-2.

PCell(Cell 1) interruption due to BWP switch of PSCell shall occur in the slot i to (i + TBD).

During T2,

Time period T2 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 back to BWP-1.

The UE shall be continuously scheduled on PSCell’s BWP-1.

PCell(Cell 1) interruption due to BWP switch of PSCell shall occur in the slot j to (j + TBD).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of PSCell, respectively.

Table 7.x.x.1.1-1: General test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 38.104 | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |

Table 7.x.x.1.1-2: E-UTRAN Cell specific test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| E-UTRA RF Channel Number |  | 1 |
| Special subframe configuration Note 5 |  | 6 |
| Uplink-downlink configuration Note 5 |  | 1 |
| PDSCH parametersNote1 |  | R.0 TDD |
| PCFICH/PDCCH/PHICH Note1 |  | R.6 TDD |
| BWchannel | MHz | 10 |
| OCNG PatternsNote1 |  | OP.1 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote 2 | dB |  |
| OCNG_RBNote 2 | dB |  |
| NocNote 3 | dBm/15 kHz | -104 |
| RSRP Note 4 | dBm/15 kHz | -87 |
| Ês/Iot | dB | 17 |
| SCH_RP Note 4 | dBm/15 kHz | -87 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211 [23]. |  |  |

Table 7.x.x.1.1-3: NR Cell specific test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| Frequency Range |  | FR1 |
| BWchannel | MHz | 40 |
| Subcarrier Spacing | KHz | 30 |
| Uplink-Downlink Configuration |  | 3D1S1U |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |
| PDSCH parameters |  | SR.2.1 TDD |
| CORESET parameters |  | CR.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| BWP ID |  | 1, 2 |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR1 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.7.x.x.2.2	Test Requirements

During T1 interruption of PCell during PSCell active BWP switch shall not happen outside the slots i to (i+TBD).

During T2 interruption of PCell during PSCell active BWP switch shall not happen outside the slots j to (j+TBD).

The interruption of PCell shall not be more than the values specified for active BWP switch in Section 7.32.x.x.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2 if there are no uplink resources for reporting the ACK/NACK in a slot (i+TBD), (j+TBD) then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

A.7.x.x.3	E-UTRAN TDD – NR FR2 TDD DL active BWP switch interruptions of PCell with non-DRX in synchronous EN-DC

A.7.x.x.3.1	Test Purpose and Environment

The purpose of this test is to verify that the DL BWP switch interruption requirements stated in section 7.32.x.

The test scenario comprises of one E-UTRA TDD PCell (Cell 1), one NR TDD PSCell (Cell 2) as given in Tables A.7.x.x.2.1-1. Cell-specific parameters of E-UTRA PCell are specified in Table A.7.x.x.2.1-2 and cell-specific parameters of NR PSCell are specified in Table A.7.x.x.2.1-3 below, each with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 1 interruption.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

All cells have constant signal levels throughout the test.

The test consists of 2 successive time periods, with durations of T1, and T2, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted i. The UE should switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be continuously scheduled on PSCell’s BWP-2.

PCell(Cell 1) interruption due to BWP switch of PSCell shall occur in the slot i to (i + TBD) if the UE is not capable of per-FR gap, otherwise, the interruption of PCell(Cell 1) is not permitted.

During T2,

Time period T2 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 back to BWP-1.

The UE shall be continuously scheduled on PSCell’s BWP-1.

PCell(Cell 1) interruption due to BWP switch of PSCell shall occur in the slot j to (j + TBD) if the UE is not capable of per-FR gap, otherwise, the interruption of PCell(Cell 1) is not permitted.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of PSCell, respectively.

Table 7.x.x.1.1-1: General test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 38.104 | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |

Table 7.x.x.1.1-2: E-UTRAN Cell specific test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| E-UTRA RF Channel Number |  | 1 |
| Special subframe configuration Note 5 |  | 6 |
| Uplink-downlink configuration Note 5 |  | 1 |
| PDSCH parametersNote1 |  | R.0 TDD |
| PCFICH/PDCCH/PHICH Note1 |  | R.6 TDD |
| BWchannel | MHz | 10 |
| OCNG PatternsNote1 |  | OP.1 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote 2 | dB |  |
| OCNG_RBNote 2 | dB |  |
| NocNote 3 | dBm/15 kHz | -104 |
| RSRP Note 4 | dBm/15 kHz | -87 |
| Ês/Iot | dB | 17 |
| SCH_RP Note 4 | dBm/15 kHz | -87 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211 [23]. |  |  |

Table 7.x.x.1.1-3: NR Cell specific test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | OTA |
| Frequency Range |  | FR2 |
| BWchannel | MHz | 100 |
| Subcarrier Spacing | KHz | 120 |
| Uplink-Downlink Configuration |  | 3D1S1U |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |
| PDSCH parameters |  | SR.3.1 TDD |
| CORESET parameters |  | CR.3.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| BWP ID |  | 1, 2 |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR2 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.7.x.x.2.2	Test Requirements

During T1 interruption of PCell during PSCell active BWP switch shall not happen outside the slots i to (i+TBD) if the UE is not capable of per-FR gap, otherwise, the interruption of PCell(Cell 1) is not permitted..

During T2 interruption of PCell during PSCell active BWP switch shall not happen outside the slots j to (j+ TBD) if the UE is not capable of per-FR gap, otherwise, the interruption of PCell(Cell 1) is not permitted.

The interruption of PCell shall not be more than the values specified for active BWP switch in Section 7.32.x.x.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2 if there are no uplink resources for reporting the ACK/NACK in a slot (i+TBD), (j+TBD) then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

<< End of Change #1>>