3GPP TR 37.716-31-11 V0.1.0 (2018-10)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of 3 bands LTE inter-band Carrier aggregation (CA) (3 Down Link (DL) / 1 Up Link (UL)) and 1 NR band (1 DL / 1 UL)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

1	Scope	6

2	References	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Background	10

4.1	TR Maintenance	10

5	EN-DC of 3 LTE band (3DL/1UL) + 1 NR band: Specific Band Combination Part	10

5.1	Inter-band EN-DC within FR1	11

5.1.1	 DC_1-3-18_n77	11

5.1.1.1	Operating bands for EN-DC	11

5.1.1.2	Configuration for EN-DC	11

5.1.1.3	∆TIB and ∆RIB values	11

5.1.2	 DC_1-3-18_n78	11

5.1.2.1	Operating bands for EN-DC	11

5.1.2.2	Configuration for EN-DC	12

5.1.2.3	∆TIB and ∆RIB values	12

5.1.3	 DC_1-3-18_n79	12

5.1.3.1	Operating bands for EN-DC	12

5.1.3.2	Configuration for EN-DC	12

5.1.3.3	∆TIB and ∆RIB values	13

5.1.4	 DC_1-3-41_n77	13

5.1.4.1	Operating bands for EN-DC	13

5.1.4.2	Configuration for EN-DC	13

5.1.4.3	∆TIB and ∆RIB values	13

5.1.5	 DC_1-3-41_n78	14

5.1.5.1	Operating bands for EN-DC	14

5.1.5.2	Configuration for EN-DC	14

5.1.5.3	∆TIB and ∆RIB values	14

5.1.6	 DC_1-3-41_n79	15

5.1.6.1	Operating bands for EN-DC	15

5.1.6.2	Configuration for EN-DC	15

5.1.6.3	∆TIB and ∆RIB values	15

5.1.7	 DC_3-41-42_n77	16

5.1.7.1	Operating bands for EN-DC	16

5.1.7.2	Configuration for EN-DC	16

5.1.7.3	∆TIB and ∆RIB values	16

5.1.8	 DC_3-41-42_n78	17

5.1.8.1	Operating bands for EN-DC	17

5.1.8.2	Configuration for EN-DC	17

5.1.8.3	∆TIB and ∆RIB values	17

5.1.9	 DC_3-41-42_n79	18

5.1.9.1	Operating bands for EN-DC	18

5.1.9.2	Configuration for EN-DC	18

5.1.9.3	∆TIB and ∆RIB values	18

5.1.10	DC_1A-3A-7C_n78A and DC_1A-3C-7C_n78A	19

5.1.10.1	Operating bands for DC	19

5.1.10.2	Channel bandwidths per operating band for DC	19

5.1.10.3	Co-existence studies	19

5.1.10.4	∆TIB and ∆RIB values	20

5.1.10.5	REFSENS	20

5.2	Inter-band EN-DC including FR2	20

5.2.1	 DC_1-3-18_n257	20

5.2.1.1	Operating bands for EN-DC	20

5.2.1.2	Configuration for DC	20

5.2.1.3	∆TIB and ∆RIB values	21

5.2.2	 DC_1-3-41_n257	21

5.2.2.1	Operating bands for EN-DC	21

5.2.2.2	Configuration for DC	21

5.2.2.3	∆TIB and ∆RIB values	21

5.2.3	 DC_3-41-42_n257	22

5.2.3.1	Operating bands for EN-DC	22

5.2.3.2	Configuration for DC	22

5.2.3.3	∆TIB and ∆RIB values	22

Annex A: Change history	23

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Dual Connectivity (EN-DC) of 3 LTE bands (3DL/1UL) and 1 NR band (1DL/1UL) under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (EN-DC) of 3 LTE band (3DL/1UL) and 1 NR band (1DL/1UL)for the Rel-16 band combinations in Table 1-1. The co-existence analysis and RF front end requirements such as Delta RIB,C and TIB,C are described based on the band combination basis since such information have no difference between the EN-DC configulations consisting with the same E-UTRA band and the same NR band.

Table 1-1: Release 16 EN-DC of 3 LTE band (3DL/1UL) and 1 NR band (1DL/1UL) within FR1

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-5A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A |
| DC_1A-3A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-5A-41A_n79A | DC_1A_n79A DC_5A_n79A DC_41A_n79A |
| DC_3A-5A-41A_n79A | DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-8A-20A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A |
| DC_3A-8A-20A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-3A-41A_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_3A-41C-42C_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41C-42C_n78A | DC_3A_n78A DC_41A_n78A |
| DC_3A-41C-42C_n79A | DC_3A_n79A DC_41A_n79A DC_42A_n79A |
| DC_3A-41C-42A_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41C-42A_n78A | DC_3A_n78A DC_41A_n78A |
| DC_3A-41C-42A_n79A | DC_3A_n79A DC_41A_n79A DC_42A_n79A |
| DC_3A-41A-42C_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42C_n78A | DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42C_n79A | DC_3A_n79A DC_41A_n79A DC_42A_n79A |
| DC_3A-41A-42A_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n78A | DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n79A | DC_3A_n79A DC_41A_n79A DC_42A_n79A |
| DC_1A-3A-18A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DL_ 2A-66A_(n)71B | DC_2A-n71A DC_66A-n71A DC_(n)71B |
| DC_1A-3C-7C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-21A-42D_n77A | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A-42D_n77C | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A-42D_n78A | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A-42D_n78C | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A-42D_n79A | DC_1A_n79A DC_21A_n79A DC_42A_n79A |
| DC_1A-21A-42D_n79C | DC_1A_n79A DC_21A_n79A DC_42A_n79A |
| DC_3A-19A-42D_n77A | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A-42D_n77C | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A-42D_n78A | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A-42D_n78C | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A-42D_n79A | DC_3A_n79A DC_19A_n79A DC_42A_n79A |
| DC_3A-19A-42D_n79C | DC_3A_n79A DC_19A_n79A DC_42A_n79A |
| DC_3A-21A-42D_n77A | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A-42D_n77C | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A-42D_n78A | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A-42D_n78C | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A-42D_n79A | DC_3A_n79A DC_21A_n79A DC_42A_n79A |
| DC_3A-21A-42D_n79C | DC_3A_n79A DC_21A_n79A DC_42A_n79A |

Table 1-2: Release 16 EN-DC of 3 LTE band (3DL/1UL) and 1 NR band (1DL/1UL) including FR2

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-41A_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A |
| DC_3A-41C-42C_n257A | DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_3A-41C-42A_n257A | DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_3A-41A-42C_n257A | DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_3A-41A-42A_n257A | DC_3A_n257A DC_41A_n257A DC_42A_n257A |
| DC_1A-3A-18A_n257A | DC_1A_n257A DC_3A_n257A DC_18A_n257A |
| DC_1A-21A-42D_n257A | DC_1A_n257A DC_21A_n257A DC_42A_n257A |
| DC_1A-21A-42D_n257D | DC_1A_n257A DC_21A_n257A DC_42A_n257A |
| DC_1A-21A-42D_n257E | DC_1A_n257A DC_21A_n257A DC_42A_n257A |
| DC_1A-21A-42D_n257F | DC_1A_n257A DC_21A_n257A DC_42A_n257A |
| DC_3A-19A-42D_n257A | DC_3A_n257A DC_19A_n257A DC_42A_n257A |
| DC_3A-19A-42D_n257D | DC_3A_n257A DC_19A_n257A DC_42A_n257A |
| DC_3A-19A-42D_n257E | DC_3A_n257A DC_19A_n257A DC_42A_n257A |
| DC_3A-19A-42D_n257F | DC_3A_n257A DC_19A_n257A DC_42A_n257A |
| DC_3A-21A-42D_n257A | DC_3A_n257A DC_21A_n257A DC_42A_n257A |
| DC_3A-21A-42D_n257D | DC_3A_n257A DC_21A_n257A DC_42A_n257A |
| DC_3A-21A-42D_n257E | DC_3A_n257A DC_21A_n257A DC_42A_n257A |
| DC_3A-21A-42D_n257F | DC_3A_n257A DC_21A_n257A DC_42A_n257A |
| DC_1A-3A-5A_n257F | DC_1A_n257A DC_3A_n257A DC_5A_n257A |
| DC_1A-3A-5A_n257M | DC_1A_n257A DC_3A_n257A DC_5A_n257A |
| DC_1A-3A-7A_n257F | DC_1A_n257A DC_3A_n257A DC_7A_n257A |
| DC_1A-3A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_7A_n257A |
| DC_1A-3A-7A-7A_n257F | DC_1A_n257A DC_3A_n257A DC_7A_n257A |
| DC_1A-3A-7A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_7A_n257A |
| DC_1A-5A-7A_n257F | DC_1A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-5A-7A_n257M | DC_1A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-5A-7A-7A_n257F | DC_1A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-5A-7A-7A_n257M | DC_1A_n257A DC_5A_n257A DC_7A_n257A |
| DC_3A-5A-7A_n257F | DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_3A-5A-7A_n257M | DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_3A-5A-7A-7A_n257F | DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_3A-5A-7A-7A_n257M | DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-3A-8A_n257A | DC_1A_n257A DC_3A_n257A DC_8A_n257A |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The present document is a technical report for Dual Connectivity (EN-DC) of 3 LTE band (3DL/1UL) and 1 NR band (1DL/1UL) under Rel-16 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	EN-DC of 3 LTE band (3DL/1UL) + 1 NR band: Specific Band Combination Part

<Editor’s note: The requirements for specific band combinations shall be described according to the same mannter as specified in TS38.101-3.>

## 5.1	Inter-band EN-DC within FR1

## 5.1.1		DC_1-3-18_n77

### 5.1.1.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-18_n77 | CA_1-3-18 | n77 |  |

### 5.1.1.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-18A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A | CA_ 1A-3A-18A | n77 |

### 5.1.1.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | n77 | 0.5 |

## 5.1.2		DC_1-3-18_n78

### 5.1.2.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-18_n78 | CA_1-3-18 | n78 |  |

### 5.1.2.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-18A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A | CA_ 1A-3A-18A | n78 |

### 5.1.2.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | n78 | 0.5 |

## 5.1.3	 DC_1-3-18_n79

### 5.1.3.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-18_n79 | CA_1-3-18 | n79 |  |

### 5.1.3.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-18A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A | CA_ 1A-3A-18A | n79 |

### 5.1.3.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n79 | 0 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-n79 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n79 | 0 |

## 5.1.4		DC_1-3-41_n77

### 5.1.4.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41_n77 | CA_1-3-41 | n77 |  |

### 5.1.4.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-41A_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A | CA_ 1A-3A-41A | n77 |
| DC_1A-3A-41C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A | CA_ 1A-3A-41C | n77 |

### 5.1.4.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n77 | 0.5 |

## 5.1.5		DC_1-3-41_n78

### 5.1.5.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41_n78 | CA_1-3-41 | n78 |  |

### 5.1.5.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-41A_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41A | n78 |
| DC_1A-3A-41C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41C | n78 |

### 5.1.5.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n78 | 0.5 |

## 5.1.6		DC_1-3-41_n79

### 5.1.6.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41_n79 | CA_1-3-41 | n79 |  |

### 5.1.6.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A | CA_ 1A-3A-41A | n79 |
| DC_1A-3A-41C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A | CA_ 1A-3A-41C | n79 |

### 5.1.6.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-n79 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n79 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-n79 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n79 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

## 5.1.7		DC_3-41-42_n77

### 5.1.7.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41-42_n77 | CA_3-41-42 | n77 |  |

### 5.1.7.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-41A-42A_n77A | DC_3A_n77A DC_41A_n77A | CA_ 3A-41A-42A | n77 |
| DC_3A-41C-42A_n77A | DC_3A_n77A DC_41A_n77A | CA_ 3A-41A-42A | n77 |
| DC_3A-41A-42C_n77A | DC_3A_n77A DC_41A_n77A | CA_ 3A-41A-42A | n77 |
| DC_3A-41C-42C_n77A | DC_3A_n77A DC_41A_n77A | CA_ 3A-41A-42A | n77 |

### 5.1.7.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-42-n77 | 3 | 1 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41-42-n77 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

## 5.1.8		DC_3-41-42_n78

### 5.1.8.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41-42_n78 | CA_3-41-42 | n78 |  |

### 5.1.8.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-41A-42A_n78A | DC_3A_n78A DC_41A_n78A | CA_ 3A-41A-42A | n78 |
| DC_3A-41C-42A_n78A | DC_3A_n78A DC_41A_n78A | CA_ 3A-41A-42A | n78 |
| DC_3A-41A-42C_n78A | DC_3A_n78A DC_41A_n78A | CA_ 3A-41A-42A | n78 |
| DC_3A-41C-42C_n78A | DC_3A_n78A DC_41A_n78A | CA_ 3A-41A-42A | n78 |

### 5.1.8.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-42-n78 | 3 | 1 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41-42-n78 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

## 5.1.9		DC_3-41-42_n79

### 5.1.9.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41-42_n79 | CA_3-41-42 | n79 |  |

### 5.1.9.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-41A-42A_n79A | DC_3A_n79A DC_41A_n79A | CA_ 3A-41A-42A | n79 |
| DC_3A-41C-42A_n79A | DC_3A_n79A DC_41A_n79A | CA_ 3A-41A-42A | n79 |
| DC_3A-41A-42C_n79A | DC_3A_n79A DC_41A_n79A | CA_ 3A-41A-42A | n79 |
| DC_3A-41C-42C_n79A | DC_3A_n79A DC_41A_n79A | CA_ 3A-41A-42A | n79 |

### 5.1.9.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-42-n79 | 3 | 1 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
|  | n79 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41-42-n79 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n79 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

## 5.1.10	DC_1A-3A-7C_n78A and
DC_1A-3C-7C_n78A

### 5.1.10.1	Operating bands for DC

Table 5.1.10.1-1: DC band combination of LTE 3DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7C_n78A, DC_1A-3C-7C_n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 5.1.10.2	Channel bandwidths per operating band for DC

Table 5.1.10.2-1 Supported bandwidths per DC band combination of LTE 3DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7C_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-7C_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 200 |
|  |  | 3 | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 7 | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 5.1.10.3	Co-existence studies

Co-existence was studied for DC_1A-3A-7A_n78A in Rel-15 and the results are captured in 37.863-03-01. No further studies are needed for DC_1A-3A-7C_n78A and DC_1A-3C-7C_n78A.

### 5.1.10.4	∆TIB and ∆RIB values

For DC_1A-3A-7C_n78A and DC_1A-3C-7C_n78A the TIB,c and RIB values are same as for DC_1A-3A-7A_n78A given in the tables below.

Table 5.1.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7C_n78A, DC_1A-3C-7C_n78A | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |

Table 5.1.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7C_n78A, DC_1A-3C-7C_n78A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n78 | 0.5 |

### 5.1.10.5	REFSENS

No additional MSD requirements need to be defined for these dual connectivity configurations.

## 5.2	Inter-band EN-DC including FR2

## 5.2.1		DC_1-3-18_n257

### 5.2.1.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-18_n257 | CA_1-3-18 | n257 |  |

### 5.2.1.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-18A_n257A | DC_1A_n257A DC_3A_n257A DC_18A_n257A | CA_1A-3A-18A | n257 |

### 5.2.1.3	∆TIB and ∆RIB values

Table 6.2B.4.2.4.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n257 | 0 |

Table 7.3B.3.4.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-n257 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n257 | 0 |

## 5.2.2		DC_1-3-41_n257

### 5.2.2.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41_n257 | CA_1-3-41 | n257 |  |

### 5.2.2.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-41A_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A | CA_1A-3A-41A | n257 |
| DC_1A-3A-41C_n257A | DC_1A_n257A DC_3A_n257A DC_41A_n257A | CA_1A-3A-41C | n257 |

### 5.2.2.3	∆TIB and ∆RIB values

Table 6.2B.4.2.4.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-n257 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.3B.3.4.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-n257 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

## 5.2.3		DC_3-41-42_n257

### 5.2.3.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41-42_n257 | CA_3-41-42 | n257 |  |

### 5.2.3.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-41A-42A_n257A | DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 3A-41A-42A | n257 |
| DC_3A-41C-42A_n257A | DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 3A-41A-42A | n257 |
| DC_3A-41A-42C_n257A | DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 3A-41A-42A | n257 |
| DC_3A-41C-42C_n257A | DC_3A_n257A DC_41A_n257A DC_42A_n257A | CA_ 3A-41A-42A | n257 |

### 5.2.3.3	∆TIB and ∆RIB values

Table 6.2B.4.2.4.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-42-n257 | 3 | 1 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.3B.3.4.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41-42-n257 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-08 | 3GPP RAN4#88 | R4-1811504 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2018-10 | 3GPP RAN4#88bis | R4-1812780 |  |  |  | Implemented TP´s from RAN4 #88: R4-1810110, “TP for TR 37.716-31-11: DC_1-3-18_n77”, KDDI Corporation R4-1810112, “TP for TR 37.716-31-11: DC_1-3-18_n78”, KDDI Corporation R4-1810114, “TP for TR 37.716-31-11: DC_1-3-18_n79”, KDDI Corporation R4-1810120, “TP for TR 37.716-31-11: DC_1-3-41_n77”, KDDI Corporation R4-1810121, “TP for TR 37.716-31-11: DC_1-3-41_n78”, KDDI Corporation R4-1810122, “TP for TR 37.716-31-11: DC_1-3-41_n79”, KDDI Corporation R4-1811427, “TP for TR 37.716-31-11: DC_3-41-42_n77”, KDDI Corporation R4-1811428, “TP for TR 37.716-31-11: DC_3-41-42_n78”, KDDI Corporation R4-1811429, “TP for TR 37.716-31-11: DC_3-41-42_n79”, KDDI Corporation R4-1810398, “TP to 37.716-31-11, DC_1A-3C-7C_n78A, DC_1A-3A-7C_n78A”, Ericsson, BT R4-1810118, “TP for TR 37.716-31-11: DC_1-3-18_n257”, KDDI Corporation R4-1810124, “TP for TR 37.716-31-11: DC_1-3-41_n257”, KDDI Corporation R4-1810136, “TP for TR 37.716-31-11: DC_3-41-42_n257”, KDDI Corporation | 0.1.0 |