3GPP TSG-RAN WG4 Meeting #88bis	R4-1812854

Chengdu, China, 8 – 12 October 2018

Agenda Item:	7.13.1.5

Source:	Ericsson

Title:	Simplified propagation channel models for FR2 UE demodulation requirements

Document for:	Discussion

# 1	Introduction

R RAN4#88 discussed the simplification of TDL channel models for NR UE demodulation and CSI reporting requirements for FR1 and agreed with the way forward [1].

Way forward on channel models simplification for FR2 Agreement: TDL models may not necessarily characterize the spatial FR2 propagation conditions for all scenarios. Confirm Working assumption: Channel Model Option 1 will be used for FR2. TDL Model - TDL-A: 30 ns RMS delay spread - FFS if additional models are needed - Option 1: TDL-C: {60-80} ns RMS delay spread - Option 2: TDL-D: [10-30] ns RMS delay spread - Other models are not precluded Doppler spread - Low speed: 75 Hz for simulation assumptions - High speed: 300 Hz for simulation assumptions Channel models of simulation alignment - Model 1: TDL-A: 30 ns 75 Hz for CSI / PDCCH/ PDSCH/ PBCH - Model 2: TDL-A: 30 ns 300 Hz for PDCCH/ PDSCH / PBCH - Note: use channel models without simplifications for simulation alignment in RAN4 88bis Channel models simplification - The maximum number of taps: [12] - 5 ns quantization grid (200MHz sampling frequency) - Follow same procedure as for FR1 to generate the final channel models

In this contribution, we propose the simplified path profiles used for FR2 UE demodulation requirements according to the agreed way forward.

## 2.1	Simplification procedure

The simplification procedure in this paper is same as what we have done for FR1 [3].

As RAN4#87 discussed, the resolution of delay path grids for TDL channel models in TR38.901 [2] is 1 pico-second (1ps). To avoid the complexity of the test setting, RAN4 discussed to assume the simplification of the channel model.

We should also point of option 1 and option 2 are in principle equivalent. Therefore we use option 2 to find the optimal N values. Our simplification procedure is given as follows:

Step 1: Choose N strongest paths NLOS paths from the original PDPs shows in TR38.901. Step 2: Apply the desired delay spread after removing the weak paths. Step 3: Round the path delay position close to the grid. Set the grid step size to 5 ns. Step 4: If two or more paths are rounded to the same grid τ, the path power at τ is set as a sum of all the path power values (in linear) rounded to τ. Step 5: Normalize the power so that the relative power of the strongest path becomes 0dB.

## 2.2	Simplified	 TDL-C

As we discussed in RAN4#88, the desired delay spread values are in the range of 60ns to 80ns. As an example, we derived the simplified TDL-C model with DS=60ns and 80ns. Table 1 shows the simplified TDL-C with the desired delay spread 60ns and 80ns. For both models, we have selected 18 strong paths so that the final path profiles have up to 12 paths. Figure 1 compares the simplified path profiles with the original profiles.

Regarding the delay spread, the maximum tap delay should not exceed the cyclic prefix length of SCS=120kHz, 590ns. Although the maximum tap delay of TDLC80 is 570ns, we prefer to set DS=60ns considering the timing error.

Table 1	Tap delay and relative power for the simplified TDL-C with Desired DS=60ns and 80ns.

|  |  | TDLC60 |  | TDLC80 |
| --- | --- | --- | --- | --- |
|  | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
| 1 | 0 | -7.8 | 0 | -7.8 |
| 2 | 20 | -0.2 | 25 | -0.9 |
| 3 | 60 | 0 | 30 | -8.6 |
| 4 | 75 | -8.9 | 80 | 0 |
| 5 | 85 | -14.5 | 100 | -8.9 |
| 6 | 115 | -8.5 | 115 | -14.5 |
| 7 | 120 | -10.2 | 150 | -8.5 |
| 8 | 200 | -12.1 | 160 | -10.2 |
| 9 | 250 | -16.6 | 270 | -12.1 |
| 10 | 395 | -17.3 | 335 | -16.6 |
| 11 | 425 | -17.3 | 525 | -17.3 |
| 12 |  |  | 570 | -17.3 |
|  |  | DS=60.0ns |  | DS=80.4ns |

Figure 1	Example of simplified TDL-C. Desired DS=60ns (left) and DS=80ns (right).

## 2.3	Simplified TDL-D

We also applied the same simplification method for TDL-D. Table 2 shows the simplified path delay profiles. TDLD10 means the TDL-D with DS=10ns. Because of the small delay spread, the many taps are overlapped after rounding, and therefore the resulting path profiles have 8 taps without trimming. Both TDLD30A and TDLD30B are the simplified path delay profile of TDL-D with DS=30ns. TDLD30A is the delay profile without trimming. It is observed that the relative powers of some paths, e.g., 120ns, are very small (<-25dB). TDLD30B is the delay path profile after trimming weak paths.

Figure 3 and Figure 4 show the frequency correlations of TDLD10/30A/30B. The LOS path is dominant and therefore the frequency correlation is very high (>0.9) for all the frequencies. It could be beneficial if there is a demand to evaluate UE demodulation performance under static channel condition. In this case we prefer to TDLD10 because it is not so frequency selective.

Table 2	Tap delay and relative power for the simplified TDL-D with Desired DS=10ns and 30ns.

|  |  | TDLD10 |  | TDLD30A |  | TDLD30B |
| --- | --- | --- | --- | --- | --- | --- |
|  | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
| 1 | LOS | -0.2 | LOS | -0.2 | LOS | -0.2 |
| 1 | 0 | -13.5 | 0 | -13.5 | 0 | -13.5 |
| 2 | 10 | -21.1 | 20 | -21.1 | 25 | -21.1 |
| 3 | 15 | -22.9 | 40 | -16.7 | 50 | -22.9 |
| 4 | 20 | -16.8 | 55 | -18.3 | 55 | -18 |
| 5 | 25 | -20.2 | 80 | -22 | 65 | -23 |
| 6 | 30 | -22 | 120 | -27.9 | 70 | -20.2 |
| 7 | 100 | -23.7 | 240 | -23.7 | 95 | -22 |
| 8 | 120 | -24.9 | 285 | -24.9 | 300 | -23.7 |
| 9 |  |  | 295 | -30.1 | 355 | -24.9 |
| 10 |  |  | 380 | -27.8 |  |  |
|  |  | DS=10.2ns |  | DS=30.1ns |  | DS=30.2ns |

Figure 3	Simplified TDL-D (Desired DS=10ns).

Figure 4	Simplified TDL-D (Desired DS=30ns) without path trimming (Left) and with removing the path whose power < -25dB (Right).

# 3	Conclusion

Proposal 1: RAN4 uses TDLC60 (TDL-C with DS=60ns) for UE demodulation requirements.

Proposal 2: If RAN4 will specify the UE demodulation requirements under AWGN-like condition, RAN4 consider to uses TDLD10 (TDL-D with DS=10ns).

Tapped Delay Line C model with DS=60ns (TDLC60)

| Tap delay [ns] | Relative power [dB] |
| --- | --- |
| 0 | -7.8 |
| 20 | -0.2 |
| 60 | 0 |
| 75 | -8.9 |
| 85 | -14.5 |
| 115 | -8.5 |
| 120 | -10.2 |
| 200 | -12.1 |
| 250 | -16.6 |
| 395 | -17.3 |
| 425 | -17.3 |

Tapped Delay Line D model with DS=10ns (TDLD10)

|  | Tap delay [ns] | Relative power [dB] |
| --- | --- | --- |
| 1 | LOS | -0.2 |
| 1 | 0 | -13.5 |
| 2 | 10 | -21.1 |
| 3 | 15 | -22.9 |
| 4 | 20 | -16.8 |
| 5 | 25 | -20.2 |
| 6 | 30 | -22 |
| 7 | 100 | -23.7 |
| 8 | 120 | -24.9 |

# 4	References

R4-1811725, “Way forward on channel model for NR performance requirements”, Huawei, Spirent.

3GPP TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

R4-1812853, “Simplified propagation channel models for FR1 UE demodulation requirements”, Ericsson.

# Appendix

Table 7.7.2-4. TDL-D.

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -13.5 | Rayleigh |
| 2 | 0.035 | -18.8 | Rayleigh |
| 3 | 0.612 | -21 | Rayleigh |
| 4 | 1.363 | -22.8 | Rayleigh |
| 5 | 1.405 | -17.9 | Rayleigh |
| 6 | 1.804 | -20.1 | Rayleigh |
| 7 | 2.596 | -21.9 | Rayleigh |
| 8 | 1.775 | -22.9 | Rayleigh |
| 9 | 4.042 | -27.8 | Rayleigh |
| 10 | 7.937 | -23.6 | Rayleigh |
| 11 | 9.424 | -24.8 | Rayleigh |
| 12 | 9.708 | -30.0 | Rayleigh |
| 13 | 12.525 | -27.7 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 13.3 dB and a mean power of 0dB. |  |  |  |

Table 7.7.2-5. TDL-E.

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.03 | LOS path |
|  | 0 | -22.03 | Rayleigh |
| 2 | 0.5133 | -15.8 | Rayleigh |
| 3 | 0.5440 | -18.1 | Rayleigh |
| 4 | 0.5630 | -19.8 | Rayleigh |
| 5 | 0.5440 | -22.9 | Rayleigh |
| 6 | 0.7112 | -22.4 | Rayleigh |
| 7 | 1.9092 | -18.6 | Rayleigh |
| 8 | 1.9293 | -20.8 | Rayleigh |
| 9 | 1.9589 | -22.6 | Rayleigh |
| 10 | 2.6426 | -22.3 | Rayleigh |
| 11 | 3.7136 | -25.6 | Rayleigh |
| 12 | 5.4524 | -20.2 | Rayleigh |
| 13 | 12.0034 | -29.8 | Rayleigh |
| 14 | 20.6519 | -29.2 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 22 dB and a mean power of 0dB. |  |  |  |