3GPP TSG RAN WG4 Meeting #88Bis	R4-1813276

Chengdu, China, October 8 – 12, 2018

Agenda Item:	7.5.1

Source:	Huawei, HiSilicon

Title:	SCS restrictions for BWP operations for SS block

Document for:	Approval

# Introduction

In RAN#88, document [1] discussed the topic of placing a restriction on SCS for the SS block based on the configured bandwidth. Several options were captured on-line

- Restrict the BWP BW according to SSB SCS configured in the BWP

- Introduce different UE capability

- No further restriction if no UE capability

This contribution evaluates the options listed and proposes no further restriction is needed.

# Discussion

For FR1, the supported SCS for the SS block are 15 kHz and 30 kHz. In 38.104 Table 5.4.3.3-1, the mapping of supported SCS to the bands is provided. From the analysis, there are three bands where 15 kHz SCS for SS block is used for cell search and the maximum bandwidth for 30 kHz SCS is greater than 50 MHz.

Table 1. FR1 TDD bands where 30 kHz SCS > 50 MHz and 15 kHz SCS for initial access SS block is supported

| Band | Frequency | BW, MHz | Cell search | Max BW for 30 kHz SCS |
| --- | --- | --- | --- | --- |
| n40 | 2300 MHz – 2400 MHz | 100 | Case A | 100 MHz |
| n41 | 2496 MHz – 2690 MHz | 194 | Case A, B | 100 MHz |
| n50 | 1432 MHz – 1517 MHz | 85 | Case A | 80 MHz |

Observation: using the initial access SS block for measurements in three FR1 bands may cause a scenario of 15 kHz SCS for the SCS block and CC BW greater than 50 MHz.

However, additional analysis of the signaling indicates the SCS used for measurements may not be an issue. In 38.331, the information element MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements or CSI-RS intra/inter-frequency measurements.

-- ASN1START

-- TAG-MEAS-OBJECT-NR-START

MeasObjectNR ::=			SEQUENCE {

ssbFrequency			ARFCN-ValueNR		OPTIONAL,	-- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing		SubcarrierSpacing	OPTIONAL,	-- Cond SSBorAssociatedSSB

…

Because the measurement object allows the network to specify the frequency location of the SS block and the SCS, the network can ensure that 30 kHz SCS is used for the SS block when the CC bandwidth greater than 50 MHz. Thus based on the signaling:

Proposal 1: No restriction on network behavior is needed for SCS used in measurements.

# Conclusion

This contribution examines the SCS used for the SS block. For initial access

Observation: using the initial access SS block for measurements in three FR1 bands may cause a scenario of 15 kHz SCS for the SCS block and CC BW greater than 50 MHz.

However, because the signaling for measurements allows the network to configure the SCS for the SS block:

Proposal 1: No restriction on network behavior is needed for SCS used in measurements.

# References

R4-1811544, “Way forward on SCS of SSB restriction for RRM measurement in CC BW”, Samsung, RAN4#88, Aug. 20-24, 2018.

38.331