3GPP TSG-RAN Working Group 4 Meeting #88-Bis	R4-1813357

Chengdu, China, 8th October – 12th October 2018

Source:	Nokia, Globalstar

Title:	TP to TR 36.791 (E-UTRA 2.4 GHz TDD Band for US): PSD limit evaluation for UE

Document for:	Approval

Agenda Item:	          6.2.2

## 1	Introduction

The FCC has recently adopted rules for terrestrial use of the 2.4 GHz MSS band.  This band is authorized for TDD

operation and spans 11.5 MHz from 2483.5 MHz to 2495 MHz.  A new work item was created in RAN#79, to create

specifications for a corresponding new E-UTRA operating band [1].

The WI has a study phase, with the following core objectives:

- Define impact on 2.4 GHz Bluetooth operations

- Define impact on 2.4 GHz 802.11 operations

- Address potential Bluetooth and 802.11 coexistence issues

A skeleton TR 36.791 (E-UTRA 2.4 GHz TDD Band for US) was approved in RAN4#88 [2].

One aspect suggested to be covered in the study phase is the PSD limitation of 8 dBm/3 kHz. This TP covers this aspect for the UE transmissions.

## 2	References

[1]	  RP-180583, New WI Proposal for E-UTRA 2.4 GHz TDD Band, Globalstar, RAN#79

[2]	  3GPP TR 36.791 “E-UTRA 2.4 GHz TDD Band for US”

<<< Start of TP >>>

# 7	E-UTRA Specific Issues

# 7.1 	UE Aspects

### 7.1.1	Power Spectral Density limit

According to the FCC power limits outlined in clause 5.4.3, the maximum power spectral density conducted to the antenna is not greater than 8 dBm in any 3 kHz band during any time interval of continuous transmission. For UE transmissions, the highest PSD may be found during narrowband 1 PRB transmissions. Such resource allocation is simulated to verify that the limit is not exceeded.

## Figure 7.1-1:  Power Spectral Density of 1 PRB UL transmission at maximum 23 dBm output power, complete channel bandwidth (left) and zoomed to the transmitter resource block (right). The maximum PSD is 6.7 dBm/3 kHz.

At the maximum power, the highest power spectral density is 6.7 dBm/3 kHz, thereby no special measures are required to meet the FCC requirement.

<<< End of TP>>>