3GPP TSG-RAN WG4 Meeting RAN4#88bis		R4-1813407

Chengdu, China, 8-12 October, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0053 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-2 correction co-location test antenna description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-perf |  |  |  |  |  |  | Date: |  | 2018-08-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Only the co-location test antenna (CLTA) description is included in the test specification, howbere some of the definitions from the co-location referneec antenna part in the core specification are needed and are missing. Specificall the definition of d. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of d is added to the CLTA description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The co-location requirements are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |

### 4.15.2	Co-location test antenna

#### 4.15.2.1	General

Co-loation requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformace testing the requirements are translated to the output of test antenna.

The co-location test antenna (CLTA) defines a practical BS antenna which can be used to test the co-location requirements.

#### 4.15.2.2	Co-location test antenna characteristics

The co-location test antenna is a practical passive antenna based on the definition of the co-location reference antenna which is used for conformance testing of the co-location requirements. The CLTA is described in Table 4.15.2.2-1.

The CLTA is suitable for testing AAS BS implemented with a planar antenna array. AAS BS with another antenna array implimentations are FFS.

Table 4.15.2.2-1: CLTA charactersitics

| Parameter | in band | out of band |
| --- | --- | --- |
| Height (h) | Test object radiating length ±30% | N/A |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | Vertical beam width in-band ± 3° |
| Polarization | Match | Match to in-band |
| Conducted interface return loss | > 10dB | > 10dB |
| NOTE 1: If a multi-column or multi-band antenna is used the column closest to the AAS BS shall be selected while other columns are terminated during testing. |  |  |

#### 4.15.2.3	Co-location test antenna alignemnt

The alignment between the AAS BS statation under test and the co-location test antenna is described in Table 4.15.2.3-1 and Figure 4.15.2.3-1.

Table 4.15.2.3-1: CLTA alignment tolerances

| Parameter | in band | out of band |
| --- | --- | --- |
| Edge-to-edge separation between the AAS BS and the co-location reference antenna, d |  | 10cm ± 1cm |  | 10cm ± 1cm |  |
| Vertical alignment | Centre ± 1cm | Centre ± 1cm |
| Front alignment | Radome front ± 1cm | Radome front ± 1cm |

Figure 4.15.2.3-1 Alignment of AAS BS and CLTA