3GPP TSG-RAN WG4 Meeting #88b 	R4-1813445

# Chengdu, China, October 8th-12th 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104 – PUSCH requirements for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-10-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding demodulation requirement for PUSCH FR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new requirements based on WF R4-1811694 for simulation assumptions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New requirement for PUSCH can’t be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 11.3 Performance requirements for PUSCH

### 11.3.1	Requirements in FR1

### 11.3.2	Requirements in FR2

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ retransmissions.

Table 11.3.2-1 Test parameters for testing PUSCH

| Parameter | Value |
| --- | --- |
| Maximum number of HARQ transmissions | 4 |
| RV sequence | 0, 2, 3, 1, 0 |
| TDD pattern | DDSU |

#### 11.3.2.1	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in the tables 11.3.2.1-1 to 11.3.2.1-2 at the given SNR for 1Tx with subcarrier spacings as 120kHz and 60kHz.

Table 11.3.2.1-1 Minimum requirements for PUSCH, 100 MHz Channel Bandwidth, 120kHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | FRC (Annex TBD) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | [AWGN] | A-MCS2 | 70% | TBD |
|  |  |  |  | A-MCS16 | 70% | TBD |
|  |  |  |  | A-MCS20 | 70% | TBD |
|  | 2 | Normal | [AWGN] | A-MCS2 | 70% | TBD |
|  |  |  |  | A-MCS16 | 70% | TBD |
|  |  |  |  | A-MCS20 | 70% | TBD |
|  |  |  |  | A-MCS2-PRB30 | 70% | TBD |

Table 11.3.2.1-2 Minimum requirements for PUSCH, 100 MHz Channel Bandwidth, 60kHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | FRC (Annex TBD) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | [AWGN] | A-MCS2 | 70% | TBD |
|  |  |  |  | A-MCS16 | 70% | TBD |
|  |  |  |  | A-MCS20 | 70% | TBD |
|  | 2 | Normal | [AWGN] | A-MCS2 | 70% | TBD |
|  |  |  |  | A-MCS16 | 70% | TBD |
|  |  |  |  | A-MCS20 | 70% | TBD |

<End of changes>