3GPP TSG-RAN WG4#88bis Meeting                                                                R4-1813571

# Chengdu, China, 08-12 Oct. 2018

Source:	Volkswagen AG

Title:	TP on general aspect for vehicle mounted NR UE

Agenda item:	10.4.2

Document for:	Approval

# Introduction

During the RAN4#88 meeting, RAN4 discussed and defined the evaluation scenarios and the link-budget parameters to be used for the evaluation of the coverage in TR 38.826. In [1] it was agreed that the parameters of the vehicular mounted antennas are fundamental for this evaluation.

According to [1] antenna implementations supporting band n7 will be used as an indication of the performance in band n77 and n78.

Based on this assumption, the 5G Automotive Association (5GAA) provided data of typical implementations of vehicular mounted antennas in LS [2]. This information should be used to define the parameter values for the baseline antenna gain.

# Discussion

The cellular communication has a significant role for the current and future service portfolio in the vehicle business. The service portfolio is divided in vehicle centric services (i.e. vehicle status monitoring) and passenger centric services (i.e. multimedia). 5G will enable new types of services (high density maps downloads) or further improve existing services, (software updates, sensor sharing).

Observation 1. Services for passengers and vehicle manufacturers benefit from the 5G NR communication technology.

The vehicle manufacturers are aware of the relation between the service and communication system requirements. However, the vehicle specific limitations in the implementation of communication systems force the vehicle manufacturers to achieve reasonable complexity of the vehicle design. On the other hand, the implementation of the communication system in vehicles offers advantages compared to the mobile phone design. Both aspects, the implementation and benefits (antenna gain and directivity) of the antenna implementation, were presented and described in [3] and [4]. Further, vehicle manufacturers in the 5G Automotive Association intensively discussed this issue with telecom partners (MNOs, network equipment vendors and communication chip vendors). After a review of some of the typical antenna implementations of several vehicle manufacturers 5GAA WG2 send the results of this discussion in the LS [2] to RAN4:

Table 1: Appendix of the 5GAA LS to RAN 4 on the baseline antenna gains [2]

Baseline Antenna Gain Values: OEM A: The cable loss depends on the chosen package and mounting concept of the telematics unit and the antenna. OEM A has chosen a concept which allows to eliminate the cable loss completely in nearly all cars. - The average gain of the 2x2 MIMO antenna is about -1 dBi at 2.6 GHz. - The average loss of a coax cable is about 1 dB/m. - This is an example for better performance than baseline antenna values including cable loss, mentioned in section 1 of this document. Baseline Antenna Gain Values: OEM B: The following antenna gain (dBi) values are expected minimum in the frequency ranges until 3.8GHz: - 2 RX scenarios: - Antenna 1: Average Antenna gain (theta=60°-90°, phi=0° to 359°, Car implementation on PEC) including cable loss: 1 dBi, based on a rooftop antenna implementation - Antenna 2: Average Antenna gain (theta=60°-90°, phi=0° to 359°, Car implementation on PEC) including cable loss: 0 dBi, based on a second radiator in rooftop antenna implementation - This is an example for better performance than baseline antenna values including cable loss, mentioned in section 1 of this document. Baseline Antenna Gain Values: OEM C: The cable loss depends on the chosen package and mounting concept of the telematics unit and the antenna. - The average gain of the 2x2MIMO antennas is about -2/-3 dBi at 2.5 GHz. - The average loss of a coax cable is about 1 dB/m at 2.5 GHz - Window antenna: - Antenna Gain: -2 dBi / Cable length: 2 m / Overall gain: -4 dBi This is an example for performance which is closer to baseline antenna values including cable loss, mentioned in section 1 of this document.

These results show the baseline antenna gain values for the typical implementations to provide good and sufficient service experiences. It is important to emphasis that this values are based on semi-omni directive antenna radiation pattern (as shown in Table 2).

Table 2: Directivity of the typical antenna implementation in vehicles.

| OEM A (Green curve: radiation pattern of vehicle, Red curve: radiation pattern of an example HH, example @ 1.8GHz) | OEM B (Green curve/blue curve/ red curve: radiation pattern of vehicle (V/H/Sum), example @ 2.5GHz) | OEM C (Green curve/blue curve/ red curve: radiation pattern of vehicle (V/H/Sum), example @ 2.5GHz, Elevation =70°..90°, azimuth =0..360°) |
| --- | --- | --- |

Observation 2. 5GAA provided consolidated baseline values for antenna gains for typical implementation.

In the discussion in RAN#88 the agreed target for the evaluation of the coverage was to limit the number of evaluation scenarios and cases. Due to this constrain, one baseline value for the 2Rx antenna gain should be selected for the TR.

Proposal 1. For the purpose of coverage evaluation a baseline antenna gain including antenna cable loss of vehicle mounted UE with 2 RX antennas: - 3 dBi to – 4 dBi shall be assumed.

# Conclusion

In this paper, we summarise the status and propose to use the following antenna gain values for the link budget calculation to evaluate the coverage performance in the TR.

Proposal 1. Baseline antenna gain including antenna cable loss of vehicle mounted UE with 2 RX antennas: - 3 dBi to – 4 dBi.

****************** Start of the TP in Sub-clause 5 of TR38.826 ************************

# 5	General aspects for NR vehicle mounted UE at FR1

## This section contains relevant aspects for the link budget evaluation.

## 5.X Antenna implementation for 2 Rx vehicle mounted UE

The following implementation examples are a selection of typical antenna implementation incl. cable loss.

5.X.1 Typical Baseline Rx Antenna Gain in the Market

The sub-section illustrates selection of antenna gain values of prominent vehicle manufacturers.

#### 5.X.1.1 Baseline Antenna Gain Values: OEM A

The cable loss depends on the chosen package and mounting concept of the telematics unit and the antenna. OEM A has chosen a concept which allows to eliminate the cable loss completely in nearly all cars.

- The average gain of the 2x2 MIMO antenna is about -1 dBi at 2.6 GHz.

- The average loss of a coax cable is about 1 dB/m.

- This is an example for better performance than baseline antenna values including cable loss.

#### 5.X.1.2 Baseline Antenna Gain Values: OEM B

The following antenna gain (dBi) values are expected minimum in the frequency ranges until 3.8GHz:

- 2 RX scenarios:

  - Antenna 1: Average Antenna gain (theta=60°-90°, phi=0° to 359°, Car implementation on PEC) including cable loss: 1 dBi, based on a rooftop antenna implementation

  - Antenna 2: Average Antenna gain (theta=60°-90°, phi=0° to 359°, Car implementation on PEC) including cable loss: 0 dBi, based on a second radiator in  rooftop antenna implementation

- This is an example for better performance than baseline antenna values including cable loss.

#### 5.X.1.3 Baseline Antenna Gain Values: OEM C

The cable loss depends on the chosen package and mounting concept of the telematics unit and the antenna.

- The average gain of the 2x2MIMO antennas is about -2/-3 dBi at 2.5 GHz.

- The average loss of a coax cable is about 1 dB/m  at 2.5 GHz

- Window antenna:

  - Antenna Gain: -2 dBi / Cable length: 2 m  / Overall gain: -4 dBi

This is an example for performance which is closer to baseline antenna values including cable loss.

## 5.X.2 Baseline Rx antenna gain

Table 3 defines the baseline Rx antenna gain for the link budget calculation.

Table 3: Reference baseline Rx antenna gain.

| Link budget parameter name | Link budget parameter value |
| --- | --- |
| Rx antenna gain | -4 [dBi] |

****************** End of the TP in Sub-clause 5 of TR38.826 ************************

# Reference

- R4-1811897, WF on Evaluation for 2 RX exception in Rel-15 vehicle mounted UE, Volkswagen AG, LG Electronics, Samsung, General Motors, Ericsson, Huawei, Telecom Italia, Vodafone, CMCC

- R4-1811528, LS from 5GAA for rel-15 2 RX vehicle mounted UE, 5GAA WG2

- RP-181117,  Minimum requirement of 2Rx antennas for permanently mounted UE in vehicles for NR, Volkswagen AG

- RP-181282, Coverage related issues for 2Rx RF requirements of vehicle mounted NR UE as exception case, Samsung, LG Electronics, Volkswagen AG, Daimler AG, Toyota Info Technology Center