3GPP TSG RAN WG4 Meeting #88bis		R4-1813608

Chengdu, CN, 8th – 12th Oct, 2018

Source: 	NTT DOCOMO, INC.

Title: 	Remaining issues on PDCCH demodulation requirements

Agenda item:	7.13.1.3

Document for:	Approval

- Introduction

In the last RAN4 meeting, RAN4 discussed PDCCH demodulation requirements and the agreements were summarized in [1]. In this contribution, we discuss the remaining issues for PDCCH test cases.

- Discussion

2.1.	Aggregation level

The agreement at the last meeting is as follows:

- Aggregation levels - AL2, AL4, AL8 - FFS for AL16 - See page 6 for evaluation of PDCCH AL16 scenario.

The remaining issue is whether to introduce test cases for AL = 16 in FR1 and FR2. AL = {2, 4, 8} would be the most likely values in the real deployment, However, all values are mandatory without capability [2], thus for future proof any prioritization should not be taken in NR PDCCH requirements. Especially, AL = 16 would beneficial to extend cell coverage. In Fig. 1, we show the performance comparison between AL = {1, 2, 4, 8, 16} in FR1. Note that the simulation assumption is summarized in Annex. From the results, AL=16 has better demodulation performance (about 2 dB) than AL=8.

Fig.1 Performance comparison between aggregation levels.

For FR1, on the other hand, COREST time duration is {1, 2} [1]. For FR2, however, only 1 symbol duration was assumed as follows.

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | Antenna configuration with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 120 | 1_0 | [60] | 41 | 1 | 2 | Interleaved | 2 | [TDL-A, 30ns, 75Hz] | 1x2 Low | NA |
| 10 | 120 | 1_1 | [60] | [51] | 1 | 4 | Interleaved | 6 | [TDL-A, 30ns, 300Hz] | 1x2 Low | NA |
| 11 | 120 | 1_1 | [60] | [51] | 1 | 8 | Interleaved | 2 | [TDL-A, 30ns, 75Hz] | 2x2 Low | NA |

1 symbol CORESNT would be most likely to be used for FR2 considering analogue beam forming, but 2 symbol COREST needs to be specified from test coverage point of view. Fortunately, 2 symbol COREST is assumed for AL = 16 test case, thus the agreed simulation assumption should be used for the actual requirement for NR PDCCH.

Proposal: Specify the PDCCH demodulation requirements not only for AL = {2, 4, 8} but also AL = {16} in both FR1 and FR2.

- The following is simulation assumption for AL=16 (green part is modified part from [1])

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | Antenna configuration with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | [TDL-A, 30ns, 10Hz] | 1x2 Low | 1x4 Low |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | [TDL-C, 300ns, 100Hz] | 1x2 Low | 1x4 Low |
| 16 | 120 | 1_0 | [60] | 41 | 2 | 16 | Interleaved or Non-interleaved | 2 or 6 | [TDL-A, 30ns, 75Hz] | 2x2 Low | NA |

- Conclusion

In this contribution, we discuss the remaining issues for PDCCH test cases.

Proposal: Specify the PDCCH demodulation requirements not only for AL = {2, 4, 8} but also AL = {16} in both FR1 and FR2.

- The following is simulation assumption for AL=16 (green part is modified part from [1])

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | Antenna configuration with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | [TDL-A, 30ns, 10Hz] | 1x2 Low | 1x4 Low |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | [TDL-C, 300ns, 100Hz] | 1x2 Low | 1x4 Low |
| 16 | 120 | 1_0 | [60] | 41 | 2 | 16 | Interleaved or Non-interleaved | 2 or 6 | [TDL-A, 30ns, 75Hz] | 2x2 Low | NA |

- Reference

- R4-1811724, Ericsson, Way forward on NR PDCCH demodulation requirements, Aug, 2018.

- RP-181483, RAN1 NR UE feature list, NTT DOCOMO, Jun. 2018.

# Annex

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Bandwidth | 40 MHz |
| Subcarrier spacing | 30 kHz |
| Channel model | TDL-C |
| PDCCH payload size | 40 bits |
| CRC size | 24 bits |
| Aggregation level | {1, 2, 4, 8, 16} |
| OFDM symbol size | 1 |
| Assigned RBs | 96 |
| Resource mapping | Distributed |
| REG bundle size | 6 |
| Interleaver row | 6 |
| Transmission scheme | Precoder cycle |
| gNB antenna config | 2 TX |
| UE antenna config | 4 RX |
| Delay spread | 300 ns |
| Velocity of UE | 3 km/h |