3GPP TSG-RAN WG4 Meeting #88bis	R4-1813685

Chengdu, China, 8 – 12 October 2018

Agenda Item:	6.4.7

Source:	Ericsson

Title:	Simulation results of NPRACH demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#88 agreed with the way forward on Rel-15 NB-IoT BS demodulation requirements [1].

Table 1 shows the parameters used for the NPRACH preamble format 2 tests and Table 2 lists the final requirement table. This contribution provides our simulation results for NPRACH preamble format 2.

Table 1	Parameters for NPRACH preamble format 2 performance requirements.

| Parameter | Values |
| --- | --- |
| NPRACH preamble format | 2 |
| Number of Repetitions | 8, 32 |
| Number of Subcarriers | 12 |
| Antenna configuration | 1 x 2 |
| Narrowband physical layer cell identity | 0 |
| NPRACH signature | 0 |
| frequency offset | 0Hz (AWGN); 200Hz EPA1 |
| Propagation channels | AWGN, EPA1 |
| Detection performance | Missed detection rate (1%) with false alarm rate (0.1%) |
| Timing estimation | Timing error probability with the limit of 3.646us |
| Timing Offset | T_CP/2 |

Table 2	Test case for NPRACH performance requirements.

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 2 |
| 1 | 2 | 8 | AWGN | 0 | -2.1 | -2.1 | TBD |
|  |  |  | EPA1 Low | 200 Hz | 6.1 | 6.1 | TBD |
|  |  | 32 | AWGN | 0 | -6.8 | -6.8 | TBD |
|  |  |  | EPA1 Low | 200 Hz | 0.5 | 0.5 | TBD |

# 2	Simulation results

Table 3 shows our NPRACH format 2 simulation results for alignment.

Table 3	NPRACH format 2 simulation results for alignment.

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 2 |
| 1 | 2 | 8 | AWGN | 0 | -1.7 |
|  |  |  | EPA1 Low | 200 Hz | 8.4 |
|  |  | 32 | AWGN | 0 | -6.2 |
|  |  |  | EPA1 Low | 200 Hz | -0.2 |

# References

- R4-1811371, “Way forward on BS FeNB-IoT demodulation performance”, Huawei.