3GPP TSG-RAN WG4 Meeting #88bis	R4-1813860

Chengdu, China, 8 – 12 October 2018

Source:	Ericsson

Title:	TP to TR 36.716-02-01: Addition of LTE configuration for CA_28-32

Agenda item:	8.2.2

Document for:	Approval

# Background

This contribution provides text proposal on the LTE CA configuration 28A-32A_BCS0.

# Text Proposal

##### ---Start of changes---

## 5.X	CA_28-32

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_28A-32A | - | 28 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.x.2	UE co-existence studies

Table 5.x.3-1 gives the harmonic frequency limits for Band 28 and Band 32 2DL CA. As shown in table 5.x.3-1, UL carrier of band 28 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.x.3-1: DL harmonics frequency limits for CA of Band 28 and Band 32

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 1452 | 1496 |
| 2nd order DL harmonics frequency range (MHz) | 1516 – 1606 |  | 2904 – 2992 |  |
| 3rd order DL harmonics frequency range (MHz) | 2274 - 2409 |  | 4356 - 4488 |  |

As shown in Table 5.x.3-2, the 2nd harmonic frequencies of Band 28 UL might affect the band 32 DL.

Table 5.x.3-2: UL harmonics frequency limits for CA of Band 28 and Band 32

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 |
| 32 | N/A | N/A | 1452 | 1496 | N/A | N/A | N/A | N/A |

### 5.X.3		ΔTIB,c and ΔRIB,c values

The ΔTIB,c and ΔRIB,c values are derived from existing combination CA_8-32 and CA_20-32 with the low band 8 or 20 replaced with low band 28, and are specified by the following inclusions into tables in TS 36.101.

Table 6.2.5-2: ΔTIB,c (two bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_28-32 | 28 | 0.3 |

Table 7.3.1-1A: ΔRIB,c (two bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_28-32 | 28 | 0 |

### 5.X.4	REFSENS requirements

REFSENS requirements are needed since second UL harmonic of band 28 is affecting band 32 DL. Similar values as from CA_3-42 which also has 2nd harmonic issues, and need to be specified by the following inclusions into tables in TS 36.101

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_28A-32A9,10 | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | TDD |
| CA_28A-32A11 | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 | TDD |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_13A-48A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A and CA_3A-28A-41A-42A. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |  |

##### ---End of changes---

# Reference

[1]	RP-181930, “Revised WID: Rel16 LTE inter-band CA for 2 bands DL with 1 band UL”, Qualcomm