3GPP TSG-RAN WG4 Meeting #88bis	R4-1813883

Chengdu, China, 8th - 12th Oct, 2018

Agenda item:	7.9.2.3

Source: 	ZTE Corporation .

Title: 	TP to TS 38.141-2: FR2 test model(Section 4.9.3)

Document for:	Approval

Introduction

This contribution is a text proposal to draft TS 38.141-2 [1] to complete FR2 test model.

Text Proposal

--------------Start of text proposal-------------

4.9.3	Test models

4.9.3.1	General

The NR test models described in TS 38.141-1 is also applicable for 1-O. The following sections will describe the NR test models needed for 2-O.

4.9.3.2	NR FR2 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR-FR2-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier space

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-2 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For NR FR2 TDD, test models are derived based on the uplink/downlink configuration as showing in the table4.9.3.2-1 using information element TDD-UL-DL-ConfigCommon..

[If the number of downlink symbols in a special slot is less than 7 symbols, no measurements will be performed on that special slot.]

Table4.9.3.2-1: Configurations of TDD for BS type 2-O  test models

| Field name | 60 kHz SCS | 120 kHz SCS |
| --- | --- | --- |
| referenceSubcarrierSpacing |  |  |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25ms | 1.25ms |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

Common physical channel parameters for all NR FR2 test models are specified in the following tables: table 4.9.3.2-2 for PDCCH, table 4.9.3.2-3 for PDSCH. Specific physical channel parameters for NR FR2 test models are described in sections 4.9.3.2.1 to 4.9.3.2.3.

Table 4.9.3.2-2: Common physical channel parameters for PDCCH

| Parameter |  |
| --- | --- |
| # of symbols used for control channel | 1 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in the each symbol | NRB – 6 |
| DM-RS configuration and density | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |
| ratio of PDCCH EPRE to DM-RS EPRS | 0 dB |

Table 4.9.3.2-3: Common physical channel parameters for PDSCH

| Parameter |  |
| --- | --- |
| DM-RS configuration and density | Type 1, comb 2 (every other subcarrier) in symbol 3 |
| ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS 4 KPT-RS 2 |
| ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

4.9.3.2.1	NR FR2 test model 1.1 (NR-FR2-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

Table 4.9.3.2.1-1: Physical channel parameters of NR-FR2-TM1.1

| Parameter |  |
| --- | --- |
| # of QPSK PDSCH PRBs | NRB |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |

4.9.3.2.2	NR FR2 test model 2 (NR-FR2-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Table 4.9.3.2.2-1: Physical channel parameters of NR-FR2-TM 2

| Parameter |  |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1-1; n=0,…,10×2μ-23-1. |

##### 4.9.3.2.3	NR FR2 test model 3.1 (NR-FR2-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation(at max power)

Note: if power back off is declared for 64QAM, EVM shall be tested on highest modulation without power back off

Physical channel parameters are defined in table 4.9.3.2.1-1 with all QPSK PDSCH PRBs replaced by 64QAM PDSCH PRBs.