3GPP TSG-RAN WG4 Meeting #88b	R4-1814042

# Chengdu, China, October 8 – 12, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: CSI-RS configuration for NR RRM test (Section A.3.1.x) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - CSI-RS RMC is needed for multiple test cases but is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add CSI-RS RMC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS/TR ... CR ...TS38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF 1st  CHANGES----------------------------

A.3.1.x	CSI-RS

A.3.1.x.1	FDD

Table A.3.1.x.1-1: CSI-RS Reference Measurement Channels for SCS=15kHz

| Parameter |  | Unit | value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | CSI-RS.1.1 FDD |  |  |  |
| Bandwidth |  | MHz | 10 |  |  |  |
| SCS |  | kHz | 15 |  |  |  |
| NZP CSI-RS | resourceType |  | periodic |  |  |  |
|  | Number of ports |  | 2 |  |  |  |
|  | CDM Type |  | noCDM |  |  |  |
|  | Density |  | 1 |  |  |  |
|  | firstOFDMSymbolInTimeDomain |  | 5 |  |  |  |
|  | frequencyDomainAllocation |  | 000001 |  |  |  |
|  | period | slot | 10 |  |  |  |
|  | offset | slot | 1 |  |  |  |
|  | EPRE ratio to SSS | dB | 0 |  |  |  |
| CSI-IM | subcarrierLocation-p0 |  | s0 |  |  |  |
|  | symbolLocation-p0 |  | 6 |  |  |  |
|  | period | slot | 10 |  |  |  |
|  | offset | slot | 1 |  |  |  |
| ReportConfigType |  |  | periodic |  |  |  |
| CQI-table |  |  | Table 1 |  |  |  |
| reportQuantity |  |  | TBD |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  | configured |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| CodebookType |  |  | Type1 single panel |  |  |  |
| CodebookMode |  |  | 1 |  |  |  |
| TypeI-SinglePanel-2Tx-CodebookSubsetRestriction |  |  | 111111 |  |  |  |
| TypeI-SinglePanel-RI-Restriction |  |  | 11111111 |  |  |  |
| Physical channel for CSI report |  |  | PUSCH |  |  |  |
| Reporting interval |  | ms | 10 |  |  |  |
| CQI/RI/PMI delay |  | ms | TBD |  |  |  |

A.3.1.x.2	TDD

Table A.3.1.x.2-1: CSI-RS Reference Measurement Channels for SCS=15kHz

| Parameter |  | Unit | value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | CSI-RS.1.1 TDD |  |  |  |
| Bandwidth |  | MHz | 10 |  |  |  |
| SCS |  | kHz | 15 |  |  |  |
| NZP CSI-RS | resourceType |  | periodic |  |  |  |
|  | Number of ports |  | 2 |  |  |  |
|  | CDM Type |  | noCDM |  |  |  |
|  | Density |  | 1 |  |  |  |
|  | firstOFDMSymbolInTimeDomain |  | 5 |  |  |  |
|  | frequencyDomainAllocation |  | 000001 |  |  |  |
|  | period | slot | 10 |  |  |  |
|  | offset | slot | 1 |  |  |  |
|  | EPRE ratio to SSS | dB | 0 |  |  |  |
| CSI-IM | subcarrierLocation-p0 |  | s0 |  |  |  |
|  | symbolLocation-p0 |  | 6 |  |  |  |
|  | period | slot | 10 |  |  |  |
|  | offset | slot | 1 |  |  |  |
| ReportConfigType |  |  | periodic |  |  |  |
| CQI-table |  |  | Table 1 |  |  |  |
| reportQuantity |  |  | TBD |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  | configured |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| CodebookType |  |  | Type1 single panel |  |  |  |
| CodebookMode |  |  | 1 |  |  |  |
| TypeI-SinglePanel-2Tx-CodebookSubsetRestriction |  |  | 111111 |  |  |  |
| TypeI-SinglePanel-RI-Restriction |  |  | 11111111 |  |  |  |
| Physical channel for CSI report |  |  | PUSCH |  |  |  |
| Reporting interval |  | ms | 10 |  |  |  |
| CQI/RI/PMI delay |  | ms | TBD |  |  |  |

Table A.3.1.x.2-2: CSI-RS Reference Measurement Channels for SCS=30kHz

| Parameter |  | Unit | value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | CSI-RS.2.1 TDD |  |  |  |
| Bandwidth |  | MHz | 40 |  |  |  |
| SCS |  | kHz | 30 |  |  |  |
| NZP CSI-RS | resourceType |  | periodic |  |  |  |
|  | Number of ports |  | 2 |  |  |  |
|  | CDM Type |  | noCDM |  |  |  |
|  | Density |  | 1 |  |  |  |
|  | firstOFDMSymbolInTimeDomain |  | 5 |  |  |  |
|  | frequencyDomainAllocation |  | 000001 |  |  |  |
|  | period | slot | 20 |  |  |  |
|  | offset | slot | 2 |  |  |  |
|  | EPRE ratio to SSS | dB | 0 |  |  |  |
| CSI-IM | subcarrierLocation-p0 |  | s0 |  |  |  |
|  | symbolLocation-p0 |  | 6 |  |  |  |
|  | period | slot | 20 |  |  |  |
|  | offset | slot | 2 |  |  |  |
| ReportConfigType |  |  | periodic |  |  |  |
| CQI-table |  |  | Table 1 |  |  |  |
| reportQuantity |  |  | TBD |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  | configured |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| CodebookType |  |  | Type1 single panel |  |  |  |
| CodebookMode |  |  | 1 |  |  |  |
| TypeI-SinglePanel-2Tx-CodebookSubsetRestriction |  |  | 111111 |  |  |  |
| TypeI-SinglePanel-RI-Restriction |  |  | 11111111 |  |  |  |
| Physical channel for CSI report |  |  | PUSCH |  |  |  |
| Reporting interval |  | ms | 10 |  |  |  |
| CQI/RI/PMI delay |  | ms | TBD |  |  |  |

Table A.3.1.x.2-3: CSI-RS Reference Measurement Channels for SCS=120kHz

| Parameter |  | Unit | value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | CSI-RS.3.1 TDD |  |  |  |
| Bandwidth |  | MHz | 100 |  |  |  |
| SCS |  | kHz | 120 |  |  |  |
| NZP CSI-RS | resourceType |  | periodic |  |  |  |
|  | Number of ports |  | 2 |  |  |  |
|  | CDM Type |  | noCDM |  |  |  |
|  | Density |  | 1 |  |  |  |
|  | firstOFDMSymbolInTimeDomain |  | 5 |  |  |  |
|  | frequencyDomainAllocation |  | 000001 |  |  |  |
|  | period | slot | 80 |  |  |  |
|  | offset | slot | 8 |  |  |  |
|  | EPRE ratio to SSS | dB | 0 |  |  |  |
| CSI-IM | subcarrierLocation-p0 |  | s0 |  |  |  |
|  | symbolLocation-p0 |  | 6 |  |  |  |
|  | period | slot | 80 |  |  |  |
|  | offset | slot | 8 |  |  |  |
| ReportConfigType |  |  | periodic |  |  |  |
| CQI-table |  |  | Table 1 |  |  |  |
| reportQuantity |  |  | TBD |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  | configured |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| CodebookType |  |  | Type1 single panel |  |  |  |
| CodebookMode |  |  | 1 |  |  |  |
| TypeI-SinglePanel-2Tx-CodebookSubsetRestriction |  |  | 111111 |  |  |  |
| TypeI-SinglePanel-RI-Restriction |  |  | 11111111 |  |  |  |
| Physical channel for CSI report |  |  | PUSCH |  |  |  |
| Reporting interval |  | ms | 10 |  |  |  |
| CQI/RI/PMI delay |  | ms | TBD |  |  |  |

----------------------END OF CHANGES----------------------------