3GPP TSG-RAN WG4 Meeting # 89	R4-1814425

# Spokane, US, 12th - 16th Nov, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | draftCR | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Simplification of requirements for EN-DC configuration including FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC., Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Redundant information has been captured in the spec. This requires unnecessary work when new band configurations are introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Unnecessary tables for Delta TIB, RIB and spurious emission UE co-existene including FR2 bands are omited and texts to eloborate the requirements are added instead. Missing subclauses of four to six bands for Inter-band EN-DC including both FR1 an FR2 are added to the spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Whenever new band combinaitons are introduced, unnecessary work is required and this just provokes more errors in unnecessary information in the spec. This further requires more work to corret the errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.2.4, 6.2B.4.2.5, 6.5B.3.4, 6.5B.3.5, 7.3B.3.4, 7.3B.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note that the corresponding subclauses and tables remained since band combinations including FR2 whose requirements cannot be covered by the current ones may come in the future. |  |  |  |  |  |  |  |  |

< Start of changes >

<< Unchanged sections omitted >>

6.2B.4.2.4	Inter-band EN-DC including FR2

6.2B.4.2.4.1	ΔTIB,c for EN-DC two bands

Unless otherwise stated, ΔTIB,c for E-UTRA and FR2 NR bands of inter-band EN-DC combinations defined in table 5.2B.5.1-1 is set to zero.

Table 6.2B.4.2.4.1-1: ΔTIB,c due to EN-DC(two bands)

(Void)

6.2B.4.2.4.2	ΔTIB,c for EN-DC three bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.2B.5.2-1 is the same as those for the corresponding E-UTRA CA configuration without the FR2 NR bands specified in TS 36.101 [4].

Table 6.2B.4.2.4.2-1: ΔTIB,c due to EN-DC (three bands)

(Void)

6.2B.4.2.4.3	ΔTIB,c for EN-DC four bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.2B.5.3-1 is the same as those for the corresponding E-UTRA CA configuration without the FR2 NR bands specified in TS 36.101 [4].

Table 6.2B.4.2.4.3-1: ΔTIB,c due to EN-DC(four bands)

(Void)

6.2B.4.2.4.4	ΔTIB,c for EN-DC five bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.2B.5.4-1 is the same as those for the corresponding E-UTRA CA configuration without the FR2 NR bands specified in TS 36.101 [4].

Table 6.2B.4.2.4.4-1: ΔTIB,c due to EN-DC (five bands)

(Void)

6.2B.4.2.4.5	ΔTIB,c for EN-DC six bands

(Void)

6.2B.4.2.5	Inter-band EN-DC including both FR1 an FR2

6.2B.4.2.5.1	ΔTIB,c for EN-DC three bands

Unless otherwise stated, for a certain inter-band EN-DC configuration, ΔTIB,c for constituent FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA and FR1 NR bands for inter-band EN-DC defined in table 5.2B.6.2-1 is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

Table 6.2B.4.2.5.1-1: ΔTIB,c due to EN-DC (three bands)

(Void)

6.2B.4.2.5.2	ΔTIB,c for EN-DC four bands

Unless otherwise stated, for a certain inter-band EN-DC configuration, ΔTIB,c for constituent FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA and FR1 NR bands for inter-band EN-DC defined in table 5.2B.6.3-1 is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

6.2B.4.2.5.3	ΔTIB,c for EN-DC five bands

Unless otherwise stated, for a certain inter-band EN-DC configuration, ΔTIB,c for constituent FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA and FR1 NR bands for inter-band EN-DC defined in table 5.2B.6.4-1  is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

6.2B.4.2.5.4	ΔTIB,c for EN-DC six bands

Unless otherwise stated, for a certain inter-band EN-DC configuration, ΔTIB,c for constituent FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA and FR1 NR bands for inter-band EN-DC defined in table 5.2B.6.5-1 is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

<Uncecessary sections are omitted>

<Next change>

6.5B.3.4	Inter-band EN-DC including FR2

< OTA requirements >

< Editor’s note: Chapter numbers to be updated >

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4], sub-clause 6.5.3.1 of [2] and [3] apply for each component carrier.

6.5B.3.4.1	Spurious emission band UE co-existence

< Editor’s note: Chapter numbers to be updated >

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. Unless otherwise stated, for a certain inter-band EN-DC configuration defined in table 5.2B.5.1-1, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. And spurious emission band UE co-existence specified in TS 36.101 [4] is applied to the constituent E-UTRA bands for the EN-DC configuration.

Table 6.5B.3.4.1-1: Requirements

(Void)

6.5B.3.5	Inter-band EN-DC including both FR1 and FR2

<OTA requirements >

< Editor’s note: Chapter numbers to be updated >

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4], sub-clause 6.5.3.1 of [2] and [3] apply for each component carrier.

6.5B.3.5.1	Spurious emission band UE co-existence

< Editor’s note: Chapter numbers to be updated >

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. Unless otherwise stated, for a certain inter-band EN-DC configuration defined in table 5.2B.6.2-1, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. And spurious emission band UE co-existence for constituent E-UTRA and FR1 NR bands for the inter-band EN-DC is the same as those for the corresponding EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

Table 6.5B.3.5.1-1: Requirements

(Void)

<Uncecessary sections are omitted>

<Next change>

7.3B.3.4	Inter-band EN-DC including FR2

7.3B.3.4.1	ΔRIB,c for EN-DC in two bands

Unless otherwise stated, ΔRIB,c for E-UTRA and FR2 NR bands of inter-band EN-DC combinations defined in table 5.2B.5.1-1 is set to zero.

Table 7.3B.3.4.1-1: ΔRIB,c due to EN-DC(two bands)

(Void)

7.3B.3.4.2	ΔRIB,c for EN-DC three bands

Unless otherwise stated, ΔRIB,c for FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.2B.5.2-1 is the same as those for the corresponding E-UTRA CA configuration without the FR2 NR bands specified in TS 36.101 [4].

Table 7.3B.3.4.2-1: ΔRIB,c due to EN-DC (three bands)

(Void)

7.3B.3.4.3	ΔRIB,c for EN-DC four bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.2B.5.3-1 is the same as those for the corresponding E-UTRA CA configuration without the FR2 NR bands specified in TS 36.101 [4].

Table 7.3B.3.4.3-1: ΔRIB,c due to EN-DC (four bands)

(Void)

7.3B.3.4.4	ΔRIB,c for EN-DC five bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.2B.5.4-1 is the same as those for the corresponding E-UTRA CA configuration without the FR2 NR bands specified in TS 36.101 [4].

Table 7.3B.3.4.4-1: ΔRIB,c due to EN-DC (five bands)

(Void)

7.3B.3.4.5	ΔRIB,c for EN-DC six bands

(Void)

7.3B.3.5	Inter-band EN-DC including both FR1 and FR2

7.3B.3.5.2	ΔRIB,c for EN-DC three bands

Unless otherwise stated, for a certain inter-band EN-DC configuration, ΔRIB,c for constituent FR2 NR bands is set to zero and ΔRIB,c for constituent E-UTRA and FR1 NR bands for inter-band EN-DC defined in table 5.2B.6.2-1 is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

Table 7.3B.3.5.2-1: ΔRIB,c due to EN-DC (three bands)

(Void)

7.3B.3.5.3	ΔRIB,c for EN-DC four bands

Unless otherwise stated, for a certain inter-band EN-DC configuration, ΔRIB,c for constituent FR2 NR bands is set to zero and ΔRIB,c for constituent E-UTRA and FR1 NR bands for inter-band EN-DC defined in table 5.2B.6.3-1 is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

7.3B.3.5.4	ΔRIB,c for EN-DC five bands

Unless otherwise stated, for a certain inter-band EN-DC configuration, ΔRIB,c for constituent FR2 NR bands is set to zero and ΔRIB,c for constituent E-UTRA and FR1 NR bands for inter-band EN-DC defined in table 5.2B.6.4-1 is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

7.3B.3.5.5	ΔRIB,c for EN-DC six bands

Unless otherwise stated, for a certain inter-band EN-DC configuration, ΔRIB,c for constituent FR2 NR bands is set to zero and ΔRIB,c for constituent E-UTRA and FR1 NR bands for inter-band EN-DC defined in table 5.2B.6.5-1 is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

< END of changes >