3GPP TSG-RAN WG4 Meeting #89	R4-1814429

Spokane, USA, 12th - 16th Nov, 2018

Agenda item:	7.9.3.2

Source: 	ZTE Corporation .

Title: 	TP to TS 38.141-1:  Transmit ON/OFF power (Section 6.4)

Document for:	Approval

Introduction

This contribution is a text proposal to draft TS 38.141-1 [1] to complete transmit ON/OFF power conformance testing based on the contribution [2].

- Reference

- Draft TS 38.141-1 v1.1.0_afterChengdu, Huawei.

- R4-1814516, Timing alignment for intra-band contiguous CA, ZTE

Text Proposal

--------------Start of text proposal-------------

6.4	Transmit ON/OFF power

6.4.1	Transmitter OFF power

6.4.1.1	Definition and applicability

Transmit OFF power requirements apply only to TDD operation of NR BS.

Transmitter OFF power is defined as the mean power measured over 70/N us filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For multi-band connectors, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band connectors supporting transmission in multiple operating bands, the requirement is applicable per supported operating band.

For BS supporting intra-band contiguous CA, the transmitter OFF power is defined as the mean power measured over 70/N us filtered with a square filter of bandwidth equal to the Aggregated BS Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2 during the transmitter OFF period. N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth.

6.4.1.2	Minimum requirement

The minimum requirement for BS type 1-C is in TS 38.104 [2], subclause 6.4.1.2.

The minimum requirement for BS type 1-H is in TS 38.104 [2], subclause 6.4.1.3.

6.4.1.3	Test purpose

The purpose of this test is to verify the transmitter OFF power is within the limits of the minimum requirements.

6.4.1.4	Method of test

Requirement is tested together with transmitter transient period, as described in subclause 6.4.2.4.

6.4.1.5	Test requirements

The conformance testing of transmit OFF power is included in the conformance testing of transmitter transient period; therefore, see subclause 6.4.2.5 for test requirements.

6.4.2	Transmitter transient period

6.4.2.1	Definition and applicability

Transmitter transient period requirements apply only to TDD operation of NR BS.

The transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.4.2.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at each antenna connector or TAB connector supporting transmission in the operating band.

6.4.2.2	Minimum requirement

The minimum requirement for BS type 1-C and BS type 1-H is in TS 38.104 [2], subclause 6.4.2.2.

6.4.2.3	Test purpose

The purpose of this test is to verify the transmitter transient periods are within the limits of the minimum requirements.

6.4.2.4	Method of test

6.4.2.4.1	Initial conditions

Test environment:

-	normal; see annex B.2.

RF channels to be tested for single carrier:

-	M; see subclause 4.9.1.

RF bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

6.4.2.4.2	Procedure

The minimum requirement is applied to all antenna connectors or TAB connectors, they may be tested one at a time or multiple antenna connectors or TAB connectors may be tested in parallel as shown in annex D.1.1 for BS type 1-C or in annex D.3.1 for BS type 1-H. Whichever method is used the procedure is repeated until all antenna connectors or TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect antenna connector or TAB connector to measurement equipment as shown in annex D.1.1 for BS type 1-C or in annex D.3.1 for BS type 1-H. All antenna connectors or TAB connectors not under test shall be terminated.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

2)	For a Set each antenna connector or TAB connector to output according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.9.2. For single carrier set the antenna connector or TAB connector to transmit at manufacturers declared rated carrier output power per antenna connector or TAB connector (Prated,c,AC, or PRated,c,TABC, D.25).

3)	Measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the antenna connector or TAB connector centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period – 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

4)	For an antenna connector or TAB connector supporting contiguous CA, measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period – 10 μs. N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth.

In addition, for multi-band connector(s), the following steps shall apply:

5)	For multi-band connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.4.2.5	Test requirements

The measured mean power spectral density according to subclause 6.4.2.4.2 shall be less than -83 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The measured mean power spectral density according to subclause 6.4.2.4.2 shall be less than -82.5 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band connector, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

--------------End of text proposal-------------