3GPP TSG-RAN WG4 Meeting #89 	R4-1814895

# Spokane, USA, 12th – 16th Nov., 2018

Agenda Item:       7.11.3.5

Source: 	MediaTek Inc.

Title: 	On the autonomous gap for inter-RAT RSTD

Document for:	Discussion

- Introduction

In RAN4-88b, the autonomous gap for RSTD was discussed [1][2][3]. However, required time for LTE cell search and corresponding interruption number are not finalized. In the approved LS [4], the nr-LTE-fineTiming-Offset-r15 is optionally provided when nr-LTE-SFN-Offset is provided and shall not be provided when the nr-LTE-SFN-Offset is not present. Because the resolution of nr-LTE-fineTiming-Offset-r15 is 0.5ms, we also discuss this issues and provide views.

- Discussion

In the section 9.4.4.1.1 of TS 38.133, the UE may be using autonomous gaps in FR1 to acquire the necessary timing in following two conditions:

- Condition1: when the UE is not aware of the SFN of at least one LTE cell in the OTDOA assistance data or

- Condition2: when the UE is not aware of the subframe timing difference between the NR serving cell and the OTDOA assistance data reference cell

For condition2, the definition of not knowing the subframe timing difference between the NR serving cell and the OTDOA assistance data reference cell is not well-defined. According to the approved RAN2 LS [4], the resolution of nr-LTE-fineTiming-Offset-r15 is 0.5ms. Even with both nr-LTE-SFN-Offset and nr-LTE-fineTiming-Offset-r15, the timing ambiguity of reference cell is still in the range of (-0.25ms, +0.25ms).  Under such range of ambiguity, there is no clear definition on whether the UE is aware of the subframe timing difference between the NR serving cell and the OTDOA assistance data reference cell or not. Moreover, to handle reference cell timing ambiguity, UE shall pay additional complexity on RSTD.

Observation 1: Even with both nr-LTE-SFN-Offset and nr-LTE-fineTiming-Offset-r15, the timing ambiguity of reference cell is still in the range of (-0.25ms, +0.25ms).

Proposal 1: When configured with both nr-LTE-SFN-Offset and nr-LTE-fineTiming-Offset-r15, clarification is still needed on whether is aware of the subframe timing difference between the NR serving cell and the OTDOA assistance data reference cell or not.

According to [4], there are three possible scenarios.

|  | Scenario 1 | Scenario 2 | Scenario 3 |
| --- | --- | --- | --- |
| High layer signalling | Both nr-LTE-SFN-Offset and nr-LTE-fineTiming-Offset-r15 | Only nr-LTE-SFN-Offset | None |
| Need LTE cell search | FFS | Yes | Yes |
| Need LTE MIB decoding | N | N | Yes |

To acquire accurate reference cell timing by LTE cell search, at least 6 LTE cell search attempts are needed. For each LTE cell search attempt, the autonomous gap length is 6ms and autonomous gap repetition period is 40ms.

Proposal 2: To acquire accurate reference cell timing by LTE cell search, at least 6 LTE cell search attempts are needed. For each LTE cell search attempt, the autonomous gap length is 6ms and autonomous gap repetition period is 40ms.

3	Summary

Based on the discussion in section 2, we have the following observation and proposals:

Proposal 1: When configured with both nr-LTE-SFN-Offset and nr-LTE-fineTiming-Offset-r15, clarification is still needed on whether is aware of the subframe timing difference between the NR serving cell and the OTDOA assistance data reference cell or not.

Observation 1: Even with both nr-LTE-SFN-Offset and nr-LTE-fineTiming-Offset-r15, the timing ambiguity of reference cell is still in the range of (-0.25ms, +0.25ms).

Proposal 2: To acquire accurate reference cell timing by LTE cell search, at least 6 LTE cell search attempts are needed. For each LTE cell search attempt, the autonomous gap length is 6ms and autonomous gap repetition period is 40ms.

4	Reference

[1], R4-1812152, ``Discussion on inter-RAT RSTD measurement, ‘’ Intel

[2], R4-1812887, ``On SFN acquisition requirements for E-UTRA RSTD measurements,’’ Ericsson

[3], R4-1814202,	``SFN acquisition requirements for E-UTRA RSTD measurements,’’ Ericsson

[4], R2-1816059, `` LS on SFN offset for OTDOA’’