3GPP TSG-RAN WG4 Meeting # 89	R4-1825597

# Spokane, USA, 12 November, 2018 - 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to FR2 SS-RSRP measurement accuracy test for SA NR |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  |  |  | Date: |  | 2018/02/11 |
| Category: | F |  |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | BPW configuration does not refer to common section. DRX configuration is missed. Add OTA setup according to agreement in RP-182149 with an editor note that it may be revised in 1H19 |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing parameters are added. AoA setup added in table assuming 1AoA in peak beam direction with an editor’s note “The use of dual AoA setup in this test is under discussion, and the OTA setup may be revised. It is expected to address this aspect by RAN#84” |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test is incomplete |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  |  | Test specifications |  |  | TS/TR TBD... CR ... |  |  |  |
| (show related CRs) |  |  | X |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |

## A.7.7.1	SS-RSRP

### A.7.7.1.1	Intra frequency case

#### A.7.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 10.1.2.1.1and 10.1.2.1.1 for intra-frequency measurements.

#### A.7.7.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.7.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra frequency measurements are tested by using the parameters in Table A.7.7.1.1.2-2 and A.7.7.1.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.7.7.1.1.2-1: SS-RSRP  Intra frequency SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |

Table A.7.7.1.1.2-2: SS-RSRP  Intra frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1 |  |  |  |  |  |
| DRX Cycle | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SSB configuration |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
|  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Propagation conditions |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.7.7.1.1.2-3: SS-RSRP  Intra frequency OTA related test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  |  | According to section A.3.8.1 |  | According to section A.3.8.1 |  | According to section A.3.8.1 |  |
| Note1 | NR_TDD_FR2_A | dBm/15kHzNote4 | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| Note1 | NR_TDD_FR2_A | dBm/SCSNote3 | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note4 | TBD | TBD | TBD | TBD | TBD | TBD |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD | TBD |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone |  |  |  |  |  |  |  |  |

Editor’s note: The use of dual AoA setup in this test is under discussion, and the OTA setup may be revised. It is expected to address this aspect by RAN#84

#### A.7.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in sections 10.1.3.1.1 and relative accuracy requirements in section 10.1.3.1.2.