3GPP TSG-RAN WG4 Meeting #89	R4-1815711

Spokane, Washington, USA 12 – 16 November 2018

Source:	Ericsson

Title:	Discussion and FR1 simulation results for NR PUSCH demodulation requirements

Agenda item:	7.13.2.2.1

Document for:	Discussion

# Introduction

In this contribution, prioritized FR1 simulation results for both ideal and with impairments based on agreed assumptions in [1] are presented herein.

# Simulation

## Simulation assumptions

| Parameters | Values |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1, 2 |
| Number of Rx | 2 (and 4, 8) |
| Number of layers | 1, 2 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| Number of DMRS symbols | 1, 1+1 |
| Number of symbols | 14 |
| Start symbol index | 0 |
| Time domain resource allocation type | type A |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 2, 16, 20 (20 only for 1 Tx) |
| Carrier frequency (GHz) | 4 |
| Propagation channel | MCS 2: TDL-B 100 ns 400 Hz MCS 16: TDL-C 300 ns 100 Hz MCS 20: TDL-A 30 ns 10 Hz |
| SCS and BW | 15kHz: 10 MHz, 5 MHz, 20 MHz 30kHz: 40 MHz, 10 MHz, 20 MHz, 100 MHz |
| PTRS | Not configured |
| Timing offset / Frequency offset | 0 |
| Code block group / Frequency hopping / Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| Testing metric | SNR @70% of maximum throughput |

Table 1. FR1 Simulation assumptions for CP-OFDM.

Table 2. FR2 simulation assumptions for DFT-OFDM.

| Parameter | value |
| --- | --- |
| Transform precoding | enabled |
| Number of Tx | 1 |
| Number of Rx | 2 (and 4, 8) |
| DMRS type | type 1 |
| Number of DMRS symbols | [1] and (1+1) |
| Number of symbols | 14 |
| Start symbol index | 0 |
| Time domain resource allocation type | type A |
| Frequency domain resource | 15 kHz: 25 PRBs 30 kHz: 24 PRBs |
| MCS index | 2 |
| Carrier frequency (GHz) | 4 |
| Propagation channel | TDL-B 100 ns 400 Hz |
| SCS and BW | 15 kHz: 5 MHz 30 kHz: 10 MHz |
| PTRS | Not configured |
| Timing / frequency offset | 0 |
| Code block group / frequency hopping / limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| Testing metric | SNR @70% of maximum throughput |

## Simulation results

In this sub-section, the simulation results are presented and are based on ideal conditions and with impairments.

### Ideal simulation results

Table 3. FR1 - ideal simulation results for CP-OFDM with 1Tx2Rx and 2Tx2Rx.

| Parameters |  | SNR [dB] at 70% of maximum throughput |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number of transmit antennas |  | 1 |  |  | 2 |  |
| Number of receiver antennas |  | 2 |  |  |  |  |
| MCS |  | 2 | 16 | 20 | 2 | 16 |
| SCS = 15 kHz |  |  |  |  |  |  |
| 10 MHz | DMRS: Front-loaded only | -1,1 | 10,7 | 9,9 | --* | --* |
|  | DMRS: 1+1 | -5,1 | 8,1 | 9,6 | -0,9 | 15,8 |
| 5 MHz | DMRS: Front-loaded only | -0,2 | 9,9 | 9,8 | --* | --* |
|  | DMRS: 1+1 | -4,8 | 7,9 | 9,5 | -1,3 | 15,6 |
| 20 MHz | DMRS: Front-loaded only | 28,3 | 11,2 | 10,0 | --* | --* |
|  | DMRS: 1+1 | -4,5 | 7,9 | 9,5 | -0,7 | 15,6 |
| SCS = 30 kHz |  |  |  |  |  |  |
| 40 MHz | DMRS: Front-loaded only | -3,6 | 8,6 | 10,0 | 1,8 | 21,5 |
|  | DMRS: 1+1 | -5,0 | 7,9 | 9,5 | -1,1 | 15,8 |
| 10 MHz | DMRS: Front-loaded only | -4,0 | 8,4 | 9,5 | -0,3 | 19,7 |
|  | DMRS: 1+1 | -4,9 | 8,1 | 9,5 | -1,4 | 15,9 |
| 20 MHz | DMRS: Front-loaded only | -4,3 | 8,6 | 9,9 | 0,7 | 20,5 |
|  | DMRS: 1+1 | -5,2 | 8,1 | 9,6 | -1,2 | 15,8 |
| 100 MHz | DMRS: Front-loaded only | -2,9 | 8,7 | 10,8 | 2,8 | --* |
|  | DMRS: 1+1 | -5,1 | 8,0 | 10,4 | -1,2 | 16,3 |
| Note: *ill conditioned channel conditions, 70% of maximum throughput cannot be reached. |  |  |  |  |  |  |

Table 4. FR1 - ideal simulation results for CP-OFDM with 1Tx4Rx and 1Tx8Rx.

| Parameters |  | SNR [dB] at 70% of maximum throughput |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of transmit antennas |  | 1 |  |  |  |  |  |  |  |
| Number of receiver antennas |  | 4 |  |  | 8 |  |  |  |  |
| MCS |  | 2 | 16 | 20 | 2 | 16 |  | 20 |  |
| SCS = 15 kHz |  |  |  |  |  |  |  |  |  |
| 10 MHz | DMRS: Front-loaded only | -5,1 | 5,5 | 6,4 | -8,7 |  | 1,5 |  | 3,1 |  |
|  | DMRS: 1+1 | -8,3 | 4,2 | 6,0 | -11,1 |  | 0,8 |  | 3,0 |  |
| SCS = 30 kHz |  |  |  |  |  |  |  |  |  |
| 40 MHz | DMRS: Front-loaded only | -6,9 | 4,5 | 6,5 | -10,0 |  | 0,75 |  | 3,2 |  |
|  | DMRS: 1+1 | -8,1 | 4,0 | 6,0 | -10,9 |  | 0,6 |  | 2,9 |  |

Table 5. FR1 - ideal simulation results for DFT-S-OFDM with 1Tx2Rx, 1Tx4Rx and 1Tx8Rx.

| Parameters |  | SNR [dB] at 70% of maximum throughput |  |  |
| --- | --- | --- | --- | --- |
| Number of transmit antennas |  | 1 |  |  |
| Number of receive antennas |  | 2 | 4 | 8 |
| MCS |  | 2 | 2 | 2 |
| SCS = 15 kHz |  |  |  |  |
| 5 MHz | DMRS: Front-loaded only | -0,1 | -4,8 | -8,5 |
|  | DMRS: 1+1 | -4,7 | -8,0 | -10,9 |
| SCS = 30 kHz |  |  |  |  |
| 10 MHz | DMRS: Front-loaded only | -3,9 | -7,1 | -10,2 |
|  | DMRS: 1+1 | -4,75 | -7,9 | -10,7 |

Observation 1: From the simulation results, it clearly shows that with only one DMRS, it is unreliable (i.e. very bad performance or 70% of maximum throughput cannot be reached) and not suitable for most of the test cases with the chosen channel conditions.

Proposal 1: Based on observation 1, suggest removing test cases with front-loaded only DMRS pattern for the current specified channel conditions in the specification.

Observation 2: Application rule holds for DFT-S-OFDM with (1+1) DMRS pattern and the specified bandwidth for FR1. However, the application rule fails for DFT-S-OFDM with front-loaded only DMRS due to the reason mentioned in observation 1.

### Simulation results with impairments

Table 6. FR1 - simulation results with impairments for CP-OFDM with 1Tx2Rx and 2Tx2Rx.

| Parameters |  | SNR [dB] at 70% of maximum throughput |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number of transmit antennas |  | 1 |  |  | 2 |  |
| Number of receiver antennas |  | 2 |  |  |  |  |
| MCS |  | 2 | 16 | 20 | 2 | 16 |
| SCS = 15 kHz |  |  |  |  |  |  |
| 10 MHz | DMRS: Front-loaded only | 1,4 | 12,9 | 12,0 | --* | --* |
|  | DMRS: 1+1 | -2,6 | 10,5 | 11,8 | 1,2 | 18,2 |
| 5 MHz | DMRS: Front-loaded only | 2,2 | 12,0 | 11,8 | --* | --* |
|  | DMRS: 1+1 | -2,8 | 9,9 | 11,9 | 0,8 | 17,7 |
| 20 MHz | DMRS: Front-loaded only | 30,8 | 13,7 | 12,2 | --* | --* |
|  | DMRS: 1+1 | -2,3 | 9,9 | 11,7 | 1,6 | 17,6 |
| SCS = 30 kHz |  |  |  |  |  |  |
| 40 MHz | DMRS: Front-loaded only | -1,1 | 11,1 | 12,0 | 3,8 | 23,6 |
|  | DMRS: 1+1 | -2,6 | 10,2 | 11,9 | 0,9 | 18,1 |
| 10 MHz | DMRS: Front-loaded only | -1,6 | 10,5 | 11,7 | 2,2 | 22,0 |
|  | DMRS: 1+1 | -2,5 | 10,3 | 11,8 | 1,0 | 18,1 |
| 20 MHz | DMRS: Front-loaded only | -1,9 | 10,7 | 12,2 | 2,9 | 22,8 |
|  | DMRS: 1+1 | -2,8 | 10,4 | 11,7 | 1,0 | 18,1 |
| 100 MHz | DMRS: Front-loaded only | -0,9 | 11,2 | 13,1 | 5,2 | --* |
|  | DMRS: 1+1 | -2,9 | 10,2 | 12,5 | 1,2 | 18,5 |
| Note: *ill conditioned channel conditions, 70% of maximum throughput cannot be reached. |  |  |  |  |  |  |

Table 7. FR1 - simulation results with impairments for CP-OFDM with 1Tx4Rx and 1Tx8Rx.

| Parameters |  | SNR [dB] at 70% of maximum throughput |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of transmit antennas |  | 1 |  |  |  |  |  |
| Number of receiver antennas |  | 4 |  |  | 8 |  |  |
| MCS |  | 2 | 16 | 20 | 2 | 16 | 20 |
| SCS = 15 kHz |  |  |  |  |  |  |  |
| 10 MHz | DMRS: Front-loaded only | -2,9 | 7,6 | 8,6 | -6,7 | 3,9 | 5,1 |
|  | DMRS: 1+1 | -6,2 | 6,5 | 8,2 | -8,7 | 3,2 | 5,2 |
| SCS = 30 kHz |  |  |  |  |  |  |  |
| 40 MHz | DMRS: Front-loaded only | -4,5 | 6,8 | 8,6 | -7,9 | 3,0 | 5,3 |
|  | DMRS: 1+1 | -5,8 | 6,5 | 8,4 | -8,5 | 3,1 | 5,0 |

Table 8. Table 3. FR1 - simulation results with impairments for DFT-S-OFDM with 1Tx2Rx, 1Tx4Rx and 1Tx8Rx.

| Parameters |  | SNR [dB] at 70% of maximum throughput |  |  |
| --- | --- | --- | --- | --- |
| Number of transmit antennas |  | 1 |  |  |
| Number of receive antennas |  | 2 | 4 | 8 |
| MCS |  | 2 | 2 | 2 |
| SCS = 15 kHz |  |  |  |  |
| 5 MHz | DMRS: Front-loaded only | 2,1 | -2,8 | -6,3 |
|  | DMRS: 1+1 | -2,3 | -5,8 | -8,4 |
| SCS = 30 kHz |  |  |  |  |
| 10 MHz | DMRS: Front-loaded only | -1,8 | -5,1 | -7,7 |
|  | DMRS: 1+1 | -2,4 | -5,4 | -8,4 |

# Remaining open issues

PUSCH mapping type B for FR1

As there has been many meetings regarding the test requirements for PUSCH mapping type B for FR1 and no agreement has been reached. Due to the nature of the requirements for PUSCH mapping type B is similar to that of the requirements of different format for other channels, in order to progress with the discuss on this topic, we would like to treat the requirement for PUSCH mapping type B for FR1 as part of the applicability rule and further discussed in [2].

UCI multiplexing

UCI multiplexing would affect different part of decoder depending on payload sizes, thus test cases should be defined to test different decoders. However, further study (for the next meeting) is required to formulate appropriate test cases and test metrics that could cover necessary scenarios, this should not hinder the time line for this requirement as in the WF [1] it is agreed to only discuss UCI multiplexing test cases after the November meeting.

Proposal 2:  Should define test cases that could test different decoders, and to further study (for the next meeting) to formulate appropriate test cases and test metrics.

# Conclusion

Simulation results based on ideal conditions and with impairments are presented in Section 2.

Observation 1: From the simulation results, it clearly shows that with only one DMRS, it is unreliable and not suitable for most of the test cases with the chosen channel conditions.

Observation 2: Application rule holds for DFT-S-OFDM with (1+1) DMRS pattern and the specified bandwidth for FR1. However, the application rule fails for DFT-S-OFDM with front-loaded only DMRS due to the reason mentioned in observation 1.

Proposal 1: Based on observation 1, suggest removing test cases with front-loaded only DMRS pattern for the current specified channel conditions in the specification for FR1.

Proposal 2: Should define test cases that could test different decoders, and to further study (for the next meeting) to formulate appropriate test cases and test metrics.

# References

R4-1813942, “WF for NR PUSCH demodulation”, Nokia, Nokia Shanghai Bell.

R4-1815365, “BS demod: Further considerations on applicability”, Ericsson.