3GPP TSG-RAN WG4 Meeting #89	R4-1815796

# Spokane, USA, 12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5300 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to add LTE Intra-band band combinations in TS 36.101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2018-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding approved LTE Intra-band combinations from RAN4 88bis as defined in R4-1812413 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the following LTE Intra-band CA combinations: CA_42A-42A_BCS1 CA_42A-42C_BCS1 CA_42A-42C_UL_42C_BCS1 CA_42C-42C_BCS1 CA_42C-42C_UL_42C_BCS1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for Intra-band CA band combinations are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 5.6A.1-3: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] |  | Channel bandwidths for carrier [MHz] |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_1A-1A | - | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 0 |
| CA_2A-2A | - | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 0 |
| CA_3A-3A | - | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 0 |
|  |  | 5, 10 |  | 5, 10, 15, 20 |  |  |  |  |  | 30 | 1 |
|  |  | 5 |  | 3 |  |  |  |  |  | 10 | 2 |
|  |  | 3, 5 |  | 5 |  |  |  |  |  |  |  |
| CA_4A-4A | CA_4A-4A | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 0 |
|  |  | 5, 10 |  | 5, 10 |  |  |  |  |  | 20 | 1 |
| CA_5A-5A | - | 5,10 |  | 5,10 |  |  |  |  |  | 20 | 0 |
|  |  | 3 |  | 5 |  |  |  |  |  | 8 | 1 |
| CA_7A-7A | - | 5 |  | 15 |  |  |  |  |  | 40 | 0 |
|  |  | 10 |  | 10, 15 |  |  |  |  |  |  |  |
|  |  | 15 |  | 15, 20 |  |  |  |  |  |  |  |
|  |  | 20 |  | 20 |  |  |  |  |  |  |  |
|  |  | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 |  | 5, 10 |  |  |  |  |  | 30 | 2 |
|  |  | 10, 15, 20 |  | 10, 15, 20 |  |  |  |  |  | 40 | 3 |
| CA_12A-12A | - | 5 |  | 5 |  |  |  |  |  | 10 | 0 |
| CA_23A-23A | - | 5 |  | 10 |  |  |  |  |  | 15 | 0 |
| CA_25A-25A | - | 5, 10 |  | 5, 10 |  |  |  |  |  | 20 | 0 |
|  |  | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 1 |
| CA_40A-40A | - | 10, 20 |  | 10, 20 |  |  |  |  |  | 40 | 0 |
|  |  | 10,15,20 |  | 10,15,20 |  |  |  |  |  | 40 | 1 |
| CA_40A-40C | CA_40C | 20 |  | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | 20 |  |  |  |  |
| CA_40C-40C | CA_40C | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  | 80 | 0 |
| CA_41A-41A | - | 10, 15, 20 |  | 10, 15, 20 |  |  |  |  |  | 40 | 0 |
|  |  | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 1 |
| CA_41A-41C | CA_41C | 5, 10, 15, 20 |  | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
| CA_41A-41D | CA_41C | 5, 10, 15, 20 |  | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |
| CA_41C-41C | CA_41C | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 80 | 0 |
| CA_41C-41D | CA_41C | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 100 | 0 |
|  |  | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |
| CA_42A-42A | - | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 0 |
|  |  | 10, 15, 20 |  | 10, 15, 20 |  |  |  |  |  | 40 | 1 |
| CA_42A-42C | CA_42C | 5, 10, 15, 20 |  | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
|  |  | 10, 15, 20 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  | 60 | 1 |
|  |  | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | 10, 15, 20 |  |  |  |  |
| CA_42A-42D | - | 5, 10, 15, 20 |  | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |
| CA_42C-42C | CA_42C | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 80 | 0 |
|  |  | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  | 80 | 1 |
| CA_43A-43A | - | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 0 |
| CA_46A-46A2 | - | 20 |  | 20 |  |  |  |  |  | 40 | 0 |
| CA_46A-46C2 | - | 20 |  | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 20 |  |  |  |  |
| CA_46A-46D2 | - | 20 |  | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 20 |  |  |  |
| CA_48A-48A | - | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 0 |
| CA_48A-48C | - | 5, 10, 15, 20 |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
| CA_48A-48D | - | 5, 10, 15, 20 |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |
| CA_48C-48C | - | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 80 | 0 |
| CA_48C-48D | - | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  | 100 | 0 |
|  |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |
| CA_48A-48E | - | 5, 10, 15, 20 |  |  | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 100 | 0 |
|  |  | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 5, 10, 15, 20 |  |  |
| CA_66A-66A | - | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  |  |  | 40 | 0 |
| CA_66A-66B | - | 5, 10, 15, 20 |  | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
| CA_66A-66C | - | 5, 10, 15, 20 |  | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |

---End of changes---