3GPP TSG-RAN WG4 Meeting #89	R4-1815806

Spokane, USA, 12 – 16 November 2018

Source:	Ericsson, SouthernLinc

Title:	TP to TR 36.716-02-02: Addition of CA configuration for 26-48 UL

Agenda item:	8.5.2

Document for:	Approval

# Background

This contribution provides text proposal on the CA configuration DL_26A-48A-48A_UL_ 26A-48A_BCS0 and DL_26A-48C_UL_ 26A-48A_BCS0 as defined in Revised WID on Rel16 LTE inter-band CA for 2 bands DL with 2 bands UL RP-181853 [1].

# Text Proposal

##### ---Start of changes---

## 5.X	26A-48A-48A_UL_ 26A-48A_BCS0,
26A-48C_UL_ 26A-48A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] |  | Bandwidth combination set |  |
| CA_26A-48C | CA_26A-48A | 26 |  | Yes | Yes | Yes |  |  |  |  |  |  | 50 |  | 0 |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_26A-48A-48A | CA_26A-48A | 26 |  | Yes | Yes | Yes |  |  |  |  |  |  | 50 |  | 0 |  |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.2		ΔTIB,c and ΔRIB,c values

Same ΔTIB,c and ΔRIB,c values as in TR 36.715-02-02 [2] for DL_26A-48A_UL_ 26A-48A_BCS0.

Table 6.8.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_26A-48A | 26 | 0.3 |
|  | 48 | 0,3 |

Table 6.8.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_26A-48A | 26 | 0 |
|  | 48 | 0 |

### 5.X.3	Coexistence studies

The UE-UE co-existence studies follow those investigated in TR 36.715-02-02 [2] for DL_26A-48A_UL_ 26A-48A_BCS0.

### 5.X.4	REFSENS requirements

As shown in table 6.8.3.1-2 of the TR 36.715-02-02 [2], IMD up to 5th order does not fall into own Rx of band 26 and 48. Therefore, no MSD requirement is needed for these CA configurations in TS 36.101.

##### ---End of changes---

# Reference

[1]	RP-181853, “Revised WID on Rel16 LTE inter-band CA for 2 bands DL with 2 bands UL”, Huawei

[2] RP-180978, “TR 36.715-02-02 V1.0.0”, Huawei