3GPP TSG-RAN WG4 Meeting #89	R4-1815877

Spokane, WA, USA, 12th – 16th Nov, 2018

Agenda item:	6.4.5

Source: 	Qualcomm Incorporated

Title: 	On reference NPRACH configuration for NB-IoT TDD RRM tests

Document for:	Discussion

# Introduction

In the last RAN4#88bis meeting, RRM tests for NB-IoT TDD were introduced [1-3]. The NPRACH configuration in these tests referred to the reference NPRACH configuration of Section A.3.18 in [4] which is specific to FDD. In this paper, we propose a new reference NPRACH configuration for TDD.

# Reference NPRACH configuration

Table A.3.8-2 is proposed as reference NPRACH configuration for TDD. If a parameter has a common value between TDD and FDD options, the value used for FDD as outlined in Table A.3.8-1 [4] is reused. Otherwise, the parameter value is selected to be as close as possible to its FDD counterpart.

Table A.3.18-2: NPRACH.R-2: Reference NPRACH Configuration for TDD

| Field | Value |  |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |  |  |
| nprach-Preamble-Format | fmt0 |  |  | NPRACH format 0 |  |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 |  |
| nprach-Periodicity | ms80 | ms80 | ms80 |  | {ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240} |  |
| nprach-StartTime | ms10 | ms10 | ms10 |  | {ms10, ms20, ms40, ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n32 |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r8 | r32 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v8 | v4 | v4 |  | {v4, v8, v16, v32, v48, v64, v96, v128} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| Note 1: See Clause 6.3.2 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

Proposal 1. Adopt Table A.3.8-2 as reference NPRACH configuration for NB-IoT TDD RRM tests.

# Conclusions

Proposal 1. Adopt Table A.3.8-2 as reference NPRACH configuration for NB-IoT TDD RRM tests.

# References

[1] R4-1812927

[2] R4-1812928

[3] R4-1812929

[4] TS36.133