3GPP TSG RAN WG4 Meeting #90	R4-1900101

Athens, Greece, 25th February – 1st March, 2019

3GPP TSG RAN WG1 Meeting #95		  	 R1-1814412

Spokane, USA, November 12th – 16th, 2018

Title:	LS on RRM measurements in half-duplex CA UE

Response to: 	-

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG1

To:	RAN WG4

CC:	-

Attachments:	-

Contact Person:

#### Name:	Karol Schober

E-mail Address:	karol.schober@nokia.com

1. Overall Description:

RAN1 has discussed RRM measurements in a half-duplex UE operating with carrier aggregation, and reached the proposal tabulated below. RAN1 would respectfully ask RAN4 to review this proposal:

- When the scheduling restrictions due to RRM measurements are applicable, half-duplex CA UE does not transmit in UL in one cell, if the UL transmission would collide with any SSB symbols other than indicated in ssb-PositionsInBurst inside SMTC window of another cell. RRM measurements in SMTC window are always protected.

- When the scheduling restrictions due to RRM measurements are NOT applicable, if the half-duplex CA UE is scheduled by DCI (Dynamic U) to transmit in UL in one cell, and if the UL transmission collides with any SSB symbols other than indicated in ssb-PositionsInBurst inside SMTC window of another cell, UE is not required to perform RRM measurements on the colliding SSB symbols.

- RRM measurements during the symbols indicated in ssb-PositionInBurst are always protected.

- Note: One cell and another cell are both serving cells for the UE

2. Actions:

To RAN WG4

ACTION: RAN1 would respectfully ask RAN4 to review the above proposal, provide RAN1 feedback on possible concerns or agreements made in relation to it, and consider it in the RAN4 specification work.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #96			25th  Feb – 1st Mar 2019		Athens,	Greece

TSG-RAN WG1 Meeting #96b			8th  – 12th  Apr 2019		Xi’an,	China

TSG-RAN WG1 Meeting #97			13th  – 17th May 2019		Reno,	US