3GPP TSG-RAN WG4 #90 Meeting                                        	R4-1900197

Athens, Greece, 25th Feb. – 1st March, 2019

Agenda item:	7.6.2

Source: 	LG Electronics

Title: 	Updated summary of interference studies for new x bands DL (x=3, 4, 5) with 2 bands UL inter-band CA combinations in rel-16

Document for:	Approval

1	Introduction

This contribution provides updated 5.1.5 summary of interference studies section for TR36.716-03-02. In Table 5.1.5-1 and Table 5.1.5.4, MSD levels are updated based on our proposal of MSD test results in the previous RAN4 #89 meeting.

****************** Start of the TP in subclause 5.1.5 of TR36.716-03-02 ************************

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes the CA band combinations with self-interference problems for LTE 3 bands DL and 2 bands UL CA.

Table 5.1.5-1: Summary of self-interference analysis for 3 bands DL/2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3A-11A-18A |  | CA_3A-11A |  | - |  | 5th IMD |  | - |  | 4.9 dB |  |
| CA_3A-11A-26A |  | CA_3A-11A |  | - |  | 5th IMD |  | - |  | 4.9 dB |  |
| CA_1A-3A-42C | CA_1A-42C | - | - | - | N/A |
| CA_1A-3A-42C | CA_3A-42C | - | - | - | N/A |
| CA_2A-4A-13A |  | CA_2A-13A |  | - |  | 4th IMD |  | - |  | 7.6 dB |  |
|  |  | CA_4A-13A |  | - |  | 4th IMD |  | - |  | 6.2 dB |  |
| CA_2A-2A-4A-5A |  | CA_2A-5A |  | - |  | 4th IMD |  | - |  | 7.6 dB |  |
|  | CA_4A-5A | - | - | - | N/A |
| CA_2A-2A-5A-66A-66A |  | CA_2A-5A |  | - |  | 4th IMD |  | - |  | 7.2 dB |  |
| CA_2A-5B-66A-66A |  | CA_2A-5A |  | - |  | 4th IMD |  | - |  | 7.2 dB |  |
|  | CA_5A-66A | - | - | - | N/A |
| CA_2A-5A-46D |  | CA_2A-5A |  | 3rd Harmonic |  | 4th, 5th IMDs |  | - |  | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB for IMD4 - 2.7 dB for IMD5 |  |
| CA_5A-46D-66A |  | CA_5A_46A |  | - |  | 5th IMD |  | - |  | 0.3 dB |  |
|  |  | CA_5A_66A |  | 3rd Harmonic |  | 4th, 5th IMDs |  | - |  | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB for IMD4 - 0 dB for IMD5 |  |
| CA_2A-13A-66A-66B |  | CA_2A-13A |  | - |  | 4th IMD |  | - |  | 7.2 dB |  |
|  |  | CA_13A-66A |  | - |  | - |  | - |  | N/A |  |
| CA_2A-13A-48A-48C | CA_2A-13A | 2nd Harmonic at high frequency band edge | - | - | 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. |
| CA_13A-46D-66A |  | CA_13A-66A |  | 3rd Harmonic |  | 4th, 5th IMDs |  | - |  | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 7.2 dB for IMD4 - 0 dB for IMD5 |  |
| CA_2A-13A-46D |  | CA_2A-13A |  | 3rd Harmonic |  | 4th IMD |  | - |  | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB |  |

Table 5.1.5-2 summarizes the CA_ band combinations with self-interference problems for CA_4 bands DL/2 bands UL CA.

Table 5.1.5-2: Summary of self-interference analysis for 4 bands DL/2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_2A-13A-48C-66A | CA_2A-13A | 2nd Harmonic at high frequency band edge | 4th IMD into B66 | - | - 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem will be covered in 3DL_2A-13A-66A_2UL_2A-13A |
|  | CA_13A-66A | 2nd Harmonic | 4th IMD into B2 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem will be covered in 3DL_2A-13A-66A_2UL_13A-66A - 5th IMD problem will be covered in 3DL_13A-48A-66A_2UL_13A-66A |
| CA_2A-13A-48A-48A-66A | CA_2A-13A | 2nd Harmonic at high frequency band edge | 4th IMD into B66 | - | - 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem will be covered in 3DL_2A-13A-66A_2UL_13A-66A |
|  | CA_13A-66A | 2nd Harmonic | 4th IMD into B2 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem will be covered in 3DL_2A-13A-66A_2UL_13A-66A - 5th IMD problem will be covered in 3DL_13A-48A-66A_2UL_13A-66A |

Table 5.1.5-3 summarizes the CA_ band combinations with self-interference problems for CA_5 bands DL/2 bands UL CA.

Table 5.1.5-3: Summary of self-interference analysis for 5 bands DL/2 bands UL CA

<Table will be added if there are proposed in related basket WID>

Based on the MSD test configuration as summarized in Table 5.1.5.1, 5.1.5.2, and 5.1.5.3, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3 bands DL/2 bands UL inter-band CA with IMD problems.

Table 5.1.5.4 Proposed 3 bands DL/ 2 bands UL inter-band CA REFSENS exceptions and test configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_3A-11A-18A | CA_3A-11A | 3 | 1730 | 5 | 25 | 1825 | 5 | N/A | FDD | IMD5 |
|  |  | 11 | 1442.9 | 5 | 25 | 1490.9 | 5 |  |  |  |
|  |  | 18 | 823.7 | 5 | 25 | 868.7 | 5 | 4.9 |  |  |
| CA_3A-11A-26A | CA_3A-11A | 3 | 1730 | 5 | 25 | 1825 | 5 | N/A | FDD | IMD5 |
|  |  | 11 | 1442.9 | 5 | 25 | 1490.9 | 5 |  |  |  |
|  |  | 26 | 823.7 | 5 | 25 | 868.7 | 5 | 4.9 |  |  |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | 7.6 |  |  |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  |  |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | 6.2 |  |  |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | 7.6 |  |  |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  |  |
| CA_2A-5B-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  |  |
| CA_2A-5A-46D | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5488 | 20 | 100 | 5488 | 20 | 2.5 |  |  |
|  | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5276 | 20 | 100 | 5276 | 20 | 2.7 |  |  |
| CA_5A-46D-66A | CA_5A_46D | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 0.3 |  |  |
|  | CA_5A_66A | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 |  |  |  |
|  |  | 46 | 5208 | 20 | 100 | 5208 | 20 | 2.5 |  |  |
|  | CA_5A_66A | 5 | 846.5 | 5 | 25 | 891.5 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 66 | 1774 | 5 | 25 | 2194 | 5 |  |  |  |
|  |  | 46 | 5160 | 20 | 100 | 5160 | 20 | 0 |  |  |
| CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | 7.2 |  |  |
| CA_13A-46D-66A | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 66 | 1713.5 | 5 | 25 | 2113.5 | 5 |  |  |  |
|  |  | 46 | 5922.5 | 20 | 100 | 5922.5 | 20 | 7.2 |  |  |
|  | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 66 | 1727 | 5 | 25 | 2127 | 5 |  |  |  |
|  |  | 46 | 5800 | 20 | 100 | 5800 | 20 | 0 |  |  |
| CA_2A-13A-46D | CA_2A-13A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 46 | 5374 | 20 | 100 | 5374 | 20 | 2.5 |  |  |

<End of TP>