3GPP TSG-RAN WG4 Meeting #90	R4-1900208

# Athens, Greece, February 25 – March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | DRAFT | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-2: correction of multi-band aspects in REFSENS for PC3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current version of the specification has correctly implemented the multi-band aspects of EIS spherical coverage (Clause 7.3.4.3) but has not implemented the multi-band aspects of Reference sensitivity (Clause 7.3.2.3) for FR2 PC3 UEs. During the RAN4 #89 meeting, the following agreement was captured in the RAN4 meeting minutes: Agreement: the following texts will be captured into 38.101-2 EIS: For the UEs that support operation in multiple FR2 bands, minimum requirement for reference sensitivity in section 7.3.2 and EIS spherical coverage in section [7.3.4] shall be increased, respectively, by the reference sensitivity relaxation parameter DMBP and EIS spherical coverage relaxation parameter DMBS are as specified in section 6.2.1 EIRP: For the UEs that support operation in multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in section 6.2.1 shall be decreasd, respectively, by the peak EIRP relaxation parameter DMBP and EIRP spherical coverage relaxation parameter DMBS are as specified in section 6.2.1 This draft CR provides the correction for Clause 7.3.2.3 in order to implement the multi-band framework for the Reference sensitivity requirement. It further provides an editorial correction to sub-clause numbering and another editorial correction for the terminology used to refer to the EIS spherical coverage relaxation parameter in Clause 7.3.4.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: provides a reference to the multi-band framework from the Reference sensitivity requirement for PC3 (Clause 7.3.2) and a note to clarify the applicability of the requirements in Table 7.3.2.3-1 to be for UEs which support a single FR2 band. Change 2: introduces a Void clause 7.3.3 to maintain consistent sub-clause numbering. Change 3: editorial correction for the terminology used to refer to the EIS spherical coverage relaxation parameter in Clause 7.3.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The multi-band framework for the Reference sensitivity requirement for FR2 PC3 UEs will not be fully implemented in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.3, 7.3.3 (new), 7.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<begin change 1>>

# 7	Receiver characteristics

## 7.1	General

Unless otherwise stated, the receiver characteristics are specified over the air (OTA). The reference receive sensitivity (REFSENS) is defined assuming a 0 dBi reference antenna located at the center of the quiet zone.

## 7.2	Diversity characteristics

The minimum requirements on effective isotropic sensitivity (EIS) apply to two measurements, corresponding to DL signals in orthogonal polarizations.

## 7.3	Reference sensitivity

### 7.3.1	General

The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX beam peak direction, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3.2	Reference sensitivity power level

#### 7.3.2.1	Reference sensitivity power level for power class 1

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal) with peak reference sensitivity specified in Table 7.3.2.1-1. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link Angle).

Table 7.3.2.1-1: Reference sensitivity for power class 1

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.5 | -94.5 | -91.5 | -88.5 |
| n258 | -97.5 | -94.5 | -91.5 | -88.5 |
| n260 | -94.5 | -91.5 | -88.5 | -85.5 |
| n261 | -97.5 | -94.5 | -91.5 | -88.5 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Table 7.3.2.1-2: Uplink configuration for reference sensitivity

| Operating band | NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | 120 kHz | TDD |

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

Table 7.3.2.1-3: Network signaling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n258 | NS_201 |

#### 7.3.2.2	Reference sensitivity power level for power class 2

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal) with peak reference sensitivity specified in Table 7.3.2.2-1. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link Angle).

Table 7.3.2.2-1: Reference sensitivity for power class 2

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -94.5 | -91.5 | -88.5 | -85.5 |
| n258 | -94.5 | -91.5 | -88.5 | -85.5 |
| n261 | -94.5 | -91.5 | -88.5 | -85.5 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

#### 7.3.2.3	Reference sensitivity power level for power class 3

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal) with peak reference sensitivity specified in Table 7.3.2.3-1. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link Angle).

For the UEs that support operation in multiple FR2 bands, the minimum requirement for Reference sensitivity in Table 7.3.2.3-1 shall be increased per band, respectively, by the reference sensitivity relaxation parameter ∆MBP,n as specified in section 6.2.1.3.

Table 7.3.2.3-1: Reference sensitivity

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3 | -85.3 | -82.3 | -79.3 |
| n258 | -88.3 | -85.3 | -82.3 | -79.3 |
| n260 | -85.7 | -82.7 | -79.7 | -76.7 |
| n261 | -88.3 | -85.3 | -82.3 | -79.3 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

#### 7.3.2.4	Reference sensitivity power level for power class 4

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal) with peak reference sensitivity specified in Table 7.3.2.4-1. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link Angle).

Table 7.3.2.4-1: Reference sensitivity for power class 4

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.0 | -94.0 | -91.0 | -88.0 |
| n258 | -97.0 | -94.0 | -91.0 | -88.0 |
| n260 | -95.0 | -92.0 | -89.0 | -86.0 |
| n261 | -97.0 | -94.0 | -91.0 | -88.0 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

<<end change 1>>

<<begin change 2>>

### 7.3.3	Void

<<begin change 2>>

<<begin change 3>>

### 7.3.4	EIS spherical coverage

#### 7.3.4.1	EIS spherical coverage for power class 1

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.1

The maximum EIS at the 85th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.1-1 below. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link angle).

Table 7.3.4.1-1: EIS spherical coverage for power class 1

| Operating band | EIS at 85th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | [-89.5] | [-86.5] | [-83.5] | [-80.5] |
| n258 | [-89.5] | [-86.5] | [-83.5] | [-80.5] |
| n260 | [-86.5] | [-83.5] | [-80.5] | [-77.5] |
| n261 | [-89.5] | [-86.5] | [-83.5] | [-80.5] |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

#### 7.3.4.2	EIS spherical coverage for power class 2

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.2

The maximum EIS at the 60th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.2-1 below. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link angle).

Table 7.3.4.2-1: EIS spherical coverage for power class 2

| Operating band | EIS at 60th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | [-83.5] | [-80.5] | [-77.5] | [-74.5] |
| n258 | [-83.5] | [-80.5] | [-77.5] | [-74.5] |
| n261 | [-83.5] | [-80.5] | [-77.5] | [-74.5] |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

#### 7.3.4.3	EIS spherical coverage for power class 3

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.3

The maximum EIS at the 50th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-1 below. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link angle).

For the UEs that support operation in multiple FR2 bands, the minimum requirement for EIS spherical coverage in Table 7.3.4.3-1 shall be increased per band, respectively, by the EIS spherical coverage relaxation parameter ∆MBS,n as specified in section 6.2.1.3.

Table 7.3.4.3-1: EIS spherical coverage for power class 3

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | [-77.4] | [-74.4] | [-71.4] | [-68.4] |
| n258 | [-77.4] | [-74.4] | [-71.4] | [-68.4] |
| n260 | [-73.1] | [-70.1] | [-67.1] | [-64.1] |
| n261 | [-77.4] | [-74.4] | [-71.4] | [-68.4] |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

#### 7.3.4.4	EIS spherical coverage for power class 4

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.4

The maximum EIS at the 20th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.4-1 below. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link angle).

Table 7.3.4.4-1: EIS spherical coverage for power class 4

| Operating band | EIS at 20th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | [-88.0] | [-85.0] | [-82.0] | [-79.0] |
| n258 | [-88.0] | [-85.0] | [-82.0] | [-79.0] |
| n260 | [-83.0] | [-80.0] | [-77.0] | [-74.0] |
| n261 | [-88.0] | [-85.0] | [-82.0] | [-79.0] |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

<<end change 3>>