3GPP TSG-RAN WG4 Meeting #90	R4-1900454

# Athens, Greece, Feburary 25 – March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on MTTD for intra-band contiguous EN-DC (section 7.5.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For intra-band EN-DC, it is reasonable to separate contiguous case and non-contiguous case. For non-contiguous, multiple UL timing can be supported. However, for contiguous case, one should allow UE to support single UL timing, even though the network configures two separate TAGs for LTE and NR cell. In particular in band DC_(n)71, only single PA architecture is applicable. It is reasonable for UE to support single UL timing in this case. But if MTTD is larger than CP length, it is impossible for UE to implement single UL timing. In current TS38.133, no requirement is defined on MTTD for intra-band EN-DC due to the fact MTTD is very small. This MTTD requirement may cause confusion to RAN1/RAN2. Thus, we propose that MTTD for intra-band contiguous EN-DC is zero. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification is made in section 7.5.3 to specify that MTTD for intra-band contigous EN-DC is zero. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current MTTD requirement for intra-band contiguous EN-DC is not feasible for UE implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change >>

### 7.5.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [16].

The maximum uplink transmission timing difference for synchronous E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band contiguous EN-DC is zero.

## << End of Change >>